3GPP TSG-SA5 Meeting #152	S5-237963

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1131 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.541 YANG Corrections (SA5#152) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.5.35, E.5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/930 at commit c13ed99da16f6bf74f79a34110fdc7c1fce0f50d Implements YANG for S5-235380 CR0985 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE ***

*** yang-models/_3gpp-nr-nrm-operatordu.yang ***

## E.5.35	module _3gpp-nr-nrm-operatordu.yang

<CODE BEGINS>

module _3gpp-nr-nrm-operatordu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-operatordu";

prefix "operdu3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction {prefix gnbdu3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the OperatorDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2023-11-05 { reference CR-1131 ; }

revision 2023-09-17 { reference CR-1042 ; }

revision 2021-10-01 { reference "Initial revision"; }

grouping OperatorDUGrp {

description "Represents the OperatorDU IOC.";

reference "3GPP TS 28.541";

uses gnbdu3gpp:GNBDUFunctionGrp {

refine gNBId {

mandatory true;

}

refine gNBIdLength {

mandatory true;

}

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list OperatorDU {

description "This IOC contains attributes to support operator specific

gNB-DU level information to support the NG-RAN Multi-Operator Core

Network (NG-RAN MOCN) network sharing with multiple Cell Identity

broadcast feature. An instance of OperatorDU <<IOC>> should be created

and configured for each POP. When configured the attributes override

those in parent GNBDUFunction instance.

The OperatorDU <<IOC>> is only used to support NG-RAN MOCN with

multiple cell identity broadcast feature. If NG-RAN MOCN with multiple

cell identity broadcast feature is not supported, is not used.

For scenarios with an F1 interface supporting multiple PLMN broadcast,

the values of the EP_F1C and EP_F1U attributes contained by different

OperatorDU of the same GNBDUFunction should be same.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses OperatorDUGrp;

}

uses gnbdu3gpp:GNBDUFunctionGrp;

}

}

}

<CODE ENDS>

*** END OF CHANGE ***

*** START OF CHANGE ***

*** yang-models/_3gpp-5gc-ecmconnectioninfo.yang ***

## E.5.x	module _3gpp-5gc-ecmconnectioninfo.yang

<CODE BEGINS>

module _3gpp-5gc-ecmconnectioninfo {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5gc-ecmconnectioninfo";

prefix "econn3gpp";

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the EcmConnectionInfo Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2023-11-05 { reference CR-1131 ; }

grouping GeoLocGrp {

description "This datatype represent the geographical location.";

reference "3GPP 28.538 clause 6.3.4";

list geographicalCoordinates {

description "This refers to the Topological Service Area,

(see clause 7.3.3.2 in TS 23.538) that is defined as a datatype

(see clause 6.3.8).";

key latitude;

max-elements 1;

uses types3gpp:GeographicalCoordinates;

}

leaf civicLocations {

type string;

}

}

grouping TopologicalServiceAreaGrp {

description "This datatype represents the topological service area.";

reference "3GPP TS 28.538 clause 6.3.7";

leaf-list cellIDList {

description "It represents the list of NR cells.

The cell ID, together with the gNB Identifier (using gNBId of the

parent GNBCUCPFunction or GNBDUFunction or ExternalCUCPFunction),

identifies a NR cell within a PLMN. This is the NR Cell Identity(NCI).

See subclause 8.2 of TS 38.300";

type int32;

}

list trackingAreaIdList {

description "It represents the list of tracking areas within a PLMN.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

list servingPLMN {

description "It specifies the PLMN to be served.";

max-elements 1;

key idx;

leaf idx { type uint32; }

uses types3gpp:PLMNId;

}

}

grouping ServingLocationGrp {

description "This datatype represents the location which is to be served

by the node.";

reference "3GPP TS 28.536 clause 6.3.3";

list geographicalLocation {

description "This refers to the Geographical Service Area,

(see clause 7.3.3.3 in TS 23.558 that is defined as a datatype

(see clause 6.3.4).";

max-elements 1;

key idx;

leaf idx { type uint32; }

uses GeoLocGrp;

}

list topologicalLocation {

description "This refers to the Topological Service Area,

(see clause 7.3.3.2 in TS 23.558) that is defined as a datatype

(see clause 6.3.7). ";

max-elements 1;

key idx;

leaf idx { type uint32; }

uses TopologicalServiceAreaGrp;

}

}

grouping _5GCNfConnEcmInfoGrp {

description "This data type specifies the 5GC NF connection information.";

leaf _5GCNFType {

type enumeration {

enum PCF;

enum NEF;

enum SCEF;

}

description "It indicates the type of a NF instance.";

}

leaf _5GCNFIpAddress {

type inet:host;

mandatory true;

description "This parameter defines address of a NF instance,

It can be IP address (either IPv4 address (See RFC 791) or

IPv6 address (See RFC 2373) or FQDN (See TS 23.003). ";

}

leaf _5GCNFRef {

type types3gpp:DistinguishedName;

description "This attribute holds the DN of a NF instance.";

}

}

grouping UPFConnInfoGrp {

description "This data type specifies the UPF connection information.";

leaf uPFIpAddress {

type inet:host;

description "This parameter defines address of an UPF instance,

It can be IP address (either IPv4 address (See RFC 791) or

IPv6 address (See RFC 2373) or FQDN (See TS 23.003). ";

}

leaf uPFRef {

type types3gpp:DistinguishedName;

description "This attribute holds the DN of an UPF instance.";

}

}

grouping EcmConnectionInfoGrp {

description "Represents the EcmConnectionInfo IOC.";

list eASServiceArea {

description "This parameter defines the EAS service area

(see clause 7.3.3.6 in TS 23.558).";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32 ; }

uses ServingLocationGrp;

}

list eESServiceArea {

description "This parameter defines the EES service area

(see clause 7.3.3.5 in TS 23.558).";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32 ; }

uses ServingLocationGrp;

}

list eDNServiceArea {

description "This parameter defines the EDN service area

(see clause 7.3.3.4 in TS 23.558)";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32 ; }

uses ServingLocationGrp;

}

leaf eASIpAddress {

type inet:ip-address;

mandatory true;

description "This parameter defines address of an EAS instance.

It can be IP address (either IPv4 address (See RFC 791) or IPv6

address (See RFC 2373).";

}

leaf eESIpAddress {

type inet:ip-address;

mandatory true;

description "This parameter defines address of an EES instance.

It can be IP address (either IPv4 address (See RFC 791) or IPv6

address (See RFC 2373). ";

}

leaf eCSIpAddress {

type inet:ip-address;

mandatory true;

description "This parameter defines address of an ECS instance.

It can be IP address (either IPv4 address (See RFC 791) or IPv6

address (See RFC 2373).";

}

leaf ednIdentifier {

type string;

description "The identifier of the edge data network (See TS 23.558).";

}

leaf ecmConnectionType {

type enumeration {

enum USERPLANE;

enum CONTROLPLANE;

enum BOTH;

}

description "It indicates the type of ECM connection (i.e., user plane

connection via UPF, control plane connection via PCF or NEF.";

}

list _5GCNfConnEcmInfoList {

description "The attribute specifies a list of 5GCNfConnInfo which is

defined as a datatype (see clause 5.3.120). It is used to

provide 5GC NFs, such as PCF, NEF, SCEF, that are connected EDN NFs,

such as EAS, EES, and ECS.";

config false;

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses _5GCNfConnEcmInfoGrp;

}

list uPFConnectionInfo {

description "The attribute is defined as a datatype UPFConnInfo

(see clause 5.3.121). It is used to provide the UPF IP address and

UPF DN. ";

config false;

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32 ; }

uses UPFConnInfoGrp;

}

}

grouping EcmConnectionInfoSubtree {

description "Subtree containing EcmConnectionInfo.";

list EcmConnectionInfo {

description "This IOC contains attributes to enable ECSP management

system to connect EDN NFs (i.e., EAS, ECS, and EES) to 5GC NFs

(i.e. UPF, PCF, NEF, SCEF) (See clause 7.4.3 and 7.4.4 in TS 28.538). ";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EcmConnectionInfoGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE ***