# 3GPP TSG-SA5 Meeting #153	S5-240648

# Seville, Spain, 29th January-2nd February, 2024

# Source:	Samsung, EUTC, EDF, DTAG

# Title:	Rel-18 pCR TS 28.318 NRM Definitions for coordinated rapid recovery

# Document for:	Approval

Agenda Item:	6.6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	Rationale

This pCR proposes the NRM definitions required to support the procedure defined in clause 7.1 of TS 28.318 v0.3.0.

# 3	Detailed proposal

It is proposed to agree to the following text to clause 6 in TS 28.318.

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	DIRECTIVE (EU) 2019/ 944 OF THE EUROPEAN PARLIAMENT AND OF THE COUNCIL - of 5 June 2019 - on common rules for the internal market for electricity and amending Directive 2012/ 27/ EU (europa.eu)
https://eur-lex.europa.eu/legal-content/EN/TXT/PDF/?uri=CELEX:32019L0944&from=EN

[3]	IEC TC 57 https://www.iec.ch/ords/f?p=103:7:511571509228708::::FSP_ORG_ID,FSP_LANG_ID:1273,25

[4]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements"

[5]	IEC "Bringing intelligence to the grid", International Electrotechnical Commission, Geneva, Switzerland, 2018.  https://www.iec.ch/basecamp/bringing-intelligence-grid <accessed: 12.7.23>

[6]	IEEE SMARTGRID, "Standards", IEEE, 2023. https://smartgrid.ieee.org/about-ieee-smart-grid/standards <accessed: 12.7.23>

[7]		Sendin, A., Stafford, J., Grilli, A., "Utilities and Telecommunications in a Nutshell", EUTC, Ediciones Experiencia, 2022.

[8]	3GPP TS 22.104, "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[9]	3GPP TS 22.261, "Service requirements for the 5G system; Stage 1"

[10]	IEC TR 61000-3-6 Electromagnetic compatibility (EMC) - Part 3-6: Limits - Assessment of emission limits for the connection of distorting installations to MV, HV and EHV power systems.

[11]	IETF RFC 1628, "UPS Management Information Base", May 1994.

[12]	ETSI ES 202 336-3: "Environmental Engineering (EE); Monitoring and Control Interface for Infrastructure Equipment (Power, Cooling and Building Environment Systems used in Telecommunication Networks); Part 3: AC UPS power system control and monitoring information model".

[X]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

Next Change

# 6	Informational model definitions

## 6.2       Class diagrams

### 6.2.1      Relationships

This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document

Figure 6.2.1-1: DSO Rapid Recovery NRM fragment

### 6.2.2	Inheritance

This clause depicts the inheritance relationships.

<<InformationObjectClass>>

Top

<<InformationObjectClass>>

DsoRapidRecovery

<<InformationObjectClass>>

MnoRapidInterventionInfo

Figure 6.2.2-1: DSO Rapid Recovery NRM fragment

## 6.3       Class definitions

### 6.3.1	DsoRapidRecovery

#### 6.3.1.1 Definition

This IOC represents the mutual information exchange between DSO and MNO, for a particular instance of an outage at a particular location, to achieve Rapid Recovery of DSO’s energy service outage for a  site/location which require service restoration.

#### 6.3.1.2 Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoOutStartTime | M | T | T | F | T |
| dsoRestoreTimeExpected | M | T | T | F | T |
| dsoRapidInterventionInfo | M | T | T | F | T |
| dsoServiceRestoreInfo | M | T | T | F | T |
| dsoRapidRecoveryLocation | M | T | T | F | T |
| upsInfo | M | T | F | F | T |

##### 6.3.1.3	Attribute constraints

None.

##### 6.3.1.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 6.3.2 DsoInterventionInfo  <<dataType>>

##### 6.3.2.1 Definition

This <<dataType>> is of attribute dsoRapidInterventionInfo and defines several information shared by DSO to MNO related to its requirement for MNO’s Rapid Intervention (MRI).

The attribute dsoFeederRestoreTime defines the time stamp at which DSO restores its Energy feeder service. From this time a DSO can use its automated smart energy services e.g SCADA to recover its distribution services much quicker in substations instead of manual processes. Hence this is also the time stamp from when the DSO expects MRI to start.

The attribute dsoRequiredInterventionDuration defines time “duration” for which the DSO requires MRI in order to restore its energy distribution services rapidly at the location provided by the attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC. This time duration is usually defined in minutes.

The attribute isDsoPriorityRecoveryLocation indicates whether the location of DSO substation (identified by attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC) needs recovery on priority by using smart energy services or not. Some of the substation locations might be providing electrical energy for crucial services like hospitals, mission critical service venues, government offices etc. This is used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MRI because the gNBs are servicing these substations.

##### 6.3.2.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoFeederRestoreTime | M | T | T | F | T |
| dsoRequiredInterventionDuration | M | T | T | F | T |
| isDsoPriorityRecoveryLocation | M | T | T | F | T |

##### 6.3.2.3	Attribute constraints

None.

##### 6.3.2.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable

#### 6.3.3 MnoRapidInterventionInfo

##### 6.3.3.1 Definition

This IOC defines several information feasible for fulfilment of DSO’s rapid recovery requirements. It may be possible that instead of full, some part of requested Rapid Recovery requirements, MNO is actually able to cater in terms of intervention start time, location etc.

This IOC is name-contained by the DsoRapidRecovery IOC. Instances of this IOC are implicitly created by MnS producers upon receiving the createMOI request for the DsoRapidRecovery IOC to configure the related information which can be queried by the DSO, when required.

The attribute mnoInterventionTime defines the time at which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) at the location provided by the attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC. It may depend on MNO site’s UPS backup status, engagement in other critical service venues like hospitals etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services.

The attribute mnoInterventionDuration defines the time duration for which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) at the location provided by the attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC. It may depend on its UPS backup status, engagement in other critical service venues like hospitals etc. This time duration is usually defined in minutes.

The attribute isMnoInterventionPossible indicates whether the MNO can provide its dedicated communication services (i.e. MRI) for DSO’s smart energy services at this location (identified by the attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC) or not. The MRI feasibility by MNO may depend on MNO’s UPS backup status per site, or other prior crucial service commitments in other venues like hospital, government offices etc.

##### 6.3.3.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnoInterventionTime | CM | T | F | F | T |
| mnoInterventionDuration | CM | T | F | F | T |
| isMnoInterventionPossible | M | T | F | F | T |

##### 6.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| mnoInterventionTime S | Condition: If isMnoInterventionPossible is TRUE |
| mnoInterventionDuration S | Condition: If isMnoInterventionPossible is TRUE |

##### 6.3.3.4	Notifications

The common notifications defined in clause 4.5 of [X] are valid for this IOC.

#### 6.3.4 ServiceRestoreInfo  <<dataType>>

##### 6.3.4.1 Definition

This <<dataType>> is of attribute dsoServiceRestoreInfo and defines information shared by DSO to MNO regarding actual final restoration of DSO’s distribution services. From this MNO gets to know at what time it can stop its Rapid Intervention for the DSO and at which locations. MNO also gets to know when and where (gNB sites) it does not need UPS backup anymore for its communication services/operations. DSO will update this information once the energy distribution service has been restored.

The attribute dsoRestoreTimeActual defines the time at which DSO finally restores its energy distribution services rapidly by using MNO’s MRI at the location provided by the attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC.. This time is much smaller than dsoRestoreTimeExpected.

The attribute isDsoRestoreComplete indicates whether the outage at location (identified by attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC) is finally restored or not by the DSO for resuming its energy distribution services.

##### 6.3.4.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dsoRestoreTimeActual | M | T | T | F | T |
| isDsoRestoreComplete | M | T | T | F | T |

##### 6.3.4.3	Attribute constraints

None.

##### 6.3.4.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable

#### 6.3.5 ServiceLocationIdentifier  <<datatype>>

##### 6.3.5.1 Definition

This <<datatype>> defines regions/locations where DSO energy service outage occurs. These are the regions/locations where DSO subsystems are situated. The DSO recovery related information is for a particular region/location. This is defined as latitude-longitude pair and energySupplyId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region containing substation. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is known between both the parties using out-of-band mechanisms hence not subject to standardisation.

The energySupplyId, a data element used extensively in this class definition is known to both the MNO and DSO and is the means by which information is organized, essentially the 'key identifier' used to exchange information about specific sites, which is essential for most Rapid Recovery actions (see Annex C.2) and Rapid Intervention actions (see Annex C.2).

##### 6.3.5.2 Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| geoCoordinate | M | T | T | F | T |
| energySupplyId | M | T | T | F | T |

##### 6.3.5.3	Attribute constraints

None.

##### 6.3.5.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable

#### 6.3.6	UpsInfo <<datatype>>

##### 6.3.6.1 Definition

This datatype represents the UPS related information of MNO site or a base station. This is required by DSO from MNO to achieve coordinated intelligent outage planning by DSO during its energy outage recovery. If the information like MNO sites’ UPS backup status, base station ids mapped to energy supply ids, battery draining status at MNO site is known to DSO then DSO may work more intelligently on its energy outage recovery, taking into account which base station Ids of MNO their infrastructure relies on as well as the remaining duration of UPS backup in that MNO site.

##### 6.3.6.2 Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| IsUpsAvailable | M | T | F | F | T |
| totalUpsBackupDuration | CM | T | F | F | T |
| remainingUpsBackupDuration | CM | T | F | F | T |
| baseStationId | M | T | F | F | T |

##### 6.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| totalUpsBackupDuration S | Condition: If IsUpsAvailable is TRUE |
| remainingUpsBackupDuration S | Condition: If IsUpsAvailable is TRUE |

##### 6.3.6.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 6.4	Attribute definitions

#### 6.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document in addition to those existing in clause 4.4.1 of [X].

| dsoRapidInterventionInfo | It contains attributes that define information shared by DSO to MNO related to its requirement for MNO’s Rapid Intervention (MRI) like expected time to start MRI, expected duration of MRI, expected locations that require MRI on priority. | type: DsoInterventionInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| --- | --- | --- |
| dsoServiceRestoreInfo | It contains list of attributes that define information shared by DSO to MNO regarding actual final restoration of DSO’s distribution services. From this information, MNO gets to know at what time it can stop its MRI services for DSO and at which locations. MNO also gets to know when and where (gNB sites) it does not need UPS backup anymore for its communication services/operations. | type: ServiceRestoreInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dsoRapidRecoveryLocation | It defines the region/location where DSO energy service outage occurs. These are the locations where DSO subsystems are situated. This is defined as latitude -longitude pair and energySupplyId. The point defined by the Latitude-Longitude pair serves as one point of an area polygon defining the region. The energySupplyId identifies the MNO facility using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. | Type: ServiceLocationIdentifier multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoOutStartTime | It defines the time stamp at which the DSO energy service outage occurs at the location provided by the attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC. Along with outage location and expected recovery time this could be beneficial for MNO to plan and prepare (e.g when and where to activate and de-activate) its UPS power backup at different site locations for its crucial communication services and also for assisting DSO for MRI (MNO Rapid Intervention) | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoRestoreTimeExpected | It defines the time by which DSO expects its energy distribution service to get restored through its manual dispatch and processes at the location provided by the attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC. This is usually a longer time period that may not be acceptable for some crucial public services like hospitals. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dsoFeederRestoreTime | It defines the time at which DSO restores its Energy feeder service. This is the time from when the DSO expects MRI to start. From this time a DSO can use its automated smart energy services e.g SCADA to recover its distribution services much quicker in substations instead of manual processes. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| dsoRequiredInterventionDuration | It defines the time duration for which the DSO requires MRI in order to restore its energy distribution services rapidly. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| isDsoPriorityRecoveryLocation | It indicates whether the location of DSO substation (identified by attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC) needs recovery on priority by using smart energy services or not. This can be used by MNO to ascertain the gNBs at which the UPS power needs to be reserved as part of MNO Rapid Intervention. The default value is "FALSE". allowedValues: TRUE, FALSE | Type:Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: True |
| mnoInterventionTime | It defines the time at which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) depending on its UPS backup status , other critical service venues etc. DSO needs this to use its smart energy services like SCADA for recovering its energy distribution services. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mnoInterventionDuration | It defines the time duration for which a MNO will actually be able to provide dedicated communication services to DSO (as MRI) depending on its UPS backup status , other critical service venues etc. This time duration is defined in minutes. allowedValues: Integer with a minimum value of 1 | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| isMnoInterventionPossible | It indicates whether the MNO can provide its dedicated communication services (i.e. MRI) for DSO’s smart energy services at this location (identified by attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC) or not. The feasibility may depend on MNO’s UPS backup status per site, or other prior crucial service commitments in other venues like hospital, government offices etc. The default value is "TRUE". allowedValues: TRUE, FALSE | Type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: True |
| dsoRestoreTimeActual | It defines the time at which DSO finally restores its energy distribution services rapidly by using MRI. This time is much smaller than dsoRestoreTimeExpected. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| isDsoRestoreComplete | It indicates whether the outage at locations (identified by attribute dsoRapidRecoveryLocation of DsoRapidRecovery IOC) is finally restored or not by the DSO for resuming its energy distribution services. This gives substation locations where DSO is able to finally restore its energy distribution services. The default value is "TRUE". allowedValues: TRUE, FALSE | Type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: TRUE isNullable: True |
| geoCoordinate | This attribute defines a geographical location on earth. See clause 4.3.53 in [X] | type: GeoCoordinate multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energySupplyId | energySupplyId identifies the MNO facility (e.g. base station) using DSO energy services. The energySupplyId is communicated between both the parties using out-of-band mechanisms hence not subject to standardisation. A meter with a unique energySupplyId is installed by DSO in the MNO site. | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| upsInfo | It defines the UPS related information of MNO site or a base station. This is required by DSO from MNO to achieve coordinated intelligent outage planning by DSO during its energy outage recovery. | type: UpsInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| isUpsAvailable | It tells if the MNO site has UPS installed/available or not. There might be some sites where UPS is not installed yet or is under progress etc. allowedValues: TRUE, FALSE | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| totalUpsBackupDuration | It defines the total time duration for which MNO site can operate on its UPS backup i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from onset of DSO outage. It is defined in minutes. allowedValues: Integer with a minimum value of 1 | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable:True |
| remainingUpsBackupDuration | It defines the remaining time duration for which MNO site can operate on its UPS backup at a given time i.e. capacity of UPS backup in terms of time duration (e.g. minutes) for which it can provide power supply to MNO site from a given time. It is defined in minutes. allowedValues: Integer with a minimum value of 1 [NOTE1] | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| baseStationId | Base station identity as defined by MNO for its base stations in a given site. This is like a user label for identification of the MNO facility. | type: String multiplicity: 1 isOrdered: N/A isUnique:True defaultValue: None isNullable: True |
| NOTE 1: The value of remainingUpsBackupDuration can be taken from IETF RFC 1628 [11], upsEstimatedMinutesRemaining. ETSI ES 202 336-11 [12] defines a field "Estimated remaining battery autonomy (time) during discharge" as provided by the monitoring and control interface for battery system with integrated control and monitoring information model, however there is no corresponding stage 3 definition of this value. |  |  |

### 6.Y	Common notifications

Refer to clause 4.5.2 of [X] for Configuration notifications.

End of Change