3GPP TSG-SA5 Meeting #153 	S5-240367

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0499 | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Addition of charging principle for PNI-NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_CH |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The 5G data connectivity charging should be supported for PNI-NPN, but converged charging for data connectivity in PNI-NPN has not been considered in this specification. The CR is related to the conclusion in clause 6.2.6 of TR 28.828. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PNI-NPN data connectivity charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging principle for NPN charging is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.19	NPN Data Connectivity Charging

5.1.19.1	General

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [102], which is either:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN, as specified in the TS 23.501 [200] clause 5.30.2; or

-	a Public Network Integrated NPN (PNI-NPN), i.e. a non-public network deployed with the support of a PLMN. as specified in the TS 23.501 [200] clause 5.30.3.

In order to support the SNPN data connectivity charging, the following charging principles are used.

5.1.19.2	SNPN Data Connectivity Charging

For UE accessing network belong to SNPN operator, SMF in the SNPN reports the charging information contains SNPN ID (PLMN ID and NID identifying an SNPN) to the CHF in the SNPN.

For UE accessing the SNPN service via PLMN,

-	SMF in SNPN reports the charging information per UE, which contains access type ("Non-3GPP access") to the CHF belongs to the same SNPN, using PLMN ID and NID to identify the SNPN.

-	SMF in PLMN reports the charging information per SNPN.

For UE accessing the PLMN service via SNPN,

-	SMF in the PLMN reports the charging information per UE.

-	SMF in the SNPN reports the charging information per PLMN.

5.1.19.3	PNI-NPN Data Connectivity Charging

For UE accessing PNI-NPN, SMF in the PNI-NPN reports the charging information contains S-NSSAI or dedicated DNNs, or dedicated DNN and S-NSSAI combination which identifies PNI-NPN to the CHF in the PNI-NPN.

End of changes