3GPP TSG-SA5 Meeting #153	S5-240654

# Sevilla, Spain, 29th Jan 2024 - 2nd Feb 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0315 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Add Schema related operations for provisioning MnS (stage 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently there is no standard mechanism to retrieve the schema(s) for resources managed by a Provisioning MnS. For an MnS Consumer to be able to execute on the CRUD or notification operations as described in TS 28.532 (section 11.1 Generic Provisioning Management Service) it is imperative that the MnS Consumer has access to the schema(s) supported by that MnS producer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces stage 2 operations to allow an MnS Consumer to retrieve the schema(s) of resources managed by the Provisioning Management Service. The Generic Provisioning Management Service is updated to add three new operations: - Get Schema Information - Get Schema Manifest - Get Schema Content |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Generic Provisioning Management Service (MnS) Consumers will not be able to retrieve the schema(s) of the network resources managed by the provisioning Management Service (MnS) in a standard manner. This will make it difficult for MnS Consumers to use the MnS Producer and will adversely affect interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.0, 11.1.1.x (NEW), 11.1.1.x (NEW), 11.1.1.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 11.1.0	Introduction

This clause provides the stage 2 definitions of Create, Read, Update, Delete, Get Schema Information, Get Schema Manifest, and Get Schema Content operations for managing managed objects. According to clause 4.2.2 of TS 28.533 [13], these operations are the MnS component type A. The operations specified in this clause in combination with a NRM (MnS component type B) constitute a MnS, as defined in clause 4.3 of TS 28.533 [13] providing generic provisioning services for supported NRM (MnS component type B) of all MnS.

In addition, notifications to report changes related to managed objects and their attributes are specified.

2nd Change

#### 11.1.1.x	getSchemaInformation operation

##### 11.1.1.x.1	Description

This operation is invoked by MnS consumer to request schema information associated with the Managed Object instances managed by the MnS producer.  One or more schema information objects may be retrieved depending on the parameters provided.

##### 11.1.1.x.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| schemaName | M | Name of the schema | The name of the schema |
| schemaVersion | CO | Version of the schema | The version of the schema. If it is not provided, then the latest version shall be inferred. The schemaVersion may be provided only when schemaName is used. |
| schemaSetLabel | M | A label identifier | A set of schemas may have a unique schemaSetLabel identifier associated with them. Schema sets containing different schemas may not have the same schemaSetLabel. It is the responsibility of the MnS producer to define and set the schemaSetLabel. |

Only one of schemaName or schemaSetLabel may be used in a single operation request.

##### 11.1.1.x.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| schemaList | M | LIST of Schema information | The list of Schemas matching the query criteria |
| > name | M | Name of the schema | Name property of an individual schema |
| > version | M | Version of the schema | Version property of an individual schema |
| > namespace | O | Namespace of the schema | Namespace property of an individual schema |
| > contentLocation | O | Location of the schema content | Location of an individual schema content |
| schemaFormat | M | Format of the schema content | Format of the Schema content returned from contentLocation referenced above. All schemas with content referenced by contentLocation in the schemaList returned in this response must be of the same schemaFormat. |
| Status | M | ENUM (OPERATION_SUCCEEDED, OPERATION_FAILED) | An operation may fail because of a specified or unspecified reason. |

If no input parameters are provided, then the full list of schema information related to network data managed by the MnS shall be returned.

##### 11.1.1.x.4	Results

In case of success, all the schemas in the scope of the request are returned. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

3rd Change

#### 11.1.1.x	getSchemaManifest operation

##### 11.1.1.x.1	Description

This operation is invoked by MnS consumer to request the schema manifest for one or more root object instances managed by the MnS producer.  One or more schema manifests may be retrieved depending on the parameters provided.

##### 11.1.1.x.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| rootObjectInstances | O | List<DN> | List of object instances. Each object instance is identified by its DN and designates the root of a subtree that contains the root object and all descendant objects |

##### 11.1.1.x.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| schemaManifestList | M | List of Schema Manifests | A list of Schema Manifests associated with the input rootObjectInstances |
| > rootObjectInstanceId | M | DN of the rootObjectInstance | DN of the rootObjectInstanceId. It designates the root of a subtree that contains the root object and all descendant objects |
| > schemaSetLabel | O | Schema set label identifier | Label that uniquely identifies a related set of schemas |
| > schemaFormat | M | Format of the schema content | Format of the Schema content returned from contentLocation referenced in the schemaList below. All schemas with content referenced by contentLocation in the schemaList returned in this response must be of the same schemaFormat. |
| > schemaList | M | List of Schemas | The list of Schemas matching the query criteria |
| >> name | M | Name of the schema | Name property of an individual schema |
| >> version | M | Version of the schema | Version property of an individual schema |
| >> namespace | O | Namespace of the schema | Namespace property of an individual schema |
| >> contentLocation | O | Location of the schema content | location of an individual schema content |
| >> features | O | List of schema features | List of schema features |
| >> deviations | O | List of schema deviations | List of schema deviations |
| >> import-only-schemas | O | Import only schema list | List of all import-only schemas |
| >>> name | M | Name of the schema | Name of the import-only schema |
| >>> version | M | Version of the schema | Version of the import-only schema |
| >>> namespace | O | Namespace of the schema | Namespace of the import-only schema |
| >>> contentLocation | O | Location of the schema content | Location of the import-only schema content |
| Status | M | ENUM (OPERATION_SUCCEEDED, OPERATION_FAILED) | An operation may fail because of a specified or unspecified reason. |

##### 11.1.1.x.4	Results

In case of success, all the schema manifests in the scope of the request are returned. In case of failure, a specified or unspecified reason may be provided in the Output parameters.

4th Change

#### 11.1.1.x	getSchemaContent operation

##### 11.1.1.x.1	Description

This operation is invoked by MnS consumer to request the Schema Content for a Schema managed by the MnS producer.  One or more schemas may be retrieved depending on the parameters provided.

##### 11.1.1.x.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| schemaName | M | Name of the schema | The name of the schema. This parameter shall not be used if schemaSetLabel is used. |
| schemaVersion | CO | Version of the schema | The version of the schema. If it is not provided, then the latest version shall be inferred. The schemaVersion may be provided only when schemaName is used. |
| schemaSetLabel | M | A label identifier | A set of schemas may have a unique schemaSetLabel identifier associated with them. Schema sets containing different schemas may not have the same schemaSetLabel. It is the responsibility of the producer to define and set the schemaSetLabel. This parameter shall not be used if schemaName is used. |

Only one of schemaName or schemaSetLabel may be used in a single operation request.

##### 11.1.1.x.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| schemaContentList | M | List of Schema content | The List of Schema content matching the query criteria. If schemaName is used in the operation, then only a single schema content is returned |
| Status | M | ENUM (OPERATION_SUCCEEDED, OPERATION_FAILED) | An operation may fail because of a specified or unspecified reason. |

##### 11.1.1.x.4	Results

In case of success, the content of all the Schemas in the scope of the request are returned.  In case of failure, a specified or unspecified reason may be provided in the Output parameters.