3GPP TSG-SA5 Meeting #153 	S5-240659

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0317 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.532 Correction of attribute description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-01-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are references to "Generalised Time" from ITU standard. Since we have already agreed and implemented the use of RFC 3339 for time format, remaining referencees to ITU "Generalised Time" should be replaced by reference to the RFC in this TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove "Generalised Time" and refer to RFC 3339 section 5.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TS refers to a non-existing terminology and lacks detail information about Date/Time Format in RFC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.7.2, 11.1.1.8.2, 11.1.1.9.2, 11.1.1.10.2, 11.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 11.1.1.7.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies a new instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject MOI. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOICreation” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOICreation event time. | See RFC 3339 [52] section 5.6 for details. |
| systemDN | M | It shall carry the DN of management service providers. | - |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeList | O | LIST OF SEQUENCE <AttributeName, AttributeValue> | The attributes (name/value pairs) of the created MOI. |

Next change

##### 11.1.1.8.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies an existing instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject MOI. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOIDeletion” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOIDeletion event time. | See RFC 3339 [52] section 5.6 for details. |
| systemDN | M | It shall carry the DN of management service providers. | - |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeList | O | LIST OF SEQUENCE <AttributeName, AttributeValue> | The attributes (name/value pairs) of the deleted MOI. |

Next change

##### 11.1.1.9.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | It specifies the class name of the IOC. A network event has occurred in an instance of this class. |
| objectInstance | M | ManagedEntity.objectInstance | It specifies the existing instance of the above IOC in which the network event related to by carrying the Distinguished Name (DN) for the instance. |
| notificationId | M | This is an identifier for the notification, which may be used to correlate notifications. | The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the subject Information Object. |
| notificationType | M | It specifies the type of provisioning management services related notifications. The value “notifyMOIAttributeValueChange” shall be carried. | It specifies the type of notification. |
| eventTime | M | It indicates the MOIAttributeValueChange event time. | See RFC 3339 [52] section 5.6 for details.. |
| systemDN | M | It shall carry the DN of management service providers. | - |
| correlatedNotifications | CM | It specifies a set of notifications that are correlated to the subject notification. | The condition is that the MnS producer support the correlation of notifications |
| additionalText | O | It can contain further information in text on the event of the ManagedEntity(s). | - |
| sourceIndicator | O | ENUM( Resource_operation, Management_operation, SON_operation,Unknown) | This parameter, when present, indicates the source of the operation that led to the generation of this notification. It can have one of the following values: 1. resource operation: The notification was generated in response to an internal operation of the resource; 2. management operation: The notification was generated in response to a management operation applied across the managed object boundary external to the managed object; 3. SON operation: The notification was generated as result of a SON (Self Organising Network) process like self-configuration, self-optimization, self-healing etc. . 4. unknown: It is not possible to determine the source of the operation. Remark: A provisioning MnS provider may not in any case be aware that SON operation lead to the generation of this generation. In this case another value than SON_operation for sourceIndicator might be sent. |
| attributeValueChange | M | LIST OF SEQUENCE <AttributeName, NewAttributeValue, CHOICE [NULL, OldAttributeValue]> | The changed attributes (name/value pairs) of the MOI (with both new and, optionally, old values). |

Next change

##### 11.1.1.10.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | ManagedEntity.objectClass | -- |
| objectInstance | M | ManagedEntity.objectInstance | -- |
| notificationId | M | It carries the identifier for the subject notification. | See Note 1. |
| eventTime | M | It indicates the time of the event. | See RFC 3339 [52] section 5.6 for details.. |
| systemDN | M | It carries the DN of producer of the notification. | -- |
| notificationType | M | "notifyEvent" | -- |
| specificProblem | M | It indicates a problem detected. | -- |
| additionalText | O | It carries additional information. | -- |
| additionalInformation | O | It carries additional information. | -- |
| NOTE 1: If consumer receives notifications from one producer, consumer can use the notificationId and the objectInstance to uniquely identify all received notifications. If consumer receives notifications from multiple producers and notifications of each objectInstance are reported to at most by one producer, consumer can use the notificationId and objectInstance to uniquely identify all received notifications. If consumer receives notifications from multiple producers and notifications of one or more objectInstance(s) are reported byo two or more producers, consumer can use the notificationId together with objectInstance and the identity of producer (systemDN), to uniquely identify all received notifications. If the information systemDN is absent, consumer needs other means, which are outside the scope of this TS, to determine the identity of producer. How notificationId of notifications are re-used to correlate notifications is outside of the scope of this specification. |  |  |  |

Next change

##### 11.4.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | HeartbeatControl.objectClass |  |
| objectInstance | M | HeartbeatControl.objectInstance | Instance controlling the emission of this notifyHeartbeat notification. |
| notificationId | M | -- |  |
| notificationType | M | "notifyHeartbeat" |  |
| eventTime | M | -- | Time at which the notification is emitted. See RFC 3339 [52] section 5.6 for details.. |
| systemDN | M | MnSAgent.objectInstance |  |
| heartbeatNtfPeriod | M | HeartbeatControl.heartbeatNtfPeriod |  |

End of changes