# 3GPP TSG-SA5 Meeting #153	S5-240924

# Sevilla, Spain, January 29 – February 2, 2024	was S5-240189

# Source:	Nokia

# Title:	Rel-18 pCR 28.318 Annex A use case 1 procedure

# Document for:	Approval

Agenda Item:	6.6.4.3 (NSOEU_WoP#3)

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] S5-240019	DRAFT TS 28.318 v0.3.0, captures the agreements of SA5 152, basis for contributions for SA5 153

[2] TS 28.552		5G performance measurements

# 3	Rationale

The baseline for 28.318 into SA5#153 has been provided as S5-240019 [1]. This pCR uses [1] as baseline with changemarks.

This pCR updates Annex A (informative) Procedures A.1 exposing performance and prediction information for high availability.

This pCR includes reference to CellAvailabilityNRCellDU which has dependency on S5-240356, S5-240357, S5-240625, S5-240627.

This pCR includes reference to CellAvailAvgTimeCU and NwAvailAvgTimeRAN, which have dependency on S5-240356 and S5-240358.

# 4	Detailed proposal

Based on possible gNB implementations, non-split and split, various solutions are available. Some performance measurements are defined for specific gNB implementations [1], [2]. Therefore, this pCR provides procedures for non-split gNB and split gNB with corresponding PMs allocated to the appropriate procedure.

It is proposed to agree to the following improvements to Annex A [1] TS 28.318, 0.2.0.

Updated in revision 1

- Clarified in §3 Rationale the tdocs that also have impact to CellAvailabilityNRCellDU, CellAvailAvgTimeCU and NwAvailAvgTimeRAN.

- Removed ‘full’ from all instances of ‘full Distinguished Name’ and ‘full DN’.

- Replace PLMN_ID with PLMN

- Removed ‘Recipient’ in all cases so that notifications are sent to DSO Consumer

- Updated granularityPeriod, fileReportingPeriod, monitorGranularityPeriod and Threshold values in a.1.2 split-gNB scenario as requested.

Updated in revision 2

- Merged the relevant section of S5-240557

Begin Change

# 8	Stage 3 definitions

Annex A (informative): 
Procedures

## A.1	Exposing performance and prediction information for high availability

### A.1.1	DSO is trusted and supported by network operator, non-split gNB

#### A.1.1.1	General

In this scenario the DSO consumer, is recognized by the network operator as a trusted entity that is allowed to read from and write to selected producers and corresponding MOIs in one or more MIBs controlled by the network operator. In this basic scenario, the network operator supports the DSO consumer by providing specific necessary information such as a Distinguished Name of producer(s) and MOI(s), by some means not described in this TS. This scenario is specific to a non-split gNB.

#### A.1.1.2	Signal flow

The DSO, making use of a DSO managed service consumer (DSO consumer) wants to receive performance metric reports from one or more specific cell sites. These reports are provided continuously over the monitoring interval with the requested granularity.  The DSO consumer is in possession of the following information:

the identity of the cell site: base station ID, or

the estimated location of the cell site antenna

NOTE: 	Based upon the identity of the cell site, the location of the cell site, or both, the MNO supplies to the DSO the DN and any required scoping parameters for PerfMetricJob MOI(s), ThresholdMonitor MOI(s) and NtfSubscriptionControl MOI(s).

The MNO exposes a MnS producer (termed in this clause 'network operator producer').

Figure A.1.1.2-1 Create PerfMetricJob

- PerfMetricJob

- The DSO consumer sends createMOI to the producer with the following parameters: managedObjectClass = PerfMetricJob, managedObjectInstance = {DN supplied by the MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| jobId | pmj_group_0001 |
| performanceMetrics | DRB.AirIfDelayDL_PLMN, DRB.AirIfDelayUl_PLMN, DRB.UEThpDL.PLMN, DRB.UEThpUL.PLMN, CellAvailabilityNRCellDU |
| granularityPeriod | 60 |
| objectInstances | included if supplied by the MNO |
| rootObjectInstances | included if supplied by the MNO |
| reportingCtrl | fileReportingPeriod = 3 notificationRecipientAddress = URL |
| _linkToFiles | - |

jobID is chosen by the DSO, this example uses jobID = pmj_group_0001 to identify the performance measurements from one cell or group of cells as identified by the MNO-supplied DN and optionally objectInstances & rootObjectInstances.

PLMN used to specify the performance metrics. It is supplied by MNO.

granularityPeriod is chosen by the DSO, this example uses 60 seconds.

reportingCtrl uses implicit notification subscription. The fileReportingPeriod is chosen by the DSO, this example uses 3 minutes. The notificationRecipientAddress is chosen by the DSO, in this example it is the URL where the notifyMOICreation and notifyMOIDeletion notification types shall be sent.

2.	The network operator producer creates “PerfMetricJob” data collection job, Files and NtfSubscriptionControl MOIs.

3. The network operator producer sends notifyMOICreation to the notificationRecipientAddress included in reportingCtrl.

4.	The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

On OperationSucceeded, metric production is now active on the in-scope object instances whose object class matches the object class associated to the specified performance metrics. A notification subscription has been created.

Repeat steps 1 - 4 for each cell site (or group of cell sites as determined by DN information provided by MNO) where performance metric information is wanted.

Figure A.1.1.2-2 Create ThresholdMonitor (rapid)

B.1		ThresholdMonitor for rapidly changing PMs

5.	The DSO consumer sends createMOI for ThresholdMonitor to monitor the performance measurements for rapidly changing PMs including measurement for latency, packet loss and throughput, using the following parameters: managedObjectClass = ThresholdMonitor, managedObjectInstance = {DN supplied by MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| thresholdInfoList | ThresholdInfo performanceMetrics=DRB.AirIfDelayDl_PLMN thresholdDirection=UP_AND_DOWN thresholdValue=70.0 hysteresis=20.0 ThresholdInfo performanceMetrics=DRB.AirIfDelayUl_PLMN thresholdDirection=UP_AND_DOWN thresholdValue=70.0 hysteresis=20.0 ThresholdInfo performanceMetrics=DRB.UEThpDl.PLMN (kbit/s) thresholdDirection= UP_AND_DOWN thresholdValue=100000.0 Hysteresis=10000.0 ThresholdInfo performanceMetrics=DRB.UEThpUl.PLMN (kbit/s) thresholdDirection= UP_AND_DOWN thresholdValue=10000.0 Hysteresis=1000.0 |
| monitorGranularityPeriod | 180 |
| objectInstances | included if supplied by the MNO |
| rootObjectInstances | included if supplied by the MNO |

In this example, a ThresholdInfo datatype will be included for each performance metric that was included in the creation of the corresponding PerfMetricJob MOI where rapidly threshold monitoring is desired.

A threshold monitor is created for delay at the air interface in both DL and UL. In this example, the threshold is set at 7 ms with hysteresis of 2 ms in both the DL and UL. PLMN is provided by the MNO.

A threshold monitor is created for UE throughput in both the DL and UL. In this example, the threshold is set at 100 Mbps with hysteresis of 20 Mbps in the DL and at 10 Mbps with hysteresis of 1 Mbps in the UL.

monitorGranularityPeriod =  180 seconds, multiple of 3 X granularityPeriod established in the PerfMetricJob above.

scope information is included if provided by the MNO.

[ThresholdInfo will also contain an attribute for the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.]

6.	The network operator producer creates “ThresholdMonitor” MOI.

7. The network operator producer sends notifyMOICreation to the notificationRecipientAddress in step 1.

8.	The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 5 - 8 for each cell site (or group of cell sites as determined by DN information provided by MNO)  where rapidly changing threshold monitoring is required.

Figure A.1.1.2-3 Create ThresholdMonitor (slow)

B.2		ThresholdMonitor for slowly changing PMs

9.	The DSO consumer sends createMOI for a second ThresholdMonitor to monitor the performance measurements for slowly changing PMs like cell availability, using the following parameters: managedObjectClass = ThresholdMonitor, managedObjectInstance = {DN supplied by MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| thresholdInfoList | ThresholdInfo performanceMetrics=CellAvailabilityNRCellDU thresholdDirection=UP_AND_DOWN thresholdValue=99.0 hysteresis=0.5 |
| monitorGranularityPeriod | 3600 |
| objectInstances | included if supplied by the MNO |
| rootObjectInstances | included if supplied by the MNO |

In this example, a ThresholdInfo datatype will be included for each performance metric that was included in the creation of the corresponding PerfMetricJob MOI where slowly changing threshold monitoring is desired.

A threshold monitor is created for cell availability. In this example, the threshold is set at 99.0% with hysteresis of 0.5%.

monitorGranularityPeriod =  3600 seconds (1 hour), multiple of 3 X granularityPeriod established in the PerfMetricJob above.

scope information is included if provided by the MNO.

[ThresholdInfo will also contain an attribute for the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.]

10.	The network operator producer creates “ThresholdMonitor” MOI.

11. The network operator producer sends notifyMOICreation to the notificationRecipientAddress in step 1.

12.	The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 9 – 12 for each cell site (or group of cell sites as determined by DN information provided by MNO)  where slowly changing threshold monitoring is required.

Figure A.1.1.2-4 Create NtfSuscriptionControl

- NtfSubscriptionControl

13.	The DSO consumer sends createMOI for NtfSubscriptionControl to subscribe for the required notifications with the following parameters: managedObjectClass = NtfSubscriptionControl, managedObjectInstance = {DN supplied by the MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| notificationRecipientAddress | URL |
| notificationTypes | notifyFileReady, notifyFilePreparationError, notifyThresholdCrossing |
| scope | included if supplied by the MNO |
| notificationFilter | - |

notificationRecipientAddress is specified by the DSO and is the URL where the notifications shall be sent. In this example it is the same URL as provided to PerfMetricJob in step 1 above, although the DSO consumer may select a different URL in this current step.

notificationTypes = notifyFileReady, notifyFilePreparationError, notifyThresholdCrossing

scope information is included if provided by the MNO.

14.	The network operator producer creates “NtfSubscriptionControl” job.

15. The network operator producer sends notifyMOICreation to the notificationRecipientAddress in step 1.

16.	The producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 13 - 16 for each cell site (or group of cell sites as determined by DN information provided by MNO) where threshold monitoring and file notification are required.

A.1.1.3	Signal flow – completion

The DSO, making use of a DSO managed service consumer (DSO consumer) wants to end performance metric reports from one or more specific cell sites which were previously setup for performance metric reporting. For ultimate deactivation of metric production and threshold monitoring the MnS consumer should delete the job and monitor to free up resources on the MnS producer.

NOTE: 	dMNO, making use of a MnS producer (termed in this clause ‘network operator producer’) provides to the DSO the DN and any required scoping parameters for PerfMetricJob MOI(s), ThresholdMonitor MOI(s) and NtfSubscriptionControl MOI(s).

Figure A.1.1.3-1

- PerfMetricJob

- The DSO consumer sends deleteMOI to the producer with the following parameters: baseObjectInstance = {DN supplied by the MNO}, scopeType, scopeLevel and Filter are optional and shall be included if supplied by the MNO.

- The network operator producer deletes “PerfMetricJob” data collection job.

- The network operator producer sends notifyMOIDeletion to the notificationRecipientAddress in section C, step 9 above.

- The network operator producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

Best effort synchronization applies; all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it. The managed objects selected for this operation are specified by information provided by the MNO.

When the "PerfMetricJob" is deleted, the ongoing reporting period shall be aborted.

- ThresholdMonitor

- The DSO consumer sends deleteMOI to the producer with the following parameters: baseObjectInstance = {DN supplied by the MNO}, scopeType, scopeLevel and Filter are optional and shall be included if supplied by the MNO.

- The network operator producer deletes “ThresholdMonitor” data collection job.

- The network operator producer sends notifyMOIDeletion to the notificationRecipientAddress in section C, step 9 above.

- The network operator producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

Best effort synchronization applies; all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it.

- NtfSubscriptionControl

- When the DSO consumer does not wish to receive notifications any more, it shall delete the corresponding NtfSubscriptionControl instance. The DSO consumer sends deleteMOI to the producer with the following parameters: baseObjectInstance = {DN supplied by the MNO}, scopeType, scopeLevel and Filter are optional and shall be included if supplied by the MNO.

- The network operator producer deletes “NtfSubscriptionControl” job.

- The network operator producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

### A.1.2	DSO is trusted and supported by network operator, split gNB

#### A.1.2.1	General

In this scenario the DSO consumer, is recognized by the network operator as a trusted entity that is allowed to read from and write to selected producers and corresponding MOIs in one or more MIBs controlled by the network operator. In this basic scenario, the network operator supports the DSO consumer by providing specific necessary information such as a Distinguished Name of producer(s) and MOI(s), by some means not described in this TS. This scenario is specific to a split gNB.

#### A.1.2.2	Signal flow

The DSO, making use of a DSO managed service consumer (DSO consumer) wants to receive performance metric reports from one or more specific cell sites. These reports are provided continuously over the monitoring interval with the requested granularity.  The DSO consumer is in possession of the following information:

the identity of the cell site: base station ID, or

the estimated location of the cell site antenna

NOTE: 	Based upon the identity of the cell site, the location of the cell site, or both, the MNO supplies to the DSO the DN and any required scoping parameters for PerfMetricJob MOI(s), ThresholdMonitor MOI(s) and NtfSubscriptionControl MOI(s).

The MNO exposes a MnS producer (termed in this clause 'network operator producer').

Figure A.1.2.2-1 Create PerfMetricJob

- PerfMetricJob

- The DSO consumer sends createMOI to the network operator producer with the following parameters: managedObjectClass = PerfMetricJob, managedObjectInstance = {DN supplied by the MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| jobId | pmj_group_0001 |
| performanceMetrics | DRB.AirIfDelayDL_PLMN, DRB.AirIfDelayUl_PLMN, DRB.UEThpDL.PLMN, DRB.UEThpUL.PLMN, DRB.PacketLossRateUL SO.CellInServiceTotal.NCGI SO.CellOOSTimeTotal.NCGI SO.CellOOSTimeAverage.NCGI CellAvailAvgTimeCU NwAvailAvgTimeRAN |
| granularityPeriod | 7200 |
| objectInstances | included if supplied by the MNO |
| rootObjectInstances | included if supplied by the MNO |
| reportingCtrl | fileReportingPeriod = 120 notificationRecipientAddress = URL |
| _linkToFiles | - |

jobID is chosen by the DSO, this example uses jobID = pmj_group_0001 to identify the performance measurements from one cell or group of cells as identified by the MNO-supplied DN and optionally objectInstances & rootObjectInstances.

PLMN used to specify the performance metrics. It is supplied by MNO.

granularityPeriod is chosen by the DSO, this example uses 7200 seconds (2 hours).

reportingCtrl uses implicit notification subscription. The fileReportingPeriod is chosen by the DSO, this example uses 120 minutes. The notificationRecipientAddress is chosen by the DSO, in this example it is the URL where the notifyMOICreation and notifyMOIDeletion notification types shall be sent.

2.	The network operator producer creates “PerfMetricJob” data collection job, Files and NtfSubscriptionControl MOIs.

3. The network operator producer sends notifyMOICreation to the notificationRecipientAddress included in reportingCtrl.

4.	The network operator producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

On OperationSucceeded, metric production is now active on the in-scope object instances whose object class matches the object class associated to the specified performance metrics. A notification subscription has been created.

Repeat steps 1 - 4 for each cell site (or group of cell sites as determined by DN information provided by MNO) where performance metric information is wanted.

Figure A.1.2.2-2 Create ThresholdMonitor (rapid)

B.1		ThresholdMonitor for rapidly changing PMs

5.	The DSO consumer sends createMOI for ThresholdMonitor to monitor the performance measurements for rapidly changing PMs including measurement for latency, packet loss and throughput, using the following parameters: managedObjectClass = ThresholdMonitor, managedObjectInstance = {DN supplied by MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| thresholdInfoList | ThresholdInfo performanceMetrics=DRB.AirIfDelayDl_PLMN thresholdDirection=UP_AND_DOWN thresholdValue=70.0 hysteresis=20.0 ThresholdInfo performanceMetrics=DRB.AirIfDelayUl_PLMN thresholdDirection=UP_AND_DOWN thresholdValue=70.0 hysteresis=20.0 ThresholdInfo performanceMetrics=DRB.UEThpDl.PLMN (kbit/s) thresholdDirection= UP_AND_DOWN thresholdValue=100000.0 Hysteresis=10000.0 ThresholdInfo performanceMetrics=DRB.UEThpUl.PLMN (kbit/s) thresholdDirection= UP_AND_DOWN thresholdValue=10000.0 Hysteresis=1000.0 ThresholdInfo performanceMetrics=DRB.PacketLossRateUl (1 ~ 1E6) thresholdDirection=UP thresholdValue=20000 Hysteresis=_ |
| monitorGranularityPeriod | 14400 |
| objectInstances | included if supplied by the MNO |
| rootObjectInstances | included if supplied by the MNO |

In this example, a ThresholdInfo datatype will be included for each performance metric that was included in the creation of the corresponding PerfMetricJob MOI where rapidly threshold monitoring is desired.

A threshold monitor is created for delay at the air interface in both DL and UL. In this example, the threshold is set at 7 ms with hysteresis of 2 ms in both the DL and UL. PLMN is provided by the MNO.

A threshold monitor is created for UE throughput in both the DL and UL. In this example, the threshold is set at 100 Mbps with hysteresis of 20 Mbps in the DL and at 10 Mbps with hysteresis of 1 Mbps in the UL.

A threshold monitor is created for UL PacketLoss. In this example, the threshold is set at 0.02% packetloss.

monitorGranularityPeriod =  14400 seconds, multiple of 2 X granularityPeriod established in the PerfMetricJob above.

scope information is included if provided by the MNO.

[ThresholdInfo will also contain an attribute for the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.]

6.	The network operator producer creates “ThresholdMonitor” MOI.

7. The network operator producer sends notifyMOICreation to the notificationRecipientAddress in step 1.

8.	The network operator producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 5 - 8 for each cell site (or group of cell sites as determined by DN information provided by MNO)  where rapidly changing threshold monitoring is required.

Figure A.1.2.2-3 Create ThresholdMonitor (slow)

B.2		ThresholdMonitor for slowly changing PMs

9.	The DSO consumer sends createMOI for a second ThresholdMonitor to monitor the performance measurements for slowly changing PMs like cell availability, using the following parameters: managedObjectClass = ThresholdMonitor, managedObjectInstance = {DN supplied by MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| administrativeState | - |
| operationalState | - |
| thresholdInfoList | ThresholdInfo performanceMetrics= SO.CellOOSTimeAverage.NCGI thresholdDirection=UP_AND_DOWN thresholdValue=1800 ThresholdInfo performanceMetrics= CellAvailAvgTimeCU thresholdDirection=UP_AND_DOWN thresholdValue=600 ThresholdInfo performanceMetrics= NwAvailAvgTimeRAN thresholdDirection=UP_AND_DOWN thresholdValue=300 |
| monitorGranularityPeriod | 43200 |
| objectInstances | included if supplied by the MNO |
| rootObjectInstances | included if supplied by the MNO |

In this example, a ThresholdInfo datatype will be included for each performance metric that was included in the creation of the corresponding PerfMetricJob MOI where slowly changing threshold monitoring is desired.

Three threshold monitors are created for cell availability. In this example, the threshold is set at 1800 seconds (30 minutes) for metric SO.CellOOSTimeAverage.NCGI.

monitorGranularityPeriod =  43200 seconds (12 hours), multiple of 6 X granularityPeriod established in the PerfMetricJob above.

scope information is included if provided by the MNO.

[ThresholdInfo will also contain an attribute for the location (Lat/long, TAC, cellid). This is used to scope the object instance to be monitored.]

10.	The network operator producer creates “ThresholdMonitor” MOI.

11. The network operator producer sends notifyMOICreation to the notificationRecipientAddress in step 1.

12.	The network operator producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 9 – 12 for each cell site (or group of cell sites as determined by DN information provided by MNO)  where slowly changing threshold monitoring is required.

Figure A.1.2.2-4 Create NtfSuscriptionControl

- NtfSubscriptionControl

13.	The DSO consumer sends createMOI for NtfSubscriptionControl to subscribe for the required notifications with the following parameters: managedObjectClass = NtfSubscriptionControl, managedObjectInstance = {DN supplied by the MNO}, attributeListIn as defined in the following table:

| Attribute name | Value |
| --- | --- |
| notificationRecipientAddress | URL |
| notificationTypes | notifyFileReady, notifyFilePreparationError, notifyThresholdCrossing |
| scope | included if supplied by the MNO |
| notificationFilter | - |

notificationRecipientAddress is specified by the DSO and is the URL where the notifications shall be sent. In this example it is the same URL as provided to PerfMetricJob in step 1 above, although the DSO consumer may select a different URL in this current step.

notificationTypes = notifyFileReady, notifyFilePreparationError, notifyThresholdCrossing

scope information is included if provided by the MNO.

14.	The network operator producer creates “NtfSubscriptionControl” job.

15. The network operator producer sends notifyMOICreation to the notificationRecipientAddress in step 1.

16.	The network operator producer sends createMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| attributeListOut | name / value pairs of the attributes of the new object |
| status | OperationSucceeded ¦ OperationFailed |

The DSO consumer stores the attribute name/value pairs.

Repeat steps 13 - 16 for each cell site (or group of cell sites as determined by DN information provided by MNO) where threshold monitoring and file notification are required.

A.1.2.3	Signal flow – completion

The DSO, making use of a DSO managed service consumer (DSO consumer) wants to end performance metric reports from one or more specific cell sites which were previously setup for performance metric reporting. For ultimate deactivation of metric production and threshold monitoring the MnS consumer should delete the job and monitor to free up resources on the MnS producer.

NOTE: 	MNO, making use of a MnS producer (termed in this clause ‘network operator producer’) provides to the DSO the DN and any required scoping parameters for PerfMetricJob MOI(s), ThresholdMonitor MOI(s) and NtfSubscriptionControl MOI(s).

Figure A.1.2.3-1

- PerfMetricJob

- The DSO consumer sends deleteMOI to the producer with the following parameters: baseObjectInstance = {DN supplied by the MNO}, scopeType, scopeLevel and Filter are optional and shall be included if supplied by the MNO.

- The network operator producer deletes “PerfMetricJob” data collection job.

- The network operator producer sends notifyMOIDeletion to the notificationRecipientAddress in section C, step 9 above.

- The producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

Best effort synchronization applies; all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it. The managed objects selected for this operation are specified by information provided by the MNO.

When the "PerfMetricJob" is deleted, the ongoing reporting period shall be aborted.

- ThresholdMonitor

- The DSO consumer sends deleteMOI to the network operator producer with the following parameters: baseObjectInstance = {DN supplied by the MNO}, scopeType, scopeLevel and Filter are optional and shall be included if supplied by the MNO.

- The network operator producer deletes “ThresholdMonitor” data collection job.

- The network operator producer sends notifyMOIDeletion to the notificationRecipientAddress in section C, step 9 above.

- The producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

Best effort synchronization applies; all managed objects selected for this operation will perform the operation if possible regardless of whether some managed objects fail to perform it.

- NtfSubscriptionControl

- When the DSO consumer does not wish to receive notifications any more, it shall delete the corresponding NtfSubscriptionControl instance. The DSO consumer sends deleteMOI to the network operator producer with the following parameters: baseObjectInstance = {DN supplied by the MNO}, scopeType, scopeLevel and Filter are optional and shall be included if supplied by the MNO.

- The network operator producer deletes “NtfSubscriptionControl” job.

- The network operator producr sends deleteMOI response to the DSO consumer with output parameters as defined in the following table:

| Attribute name | Value |
| --- | --- |
| deletionList | LIST OF SEQUENCE <ManageedEntity DN, ManagedEntity class name> |
| status | OperationSucceeded ¦ OperationFailed ¦ OperationPartiallySucceeded |

Annex B (informative):
Coordinated energy service recovery business relationship model

Annex X: UML source code

Figure A.1.1.2-1

'S5-240924 Figure A.1.1.2-1 Create PerfMetricJob

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group A. create PerfMetricJob MOI on producer

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 1. createMOI(managedObjectClass, managedObjectInstance, attributeListIn)

Note right Cons

DN and optionally scoping DNs provided

by MNO. Attributes chosen by DSO

End Note

Prod  -> Prod: 2. Create MOIs:\n    PerfMetricJob, Files,\n    NtfSubscriptionControl

Cons <- Prod: 3. Notification (notifyMOICreation)

Cons <-  Prod: 4. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded, PerfMetricJob is created.

PM production is active, reporting has begun. A

notification subscription has been created.

End Note

end loop for each wanted MOI

loop Measurement

hnote over Prod

Performance measurements are collected at each

granularityPeriod, stored, and reported according

to fileReportingPeriod.

end note

Cons <- Prod: Notification (notifyMOICreation)

end loop

end group A.

...

@enduml

Figure A.1.1.2-2

'S5-240924 Figure A.1.1.2-2 Create ThresholdMonitor (rapid)

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group B.1 create Threshold MOI on producer (rapid changes)

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 5. createMOI(class, DN, attributeListIn)

Note right Cons

DN and optionally scoping DNs provided by MNO.

Attributes chosen by DSO. Selected performance metric

attributes must align with those on the PerfMetricJob

on the same producer.

End Note

Prod  -> Prod: 6. CreateMOI( )\n    ThresholdMonitor

Cons <- Prod: 7. Notification (notifyMOICreation)

Cons <-  Prod: 8. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded, ThresholdMonitor is

created. Monitoring is active.

End Note

end loop

loop Monitoring

hnote over Prod

Monitor measurements. There is not yet a

subscription for notifyThresholdCrossing.

end note

end loop

end group B

...

@enduml

Figure A.1.1.2-3

'S5-240924 Figure A.1.1.2-3 Create ThresholdMonitor (slow)

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

'participant "Recipient" as Recip

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group B.2 create Threshold MOI on producer (slow changes)

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 9. createMOI(class, DN, attributeListIn)

Note right Cons

DN and optionally scoping DNs provided by MNO.

Attributes chosen by DSO. Selected performance metric

attributes must align with those on the PerfMetricJob

on the same producer.

End Note

Prod  -> Prod: 10. CreateMOI( )\n    ThresholdMonitor

Cons <- Prod: 11. Notification (notifyMOICreation)

Cons <-  Prod: 12. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded, ThresholdMonitor is

created. Monitoring is active.

End Note

end loop

loop Monitoring

hnote over Prod

Monitor measurements. There is not yet a

subscription for notifyThresholdCrossing.

end note

end loop

end group B

...

@enduml

Figure A.1.1.2-4

'S5-240924 Figure A.1.1.2-4 Create NtfSuscriptionControl

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group C. create NtfSubscriptionControl MOI on producer

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 13. createMOI(class, DN, attributeListIn)

Note right Cons

DN provided by site operator. Scoping

DNs optionally provided by site operator.

Attributes are chosen by DSO.

End Note

Prod  -> Prod: 14. CreateMOI( )\n     NtfSubscriptionControl

Cons <- Prod: 15. Notification (notifyMOICreation)

Cons <-  Prod: 16. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded,

NtfSubscriptionControl

is created.

End Note

end loop

loop Monitoring

hnote over Prod

Performance Measurements, ThresholdMonitor

and Notifications are active

end note

Cons <-  Prod: Notification (notifyThresholdCrossing)

Cons <-  Prod: Notification (notifyFileReady)

end loop

end group

@enduml

Figure A.1.1.3-1

'S5-240924 Figure A.1.1.3-1

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

participant "network operator producer" as Prod

== DSO consumer wants to close & deactivate the performance monitor session ==

group A. delete PerfMetricJob MOI on producer

loop Delete: for each created cell or group of cells (information provided by MNO)

Cons  -> Prod: 1. deleteMOI(baseObjectInstance [, scopeType, scopeLevel, Filter])

Note right Cons

DN and, optionally, parameters

are supplied by MNO.

End Note

Prod  -> Prod: 2. delete MOI:\n    PerfMetricJob, Files

Cons <- Prod: 3. Notification (notifyMOIDeletion)

Cons <-  Prod: 4. result of deleteMOI(deletionList, status)

hnote over Prod

ongoing PM reporting

is abandoned.

end note

end loop for each wanted MOI

end group A.

...

group B. delete Threshold MOI on producer

loop Delete: for each created cell or group of cells (information provided by MNO)

Cons  -> Prod: 5. deleteMOI(baseObjectInstance [, scopeType, scopeLevel, Filter])

Note right Cons

DN and, optionally, parameters

are supplied by MNO.

End Note

Prod  -> Prod: 6. deleteMOI( )\n    ThresholdMonitor

Cons <- Prod: 7. Notification (notifyMOIDeletion)

Cons <-  Prod: 8. result of deleteMOI(deletionList, status)

hnote over Prod

ongoing threshold

monitoring is abandoned

end Note

end loop

end group B

...

group C. delete NtfSubscriptionControl MOI on producer

loop Delete: for each created cell or group of cells (information provided by MNO)

Cons  -> Prod: 9. deleteMOI(baseObjectInstance [, scopeType, scopeLevel, Filter])

Note right Cons

DN and, optionally, parameters

are supplied by MNO.

End Note

Prod  -> Prod: 10. deleteMOI( )\n     NtfSubscriptionControl

Cons <-  Prod: 11. result of deleteMOI(attributeListOut, status)

hnote over Prod

ongoing notification

subscription is ended

End Note

end loop

end group

@enduml

Figure A.1.2.2-1

'S5-240924 Figure A.1.2.2-1 Create PerfMetricJob

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group A. create PerfMetricJob MOI on producer

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 1. createMOI(managedObjectClass, managedObjectInstance, attributeListIn)

Note right Cons

DN and optionally scoping DNs provided

by MNO. Attributes chosen by DSO

End Note

Prod  -> Prod: 2. Create MOIs:\n    PerfMetricJob, Files,\n    NtfSubscriptionControl

Cons <- Prod: 3. Notification (notifyMOICreation)

Cons <-  Prod: 4. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded, PerfMetricJob is created.

PM production is active, reporting has begun. A

notification subscription has been created.

End Note

end loop for each wanted MOI

loop Measurement

hnote over Prod

Performance measurements are collected at each

granularityPeriod, stored, and reported according

to fileReportingPeriod.

end note

Cons <- Prod: Notification (notifyMOICreation)

end loop

end group A.

...

@enduml

Figure A.1.2.2-2

'S5-240924 Figure A.1.2.2-2 Create ThresholdMonitor (rapid)

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group B.1 create Threshold MOI on producer (rapid changes)

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 5. createMOI(class, DN, attributeListIn)

Note right Cons

DN and optionally scoping DNs provided by MNO.

Attributes chosen by DSO. Selected performance metric

attributes must align with those on the PerfMetricJob

on the same producer.

End Note

Prod  -> Prod: 6. CreateMOI( )\n    ThresholdMonitor

Cons <- Prod: 7. Notification (notifyMOICreation)

Cons <-  Prod: 8. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded, ThresholdMonitor is

created. Monitoring is active.

End Note

end loop

loop Monitoring

hnote over Prod

Monitor measurements. There is not yet a

subscription for notifyThresholdCrossing.

end note

end loop

end group B

...

@enduml

Figure A.1.2.2-3

'S5-240924 Figure A.1.2.2-3 Create ThresholdMonitor (slow)

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

'participant "Recipient" as Recip

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group B.2 create Threshold MOI on producer (slow changes)

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 9. createMOI(class, DN, attributeListIn)

Note right Cons

DN and optionally scoping DNs provided by MNO.

Attributes chosen by DSO. Selected performance metric

attributes must align with those on the PerfMetricJob

on the same producer.

End Note

Prod  -> Prod: 10. CreateMOI( )\n    ThresholdMonitor

Cons <- Prod: 11. Notification (notifyMOICreation)

Cons <-  Prod: 12. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded, ThresholdMonitor is

created. Monitoring is active.

End Note

end loop

loop Monitoring

hnote over Prod

Monitor measurements. There is not yet a

subscription for notifyThresholdCrossing.

end note

end loop

end group B

...

@enduml

Figure A.1.2.2-4

'S5-240924 Figure A.1.2.2-4 Create NtfSuscriptionControl

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

participant "network operator producer" as Prod

== DSO consumer wants to receive performance metrics from specific cell sites ==

group C. create NtfSubscriptionControl MOI on producer

loop Setup: for each wanted cell or group of cells (information provided by MNO)

Cons  -> Prod: 13. createMOI(class, DN, attributeListIn)

Note right Cons

DN provided by site operator.

scoping DNs optionally provided by site operator.

Attributes are chosen by DSO.

End Note

Prod  -> Prod: 14. CreateMOI( )\n     NtfSubscriptionControl

Cons <- Prod: 15. Notification (notifyMOICreation)

Cons <-  Prod: 16. result of createMOI(attributeListOut, status)

Note left Prod

On OperationSucceeded,

NtfSubscriptionControl

is created.

End Note

end loop

loop Monitoring

hnote over Prod

Performance Measurements, ThresholdMonitor

and Notifications are active

end note

Cons <-  Prod: Notification (notifyThresholdCrossing)

Cons <-  Prod: Notification (notifyFileReady)

end loop

end group

@enduml

Figure A.1.2.3-1

'S5-240924 Figure A.1.2.3-1

@startuml

skinparam Shadowing false

skinparam Monochrome true

participant "DSO consumer" as Cons

'participant "Recipient" as Recip

participant "network operator producer" as Prod

== DSO consumer wants to close & deactivate the performance monitor session ==

group A. delete PerfMetricJob MOI on producer

loop Delete: for each created cell or group of cells (information provided by MNO)

Cons  -> Prod: 1. deleteMOI(baseObjectInstance [, scopeType, scopeLevel, Filter])

Note right Cons

DN and, optionally, parameters

are supplied by MNO.

End Note

Prod  -> Prod: 2. delete MOI:\n    PerfMetricJob, Files

Cons <- Prod: 3. Notification (notifyMOIDeletion)

Cons <-  Prod: 4. result of deleteMOI(deletionList, status)

hnote over Prod

ongoing PM reporting

is abandoned.

end note

end loop for each wanted MOI

end group A.

...

group B. delete Threshold MOI on producer

loop Delete: for each created cell or group of cells (information provided by MNO)

Cons  -> Prod: 5. deleteMOI(baseObjectInstance [, scopeType, scopeLevel, Filter])

Note right Cons

DN and, optionally, parameters

are supplied by MNO.

End Note

Prod  -> Prod: 6. deleteMOI( )\n    ThresholdMonitor

Cons <- Prod: 7. Notification (notifyMOIDeletion)

Cons <-  Prod: 8. result of deleteMOI(deletionList, status)

hnote over Prod

ongoing threshold

monitoring is abandoned

end Note

end loop

end group B

...

group C. delete NtfSubscriptionControl MOI on producer

loop Delete: for each created cell or group of cells (information provided by MNO)

Cons  -> Prod: 9. deleteMOI(baseObjectInstance [, scopeType, scopeLevel, Filter])

Note right Cons

DN and, optionally, parameters

are supplied by MNO.

End Note

Prod  -> Prod: 10. deleteMOI( )\n     NtfSubscriptionControl

Cons <-  Prod: 11. result of deleteMOI(attributeListOut, status)

hnote over Prod

ongoing notification

subscription is ended

End Note

end loop

end group

@enduml

Next Change