3GPP TSG-SA5 Meeting #155 	S5-243254

Jeju, South Korea, 27 - 31 May 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add use case for discovering of management services

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes to add a new use case, i.e., the discovery of management services in clause 5.1 of TR 28.879 [1].

# 4	Detailed proposal

It is proposed that the following changes be made in clause 5.1 of TR 28.879 [1].

First Change

## 5.1 Exposure of management services

### 5.1.Z Use case #<Z>: Discovering of management services through the CCF

#### 5.1.Z.1	Description

In the CAPIF framework, the discovery of the published management services (now service APIs) by the external MnS consumers is through the CCF via the CAPIF-1/CAPIF-1e interface (clause 10.2 TS 23.222 [X]). Before the external MnS consumer (referred to as the API       invoker in CAPIF terms) can discover the available management services, the discovery policy for such an external MnS consumer shall have been previously configured into the CCF. Subsequently, the external MnS consumer has to be successfully onboarded to the CCF (see clause 8.1 TS 23.222[X]). To successfully onboard the external MnS consumer, the management system needs to provide the onboarding information to the external MnS consumer. This onboarding information includes the end-point URL of the CCF, as well as the required authorization and authentication information to enable the onboarding of the external MnS consumer to the CCF.

Upon successful onboarding, the external MnS consumer can send a "Discover Service API" request to the CCF specifying optional query parameters to limit the discovery results. Currently, to discover management services through CCF, the API invoker id parameter is the only mandatory parameter, and it's up to the configured discovery policy at the CCF to determine what published management services this external MnS consumer (referred to as external API invoker in CAPIF terms) can discover or not.

#### 5.1.Z.2	Potential requirements

PREQ-FS_MExpo-01 The 3GPP management system should provide the authentication and authorization information to enable external MnS consumers to onboard to the CCF successfully.

PREQ-FS_MExpo-02 The 3GPP management system should provide the authentication and authorization information to enable the CCF to onboard the external MnS consumers.

End of Change