3GPP TSG-SA5 Meeting #159 	S5-250030

Sophia-Antipolis, France, 17 – 21 February 2025

# Source:	Ericsson, AT&T

# Title:	Corrections of Trace Report Format

# Document for:	Discussion and Endorsement

Agenda Item:	6.19.14

# 1	Decision/action requested

The group is asked to discuss and endorse to this proposal.

# 2	References

[1]	3GPP TS 32.423: "Subscriber and equipment trace; Trace data definition and management".

# 3	Rationale

## 3.1	Problem statement

In current 3GPP TS 32.423[1], the trace record contents for trace and MDT are specified in subclause 4, and the Trace Report File Format is specified in annex A, which describes the format of trace or MDT result files. There are many misalignments which makes the current Trace file XML format difficult to use. The current XML format is more fulfil the legacy system requirements which is never evolved after release 10.

- MDT Trace Record tables (e.g., 4.34.1) contain misaligned information,

  - "Interface names" are defined under "MDT measurement name" which is misleading.

  - "MDT measurement names" are defined under "Measurement attribute name(s)".

  - "UE Location" is defined under "Measurement attribute name(s)" which does not actually work as a measurement.

  - This table does not contain any Trace Record descriptions which is not aligned with the section title.

  - How to map this table into the Trace Record (XML) file format is not specified.

  - In the table, the measurements are specified by “MDT measurement name”, but the output file format only support “measurement attribute name”.

Example: in section 4.34.1, Trace Record for Immediate MDT measurements, there is a table contains the Trace record description for NR immediate MDT measurements.

| MDT measurement name | Measurement attribute name(s) | Measurement attribute definition | Notes |
| --- | --- | --- | --- |
| M1 | RSRPs | List of RSRP values received in RRC measurement report. One value per measured cell. For beam level granularity, one value per measured beam. | TS 32.422 [3] TS 37.320 [32] TS 38.331 [21] |
|  | RSRQs | List of RSRQ values received in RRC measurement report. One value per measured cell. For beam level granularity, one value per measured beam. | TS 32.422 [3] TS 37.320 [32] TS 38.331 [21] |
|  | PCIs | List of Physical Cell Identity of measured cells. The order of PCI values in the list should be the same as the corresponding measured values in the RSRPs, RSRQs and SINRs attributes. | TS 38.331 [21] |
|  | SINRs | List of SINR values received in RRC measurement report. One value per measured cell. | TS 38.215 [42] TS 32.422 [3] TS 37.320 [32] |
|  | Triggering event | Event that triggered the M1 measurement report, used only in case of RRM configured measurements (events A1, A2, A3, A4, A5, A6, B1 or B2) | TS 32.422 [3] TS 37.320 [32] |
|  | UE location | UE positioning information and sensors data | TS 38.331 [21] |
|  | in-device coexistence interference | See clause 4.34.3 | TS 38.331 [21] |

- The 1st column of the table is named as "MDT measurement name". however, it does not contain any Trace Record name.

- The 2nd column of the table is named as "MDT measurement names". However, it contains more like "Measurement name(s)", though some are clearly not measurements at all, e.g. "Triggering event". Only some of the "Measurement attribute name(s)" are addressed by <meas name> tag in the Annex A, but not all of it.

- This table does not contain any Trace Record descriptions. It is also unclear what is the Trace Record format in case there is multiple "MDT measurement names".

- Further, "UE location" is listed in this table as a "Measurement attribute name". But it is also specified in section 4.34.2 as an "MDT measurement name". How does it work?

Problem statement: all the above observations may lead to different interpretations at implementation.

- MDT meas name and value in Trace Record File Format

  - meas attribute contains the information associated to a UE measurement in MDT task. It includes: meas name and the measurement value (element's content).

  - meas name provides the IE name. However, it is unclear which one is the IE name in corresponding table. It is also unclear what is the measurement value.

  - In general, how the table specified in section 4.34.1 is represented by the meas attribute in the xml format isn't clearly specified. Shall the <meas> tags to be repeated for each "Measurement attribute name" in this table?

Problem statement: all the above observations may lead to different interpretations at implementation.

- Timestamp in Trace Report File Format

  - timeStamp is mandatory field in Trace Record header for streaming. It is the time when each streaming trace record is produced internally in the producer.

  - In traceCollecFile, there is a traceCollec beginTime specified for the start of the first trace data that is stored in this file. Then it follows by a msg changeTime or meas changeTime which is the time difference with "traceCollec beginTime".

  - This changeTime must be repeated every IE entry. It does not allow to provide one timestamp for a trace record, which is not aligned with stream format.

  - Using changeTime might save a little bit of the file size. However, it requires additional calculating on the delta time per IE entry in the file which could be too much. It is very bad performance by using the changeTime.

Problem statement: with performance into the consideration, it shall allow to have timestamp per trace record as it does for streaming trace record.

- Message decoding issue in Trace Report File Format

  - msg attribute contains msg name, which is specified to provides the protocol message name of the traced message. However, there is no interface name included which is only available when rawMsg is included in the attribute (i.e., traceDepth MAX use case only).

  - Without the interface name in the msg, the protocol message names must be unique for decoding, which they are not.

Example: HANDOVER_REQUEST exists on multiple traffic interfaces.

Problem statement: trace decoding cannot be done correctly without either a unique message name or interface name into the msg attribute.

- Trace Record decoding issue in Trace Report File Format

  - There are no unique identities specified per Trace Record. This unique identity is needed at Trace Record decoding.

  - Trace Record ID and/or name is missing in trace record in both streaming and file report format.

Problem statement: Missing Trace Record ID/name into the trace record header

- Trace administrative messages.

  - The Trace administrative messages are specified in Trace Record Header for streaming. It also has File related message specified. However, it is missing in Trace Report File Format.

Problem statement: There is no way to report a Trace administrative message in the Trace Report File.

- ieGroups in Trace Report File Format

  - ieGroup is specified which contains a complex traced IE, i.e. an IE that contains other traced IEs. It includes: the IE group name, and the IE group value. This element is available only if the trace depth is medium or minimum.

  - However, this name and the value is not specified anywhere. This will be an issue in multi-vendor use case.

Problem statement: There is no standard ieGroups definition which could be an issue in multi-vendor environment.

- Report attributes at trace record level

  - The current meas attribute is specified to report at per IE level. It is not allowed to report the measurements at trace record level which are defined in corresponding table as measurement name, i.e., M1, etc.

Problem statement: Reporting at trace record level shall be allowed.

- vendor specific trace data

  - The current Trace Record File Format does not allow to report any vendor-defined Trace Records. As discussed in endorsed in S5-247217, adding support for vendor-defined Trace Records is the task of R19.

Problem statement: adding support for vendor-defined Trace Records

- Redundant meta-data in trace data file XML schema

  - The current XML file schema contains many meta-data information which are already included either in Trace Record Headers or suppliable via Notifications, e.g., “FileReady”.

  - Besides, the current structure has many redundant information as it has to be repeated every entry, which it is possible to be provided at trace record level.

Example of redundant info:

- File Format Version is available in FileReady notification

- changeTime is repeated every entry.

Problem statement: too many redundant information are required in the XML file format

- Too many small issues in XML attributes

  - ue: This element shall not be present in the Trace record of E-UTRAN and 5G NR.

  - ue idValue: shall be optional for MDT, not only management based MDT.

  - initiator and initiator type: Both elements are Signaling Based Trace specific, as both elements cannot be used for Management Based Trace. Besides, the term "NE" is used which is referring to the Management System, not a consumer.

  - ie value: It’s described in ie attribute. However, it is not specified in the table.

  - meas: missing many attributes in the list, e.g. changeTime, direction, drbId, vendorSpecific, measuredObject, measStopTime, target Cell, and ueLocation.

  - meas name: should it be mapped to "Measurement attribute name" in the referenced tables?

  - meas direction: This is a useful element for M6 (UL or DL). But it is unclear how to set it in other interfaces.

  - meas drbId: When is this needed? In the corresponding tables, drbId is not listed as a required infomation for M4-M7, where drbId is applicable only.

  - meas vendorSpecific: "The vendor specific measurements are taken at eNB or RNC". gNB is missing.

  - meas target Cell: this element is not defined in any corresponding tables. Which Mx is this relevant for?

  - meas ueLocation: Is ueLocation considered as a "Trace Record"? Not a "MDT Measurement Name" or "Measurement attribute name"? ueLocation is a complex attribute, with 5 values. This element has only 1 value (as it is of "type" meas).

Problem statement: These observations shows that there are misalignments that may lead to interoperability issues which needs to be fixed.

- RRC report

  - The current trace report format does not have the support of sending RLF, RCEF, and RRC report.

  - In TS32.423, it has an example of sending RLF/RCEF report by using ieGroup. But this is not clear defined how the RRC report is mapped into ieGroup.

Problem statement: missing clear specifications which may lead to different interpretations at implementation.

## 3.2	Potential Solutions

Given the fact that the current trace record file format is specified in very early releases, e.g. meas attribute is specified by S5-112125 in R10, and ieGroup is specified before R6, it is very difficult to update the existing trace record file format without any non-backword compatibility issue.

Therefore, it is proposed to keep current XML format unchanged to avoid backword compatibility issue. Instead, add a new subclause to specify how to output the existing streaming trace record into a file format. This requires a new trace record file format which could fix all issues listed above and at same time align file-based trace format with streaming based trace format.

- Defining how to output streaming trace record into file

  - Trace data reporting consists of trace records that may be written to files or output to streams. Ideally, it would be beneficial if the content is stipulated to be the same for file-based reporting and streaming based reporting.

  - Trace Records are used to carry the captured trace data being reported or to convey various administrative messages associated with the data collection. Administrative messages are intended for the consumer of files from the TCE for the file reporting case, or for the MnS Consumer in the case of stream output.

  - Encoding of trace records may be performed using XML (binary form) or GPB (Google Protocol Buffers).

  - Allow to output the GPB encoded trace records streaming to file format

- Enhance the Trace Record Header with following additional optional attributes:

  - Enhance traceRecordTypeId with a new ENUM value "NORMAL_VS"

  - traceRecordName: String. It is a unique name string of Trace Record(s). This attribute is not required for Administration events. For a vendor specific trace record, "VS." prefix is required.

  - traceRecordId: Integer. It is a unique integer ID of a Trace Record.

# 4	Detailed proposal

It is proposed to endorse the proposal A and B as specified in section 3.2: