3GPP TSG-SA5 Meeting #159	S5-250086

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0357 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.532 Define common notification header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The CR proposes to handle 2 issues: - Missing definition of the current common notification header - Extending the common notification header with paramenters needed for reliable notification transfer. The proposal is based on TR 28.871. - All current SA5 notification have a common header containing 6 parameters present in all notifications (objectClass, objectInstance, notificationId, notificationType, eventTime, systemDN). The common header is not defined in any 5G documentation as a concept; however, it is referenced in multiple 3GPP and external documents, which as of necessity often still refer to 3GPP TS 32.302 (which is not valid for 5G). The definition of these parameters should but is not always the same in different notifications which is confusing. To avoid inconsistent definitions and allowing documents to reference a common header definition a new section to document the notification header shall be added. This chapter is seen as missing, it is an editorial error of 28.532. - In order to ensure reliable notification transfer including detecting missing notifications a sequence number and a subscription identifier shall be added to all notifications. This shall be documented in the common notification header. The specification of individual notifications shall be changed in a separate CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clause defining the common notification header. Extend the headed with extra parameters needed for reliable notification transport. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent definitions of the common parameters may lead to interoperability problems. References to old (pre-5G) documents lead to confusion. Notification transport not reliable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.0, 11.0.1, 11.0.2, 11.0.2.1, 12.0, 12.0.1, 12.0.2, 12.0.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1534 at commit e73a570e33588564be47a9d96b2ac43344080d15 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 11	Management services – Stage 2

## 11.0	Generic management

### 11.0.1	Introduction

This clause provides specifications that are valid for multiple MnSs and capabilities.

### 11.0.2	Common notification header parameters

All notifications emitted by any MnS or function shall both support the following parameters:

#### 11.0.2.1	Input parameters

Table 11.0.2.1-1: Input parameters

| Parameter Name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| objectClass | M | Name of an IOC identified by objectInstance. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| objectInstance | M | Distinguished name (DN) of an MOI. Together with objectClass it identifies an MOI the notification is related to. It can be e.g. an MOI that caused the notification to be emitted or an MOI about which the notification reports some information. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationId | M | This is an identifier for the individual notification, which may be used to correlate notifications. The identifier of the notification shall be chosen to be unique across all notifications of a particular managed object instance throughout the time that correlation is significant, it uniquely identifies the notification from other notifications generated by the MOI specified by objectInstance. | Type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| notificationType | M | String: It specifies the type of notification and hence the information carried by the notification. It should be a fixed, short, human readable string for each type of notification. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A |
| eventTime | M | The date and time when the event that triggered the sending of the notification occurred. | Type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A |
| systemDN | M | Distinguished name of the MnS producer. If an MnSAgent MOI is present, systemDN shall be the DN of an MnSAgent. If no MnSAgent is present the DN of the root MOI (e.g. ManagedElement or Subnetwork) shall be used. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |
| sequenceNo | CM | A sequence number for all notifications that the producers intends to send for a specific notification-subscription. Numbering shall start at zero when the subscription is created or when the producer is started/restarted and shall be increased by one for every new notification. The integer shall be large enough, to ensure that the sequence number shall not reach its upper limit e.g. uint64. If a notification is re-sent, because the delivery of the notification failed, the original sequence number shall be used. The parameter shall be supported if and only if the reliable notification transport feature is supported. allowedValues: non-negative integers | Type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A |
| subscriptionId | CM | The distinguished name identifying the notification subscription. The parameter shall be supported if and only if the reliable notification transport feature is supported. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A |

The parameters may be further specified by the individual notification types.

Individual notification types may define additional parameters.

## 11.1	Generic provisioning management service

Next  changes

# 12	Management services – Stage 3

## 12.0	Generic management

### 12.0.1	Introduction

This clause provides specifications that are valid for multiple MnSs and capabilities.

### 12.0.2	RESTful HTTP-based solution set

#### 12.0.2.1	Common notification header parameters

The IS notification parameters are mapped to SS equivalents according to table 12.0.2.1-1.

Table 12.0.2.1-1: Mapping of IS notification input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type |
| --- | --- | --- | --- |
| objectClass | request body | href | Uri |
| objectInstance |  |  |  |
| notificationId | request body | notificationId | NotificationId |
| notificationType | request body | notificationType | NotificationType |
| eventTime | request body | eventTime | DateTime |
| systemDN | request body | systemDN | systemDN |
| sequenceNo | request body | sequenceNo | integer |
| subscriptionId | request body | subscriptionId | string |

## 12.1	Generic provisioning management service

Next change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1534 at commit e73a570e33588564be47a9d96b2ac43344080d15

*** START OF CHANGE ***

*** OpenAPI/TS28623_ComDefs.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Common Type Definitions

version: 19.3.0

description: >-

OAS 3.0.1 specification of common type definitions in the Generic NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM; Common type definitions

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

Float:

description: This datatype is used for writable attribute

type: number

format: float

FloatRo:

description: This datatype is used for readOnly attribute

type: number

format: float

readOnly: true

DateTime:

description: This datatype is used for writable attribute

type: string

format: date-time

DateTimeRo:

description: This datatype is used for readOnly attribute

type: string

format: date-time

readOnly: true

FullTime:

type: string

description: format (e.g., full-time) is from RFC 3339

format: full-time

DateMonth:

type: string

format: date-month

DateMonthDay:

type: string

format: date-mday

DayInYear:

type: object

properties:

month:

$ref: '#/components/schemas/DateMonth'

monthDay:

$ref: '#/components/schemas/DateMonthDay'

Latitude:

type: number

format: float

minimum: -90

maximum: 90

Longitude:

type: number

format: float

minimum: -180

maximum: 180

Dn:

description: This datatype is used for writable attribute

type: string

DnRo:

description: This datatype is used for readOnly attribute

type: string

readOnly: true

DnList:

description: This datatype is used for writable attribute

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/Dn'

DnListRo:

description: This datatype is used for readOnly attribute

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/DnRo'

Mcc:

description: This datatype is used for writable attribut

type: string

pattern: '^[0-9]{3}$'

MccRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^[0-9]{3}$'

readOnly: true

Mnc:

description: This datatype is used for writable attribut

type: string

pattern: '^[0-9]{2,3}$'

MncRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^[0-9]{2,3}$'

readOnly: true

Nid:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

PlmnId:

description: This datatype is used for writable attribute

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

PlmnIdRo:

description: This datatype is used for readOnly attribute

type: object

properties:

mcc:

$ref: '#/components/schemas/MccRo'

mnc:

$ref: '#/components/schemas/MncRo'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

UtraCellId:

type: integer

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

TimeWindow:

description: This datatype is used for writable attribute

oneOf:

- type: object

properties:

startTime:

$ref: '#/components/schemas/DateTime'

endTime:

$ref: '#/components/schemas/DateTime'

- type: object

properties:

startTime:

$ref: '#/components/schemas/DateTime'

- type: object

properties:

endTime:

$ref: '#/components/schemas/DateTime'

TimeWindowRo:

description: This datatype is used for readOnly attribute

oneOf:

- type: object

properties:

startTime:

$ref: '#/components/schemas/DateTimeRo'

endTime:

$ref: '#/components/schemas/DateTimeRo'

- type: object

properties:

startTime:

$ref: '#/components/schemas/DateTimeRo'

- type: object

properties:

endTime:

$ref: '#/components/schemas/DateTimeRo'

GeoCoordinate:

type: object

properties:

latitude:

$ref: '#/components/schemas/Latitude'

longitude:

$ref: '#/components/schemas/Longitude'

altitude:

$ref: '#/components/schemas/Float'

GeoPolygon:

type: array

uniqueItems: true

items:

$ref: '#/components/schemas/GeoCoordinate'

minItems: 1

GeoArea:

oneOf:

- type: object

properties:

geoPolygon:

$ref: '#/components/schemas/GeoCoordinate'

minItems: 1

GeoAreaToCellMapping:

type: object

properties:

geoArea:

$ref: '#/components/schemas/GeoArea'

associationThreshold:

type: integer

AreaOfInterest:

oneOf:

- type: array

items:

$ref: '#/components/schemas/GeoAreaToCellMapping'

- type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

minItems: 1

maxItems: 8

- type: array

items:

$ref: '#/components/schemas/NrCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

minItems: 1

maxItems: 32

- type: array

items:

$ref: '#/components/schemas/UtraCellId'

minItems: 1

maxItems: 32

Fqdn:

type: string

description: This datatype is used for writable attribute

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

FqdnRo:

type: string

description: This datatype is used for readOnly attribute

pattern: '^([0-9A-Za-z]([-0-9A-Za-z]{0,61}[0-9A-Za-z])?\.)+[A-Za-z]{2,63}\.?$'

minLength: 4

maxLength: 253

readOnly: true

Ipv4Addr:

description: This datatype is used for writable attribute

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

Ipv4AddrRo:

description: This datatype is used for readOnly attribute

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

readOnly: true

Ipv6Addr:

description: This datatype is used for writable attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

Ipv6AddrRo:

description: This datatype is used for readOnly attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

readOnly: true

Ipv6Prefix:

description: This datatype is used for writable attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

Ipv6PrefixRo:

description: This datatype is used for readOnly attribute

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

readOnly: true

IpAddr:

description: This datatype is used for writable attribute

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

- $ref: '#/components/schemas/Ipv6Prefix'

IpAddrRo:

description: This datatype is used for readOnly attribute

oneOf:

- $ref: '#/components/schemas/Ipv4AddrRo'

- $ref: '#/components/schemas/Ipv6AddrRo'

- $ref: '#/components/schemas/Ipv6PrefixRo'

HostAddr:

#  This definition will be deprecated, when all occurances of HostAddr

#  are replaced by Host.

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

- $ref: '#/components/schemas/Fqdn'

Host:

oneOf:

- $ref: '#/components/schemas/IpAddr'

- $ref: '#/components/schemas/Fqdn'

HostRo:

oneOf:

- $ref: '#/components/schemas/IpAddrRo'

- $ref: '#/components/schemas/FqdnRo'

Uri:

description: This datatype is used for writable attribute

type: string

UriRo:

type: string

description: This datatype is used for readOnly attribute

readOnly: true

TimeInterval:

type: object

properties:

intervalStart:

$ref: '#/components/schemas/FullTime'

intervalEnd:

$ref: '#/components/schemas/FullTime'

AdministrativeState:

type: string

enum:

- LOCKED

- UNLOCKED

default: LOCKED

BasicAdministrativeState:

type: string

enum:

- LOCKED

- UNLOCKED

OperationalState:

type: string

enum:

- ENABLED

- DISABLED

default: DISABLED

readOnly: true

UsageState:

type: string

readOnly: true

enum:

- IDLE

- ACTIVE

- BUSY

AvailabilityStatus:

type: string

enum:

- IN_TEST

- FAILED

- POWER_OFF

- OFF_LINE

- OFF_DUTY

- DEPENDENCY

- DEGRADED

- NOT_INSTALLED

- LOG_FULL

readOnly: true

AttributeNameValuePairSet:

description: >-

The key of this map is the attribute name, and the value the attribute value.

type: object

minProperties: 1

additionalProperties:

nullable: true

AttributeValueChangeSet:

description: >-

The first array item contains the attribute name value pairs with the new values,

and the second array item the attribute name value pairs with the optional old values.

type: array

items:

$ref: '#/components/schemas/AttributeNameValuePairSet'

minItems: 1

maxItems: 2

Filter:

description: >-

The filter format shall be compliant to Jex (TS 32.161).

type: string

JexConditions:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex conditions profile specified in clause 7.6 of TS 32.161.

JexNodeSelectionBasic:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex basic profile specified in clause 7.4 of TS 32.161.

JexNodeSelectionAdvanced:

type: string

description: >-

The value of the string shall be a Jex expression that is compliant to the

Jex advanced profile specified in clause 7.5 of TS 32.161.

SystemDN:

type: string

NotificationId:

type: integer

readOnly: true

NotificationType:

oneOf:

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/AlarmNotificationTypes'

- $ref: 'TS28532_ProvMnS.yaml#/components/schemas/CmNotificationTypes'

- $ref: 'TS28532_PerfMnS.yaml#/components/schemas/PerfNotificationTypes'

- $ref: 'TS28532_HeartbeatNtf.yaml#/components/schemas/HeartbeatNotificationTypes'

- $ref: 'TS28532_FileDataReportingMnS.yaml#/components/schemas/FileNotificationTypes'

NotificationHeader:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

notificationId:

$ref: '#/components/schemas/NotificationId'

notificationType:

$ref: '#/components/schemas/NotificationType'

eventTime:

$ref: '#/components/schemas/DateTime'

systemDN:

$ref: '#/components/schemas/SystemDN'

sequenceNo:

type: integer

subscriptionId:

type: string

required:

- href

- notificationId

- notificationType

- eventTime

- systemDN

ErrorResponse:

description: >-

Default schema for the response message body in case the request

is not successful.

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

Subscription:

type: object

properties:

consumerReference:

$ref: '#/components/schemas/Uri'

timeTick:

type: integer

filter:

$ref: '#/components/schemas/JexConditions'

ErrorResponseGet:

description: >-

Default schema for the response message body in case the Get

is not successful.

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badQueryParams:

type: array

items:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badQueryParams:

type: array

items:

type: string

required:

- type

required:

- type

ErrorResponseDefault:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badAttributes:

type: array

items:

type: string

badObjects:

type: array

items:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badAttributes:

type: array

items:

type: string

badObjects:

type: array

items:

type: string

required:

- type

required:

- type

ErrorResponsePatch:

description: >-

Default schema for the response message body in case the patch

is not successful.

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badOP:

type: string

otherProblems:

type: array

items:

type: object

properties:

status:

type: string

type:

type: string

reason:

type: string

title:

type: string

badOp:

type: string

required:

- type

- badOp

required:

- type

- badOp

<CODE ENDS>

*** END OF CHANGE ***