3GPP TSG-SA5 Meeting #159	S5-250185

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.561 | CR |  | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add UC of using NDT to generate ML training data for TS 28.561 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NDT |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR includes the updating of the agreed use case of using NDT in TR28.915 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description and requirements of the use case of using NDT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case of using NDT to generate ML training data will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5	Use cases

5.X	Using NDT to generate ML training data

5.X.1 Description

ML training usually requires large amounts of data to guarantee good performance of the ML models. In general, the ML training data for network related use cases is obtained through historical network management data. For instance, assuming that there is a ML model supporting MDA SLS analysis described in TS 28.104 clause 7.2.2, the raw feature of training data could be the enabling data, such as UL/DL throughput, uplink/downlink delay, etc., as specified in clause 8.4.2 of TS 28.104.

However, obtaining data from the network has the following limitations:

- The quantity of issues happened in actual mobile network is limited.

- The variety of issues happened in actual mobile network is limited. There could be corner network issues cases that hardly happen in real life.

Sufficient ML training data plays a key role to a useful ML model. The more training data provided, the better performance of ML model. To overcome these challenges, an NDT can be used as simulated data generation entity that simulates the network and its characteristics including problems to generate configuration and performance measurements data which can be used to enrich the ML training data set.

5.X.2	Potential requirements

REQ-NDT-GEN-01: The NDT MnS producer should have the capability to allow an authorized MnS consumer to request generation of simulated network data to be used for ML training.

REQ-NDT-GEN-02: The NDT MnS producer should have the capability  to report the results for generated data.

End of changes