3GPP TSG-SA5 Meeting #159	S5-250270

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	pCR TR 28.851 DC Solution for Topic #3

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

This is a pCR to TR 28.851 introducing a new Solution for Topic #3.

# 2	References

[1]	3GPP TR 28.851 " Study on charging aspects of next generation real time communication services phase 2".

# 3	Rationale

This pCR proposes to introduce a new Solution for Use Case #1 TR 28.851 [1].

# 4	Detailed proposal

Propose to include the following changes into the TR 28.851 [1].

First change

#### 5.3.4.x	Solution #2: Application converged charging

##### 5.3.4.x.y	General

Solution#2 addresses Key Issue #2a. It is based on the Northbound API converged charging specified in 3GPP TS 32.260 [11] and 3GPP TS 32.254 [13].

##### 5.3.4.x.y	Description

The solution proposes the possibility to charge and Application (which would be available from an External Application Server) once IMS DC is used.

The Northbound API converged charging architecture specified in clause 4.4 of 3GPP TS 32.254 [13] is applicable for IMS network exposure.

The Northbound API charging principles specified in clause 5.1 of 3GPP TS 32.254 [13] is applicable for IMS network exposure.

The Northbound API converged charging scenarios specified in clause 5.4 of 3GPP TS 32.254 [13] are applicable for IMS network exposure. For flows see TS 32.254 [13] clause 5.4.2 where the external NF/AF are invoking the NEF, and the NEF uses IMS for the API fulfilment, the main difference from Solution #1 (clause 5.3.4.1) is how the Charging can be triggered during a SIP Session, and subsequently include new fields in the Charging Data Request.

Extend the default trigger conditions in IMS Node (3GPP TS 32.260 – Table 5.4.3.1)

Table 5.3.4.x.y-z: Extend Default trigger condition in IMS Node (Table 5.4.3.1 of TS 32.260)

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Change of charging conditions |  |  |  |  | SCUR: Charging Data Request [Update] |
| SIP RE-INVITE or UPDATE (e.g. change in media components, terminating identity change, usage of application) | - | Immediate | No | No |  |

The solution would lead to an extension of the Charging Data Request message which would be used  Nchf_ConvergedCharging_Service as detailed in the table below

Table 5.3.4.x.y-z: Extend Common Data Structure of Charging Data Request (Table 7.1 of TS 32.290)

| Information Element | Converged Charging Catefory | Offline Only Charging Category | Description |
| --- | --- | --- | --- |
| Application Identifier | Oc | - | This field holds the application identifier defined by operator to identify the application. |
| Charging Code | Oc | - | This field holds the charging code referencing a contract under which the charge is applied. |
| Charging Type | Oc | - | This field holds charging type for an application. It can be “charged”, “reserved”, “refunded”, or “rewarded” |
| Charging Unit Type | Oc | - | This field holds type of unit. It can be “money”, “data usage”, “call time”, “specific service”. |
| Cost Information | Oc | - | This field holds the charging unit, currency for the cost for External Application Server. |

The definition of charging information for the Northbound API converged charging specified in clause 6 of 3GPP TS 32.254 [13] is applicable for IMS network exposure. The specific charging information of IMS network exposure, e.g., the calling number and the called number, are contained in "API Content" IE specified in Table 6.3.1.4.1 of 3GPP TS 32.254 [13].

End of changes