3GPP TSG-SA5 Meeting #159	S5-250490

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Huawei

# Title:	Rel-19 pCR TS 28.561 NDT NRMs

# Document for:	Approval

Agenda Item:	6.19.5.1

1	Decision/action requested

For approval

2	References

[1] 	3GPP TS 28.561: “Management aspects of Network Digital Twins”.

3	Rationale

This contribution proposes to define generic operation and information model for NDT.

4	Detailed proposal

It is proposed to make the following changes to TS 28.561 [1].

1st Change

Y	References

[a]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[b]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[c]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[d]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[e]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[f]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

Next Change

X	Stage 2 definition for NDT

X.1	Management operation for NDT (MnS component type A)

The operations (e.g. createMOI operations) and notifications (e.g. notifyMOIcreation) of generic provisioning MnS defined in 3GPP TS 28.532 [a] can be used for NDT management, including NDT lifecycle management, NDT service request management and NDT service report management. The NDT, NDTServiceRequest, NDTServiceReport can be treated as Managed Object instances.

Following is the IS to support NDT lifecycle management:

Table X.1-1: IS to support NDT lifecycle management

| NDT lifecycle management | IS operation |
| --- | --- |
| Query an NDT function | getMOIAttributes operation |

Following is the IS to support NDT service request management:

Table X.1-2: IS to support NDT service request management

| NDT lifecycle management | IS operation |
| --- | --- |
| Create an NDT service request | createMOI operation |
| Delete an NDT service request | deleteMOI operation |
| Modify an NDT service request | modifyMOIAttributes operation |
| Query an NDT service request | getMOIAttributes operation |

Following is the IS to support NDT report management:

Table X.1-2: IS to support NDT report management

| NDT lifecycle management | IS operation |
| --- | --- |
| Query an NDT report | getMOIAttributes operation |
| Subscribe an NDT report | createMOI operation |
| Notify an NDT report | notifyMOIAttributeValueChanges notification |
| Unsubscribe an NDT report | deleteMOI operation |
| Query an NDT report subscription | getMOIAttributes operation |

X.2	Information model definition for NDT (MnS component typeB)

X.2.1	Generic Information model definition

X.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [b], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.622 [b], DataType, TimeWindow | TimeWindow |
| 3GPP TS 28.622 [b], DataType, ThresholdInfo | ThresholdInfo |
| 3GPP TS 28.623 [f], DataType, AttributeNameValuePairSet | AttributeNameValuePairSet |

X.2.1.2	Class diagram

X.2.1.2.1	Relationship

Figure X.2.1.2.1-1: Relationship UML diagram for NDT management

X.2.1.2.2	Inheritance

Figure X.2.1.2.1-1: Inheritance UML diagram for NDT management

X.2.1.3	Class definition

X.2.1.3.1	NDTFunction <<InformationObjectClass>>

X.2.1.3.1.1	Definition

This IOC represents the properties of an NDT function.

The attribute "supportedNdtCapabilities" indicates the capabilities that are support by the NDTFunction instance.

The attribute "ndtStatus" indicates lifecycle status of an NDT. The ENUM value is creation, activation, operation, deactivation, deletion.

X.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNdtCapabilities | M | T | T | F | F |
| ndtStatus | M | T | T | F | F |

X.2.1.3.2	NDTService <<InformationObjectClass>>

X.2.1.3.2.1	Definition

This IOC represents the properties of an NDT service demand created by an MnS consumer.

The attribute "simulationScope" indicates the scope of mobile network that needs to be simulated, which is further specified in different scenarios.

The attribute "behaviour" indicates network behaviour that defined by MnS consumer and will be configured to NDT.

The attribute "simulationTime" is a time window which indicates if the simulation is for the past, present, or future.

The attribute "performanceRequirements" represents the performance requirements for NDT modelling, e.g., the synchronization frequency, maximum run time for each simulation task, precision, etc.

X.2.1.3.2.2	Attributes

The NDTService IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ndtServiceID | M | T | T | F | F |
| simulationScope | M | T | T | F | F |
| behaviour | M | T | T | F | F |
| simulationTime | O | T | T | F | F |
| performanceRequirements | O | T | T | F | F |
| Attribute related roles |  |  |  |  |  |
| ndtReference | M | T | T | F | F |

X.2.1.3.3	NDTReport <<InformationObjectClass>>

X.2.1.3.3.1	Definition

This IOC represents the properties of an NDT service report corresponding to an NDT service request.

The attribute "PerformanceData" is specified performance metrics and/or alarm types that needs to be collected and reported by NDT after the behaviour configured in NDT.

X.2.1.3.3.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ndtReportID | M | T | T | F | F |
| performanceData | O | T | T | F | F |
| Attribute related roles |  |  |  |  |  |
| ndtServiceReference | M | T | T | F | F |

X.2.2	Scenario specific information definition

X.2.2.1	RAN NDT service

X.2.2.1.1	simulationScope

| Attribute Name | Allowed Values |
| --- | --- |
| simulationType (CM) | RAN SubNetwork |
| simulationInstance (CM) | DN of RAN SubNetwork |
| simulationScopeFilter (O) | Filters on geographic area, gNB list, frequency range |

X.2.2.1.2	behaviour

The RAN NDT behaviour includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [b]) and the following attributes.

Table X.2.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ranEsPolicyVerification | O | T | T | F | F |

X.2.2.1.3	performanceData

The performance data related to RAN ES policy verification are captured in Table 6.2.1.2.1.2-1

Table X.2.2.1.3-1

| Performance measurements | Description | Note |
| --- | --- | --- |
| NG-RAN data Energy Efficiency | This measurement provides mobile network data energy efficiency in operational NG-RAN (see clause 6.7.1.1 in TS 28.554 [e]). |  |
| NG-RAN EC | This measurement describes the Energy Consumption (EC) of the NG-RAN (see clause 6.7.3.4.1 in TS 28.554 [e]). |  |
| gNB EC | This measurement describes the Energy Consumption (EC) of the gNB (see clause 6.7.3.4.2 in TS 28.554 [e]). |  |
| UE throughput | This set of measurements describes the UE throughput in gNB (see clause 5.1.1.3 in TS 28.552 [d]) |  |

NOTE: the detail of attributes needs further definition.

Y	Guideline for using the NDT modelling

This clause summaries a mapping relation table, as guideline for using the NDT modelling, between general framework and specific modelling information across different use cases.

Table Y-1: Guideline for using the NDT modelling to support different use cases

| Use case | NDTtype | simulatonScope | behaviour | performanceData |
| --- | --- | --- | --- | --- |
| RAN ES policy verification | RAN NDT | RAN SubNetwork with filters on geographic area, gNB list, frequency range | - ranEsPolicyVerification | - NG-RAN data Energy Efficiency - NG-RAN EC - gNB EC - UE throughput |

Next Change

A.1.1	Relationship UML diagram for NDT management

@startuml

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class ManagedEntity <<ProxyClass>>

class NDTFunction <<InformationObjectClass>>

class NDTService <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

note right of ManagedEntity

Represents SubNetwork

end note

ManagedEntity "1" *-- "*" NDTFunction: <<names>>

NDTFunction "1" *-- "*" NDTService: <<names>>

NDTFunction "1" *-- "*" NDTReport: <<names>>

NDTService "1" <--> "*" NDTReport

@enduml

A.1.2	Inheritance UML diagram for NDT management

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

skinparam  NoteBackgroundColor White

class "<<InformationObjectClass>> \n  Top " as TOP{}

class "<<InformationObjectClass>>\n NDTFunction " as NDT{}

class "<<InformationObjectClass>>\n NDTService " as NDTService{}

class "<<InformationObjectClass>>\n NDTReport " as NDTReport{}

TOP  <|--  NDT

TOP  <|--  NDTService

TOP  <|--  NDTReport

@enduml

End of changes