3GPP TSG-SA5 Meeting #159	S5-240543

Sophia Antipolis, France, 17 - 21 February 2025

# Source:                         NTT DOCOMO, China Mobile

# Title:	pCR TR28.869 VNF configuration management flows

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

- 3GPP TR 28.869 v1.2.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add example flows in technical Annexes to showcase configuration backup operations during createMOI() and modifyMOIAttributes().

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

Annex X: Example scenario for performing configuration backup during NF deployment instance creation

Figure X-1 depicts an example scenario for the interaction between the 3GPP management system and other entities e.g., PaaS Services, for performing a backup of the configuration of NF deployment instances, during NF deployment instance creation.

The NF deployment instance creation flow is using as a baseline flow described in Annex D. An authorized NF provisioning MnS Consumer requests the creation of a 3GPP NF by creating the ManagedFunction IOC on the NF provisioning MnS producer specifying the network function related requirements. These requirements have been updated to consider performing configuration backup actions.

NOTE:	Configuration backup actions can be about VNF application configuration parameters and/or VNF non-application configuration parameters, depending on the requirements specified.

Figure X-1: Example scenario for interaction with PaaS Services for configuration backup procedure during NF deployment creation

In step 6 the NF provisioning MnS producer invokes interactions with the entity which is responsible to perform the configuration backup action.

Regarding the order of execution for VNF application parameters the configuration backup operation can be performed before or after the VNF/VNFCs have been created; this depends on whether all the NF configuration data are available according to TS 28.532 [10], in the input parameters attributeListIn attributes in the createMOI(). If all the data are not available, then the backup operation needs to be performed after the NF Deployment. For example, data to be backed up can be collected according to the output parameter attributeListOut attribute.

For the VNF non-application parameters, according to ETSI GS NFV-IFA 049 [2], vnfInstanceId and vnfcInstanceId are needed like also the related VNF configuration, so the NF deployment need to be instantiated at the time to apply a backup, otherwise there are no VNF (and VNFCs) from which to collect the configuration and perform the backup operation.

Regarding possible interactions between the MnS producer and the PaaS services responsible to perform the actual configuration backup operation, different options can be considered, as depicted in Figure X-2.

Figure X-2: Example interactions between the NF Provisioning MnS producer with PaaS Services

- In case#1 the NF Provisioning MnS producer performs the configuration backup operation based on the relevant IoC describing backup operation requirements. In this case the capability to perform backup operations is an internal feature of the MnS producer.

- In case#2 the NF Provisioning MnS producer performs the configuration backup operation by invoking the interface exposed by the VNF Configuration Manager PaaS Service described in clause 5.1.1.3.1.

- In case#3 the NF Provisioning MnS producer performs the configuration backup operation by invoking the interface exposed by the Configuration Server PaaS Service described in clause 5.1.1.3.3.

- In case#4 the NF Provisioning MnS producer performs the configuration backup operation by invoking the interface exposed by the VNF Configuration Manager described in clause 5.1.1.3.1 (or the Network Configuration manager described in clause 5.1.1.3.2). Then the VNF Configuration is performing the configuration backup operation by invoking the interface exposed by the Configuration Server PaaS Service described in clause 5.1.1.3.3.

All cases described are about the way the MnS producer can manage the request send by the MnS consumer, to perform a configuration backup operation. Regarding the interactions between the MnS consumer and the MnS Producer, updated information on the NRM level is considered that will enable the MnS producer to perform the backup operation for part or all the configuration to be applied for the VNF application parameters and/or the VNF non- application parameters. See clause XXX for a discussion related to the updated NRM.

2nd Change

Annex Y: Example scenario for performing configuration  backup during NF deployment instance modification

Figure Y-1 depicts an example scenario for the interaction between the 3GPP management system and other entities e.g., PaaS Services, for performing a backup of the configuration of NF deployment instances, during NF deployment instance modification.

According to TS 28.532 [10], an authorized NF provisioning MnS Consumer requests modification of a 3GPP NF instance from the NF provisioning MnS producer by sending a modifyMOIAttributes request with DN of the MOI for the ManagedFunction and the network function related modification requirements. On the NRM level, these requirements need to be updated to consider performing configuration backup actions.

Backup operation may be performed for part or all the configuration to be applied for VNF application parameters and/or VNF non- application parameters.

Backup operation may be performed for existing configuration or the new configuration to be applied:

- Backup the "existing configuration" before applying "new modified configuration": In this case the backup has to be done before applying the new one, to preserve the existing configuration

- Backup the "new modified configuration": In this case, the backup operation has to be performed after the configuration has been applied.

Figure Y-1: Example scenario for interaction with PaaS Services for configuration backup procedure during NF deployment modification

In the case of ModifyMOIAttributes operation, the backup operation can be also performed before or after the reconfiguration of the corresponding MOI has been performed. From implementation point of view both cases are possible, but there are pre-conditions/post-conditions to be fulfilled that are dependent on what information needs to be backed up. Parameterization in the updated IOC enables for options like "backupBeforeModification" or "backupAfterModification".

In Figure Y-1 the case of performing the backup operation before reconfiguration of the MOI is depicted.

In step 3 the NF provisioning MnS producer invokes interactions with the entity which is responsible to perform the configuration backup action.  In step 3 there is no limitation regarding the order of the entity that is called first; the orchestration and management system for the NF Deployment modification or the PaaS service performing the configuration backup operation. In both cases the MnS producer provides the configuration data to be backed up.

Regarding the interactions between the NF provisioning MnS producer and the PaaS services similar options can be considered as described in Annex X.

End of Changes