3GPP TSG-SA5 Meeting #159	S5-250609

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add use case and requirements for publishing of management services to the CCF

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposals

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add a use case “Publishing of management services to the CCF” and the corresponding requirements to TS 28.579[1].

# 4	Detailed proposal

It is proposed to add the following use case to TS 28.579[1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Z]                       3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs"

.…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Second Change

## X.Y Publishing of management services into the CCF

### X.Y.1 Definition

This use case describes how to publish a management service to the CAPIF core function (CCF) to be exposed to the external MnS consumers. Before publishing a management service, the 3GPP management system shall determine, subject to the network operator policy, regulatory requirements or contractual obligations, what management services and MnS components (i.e., MnS components A, B or C)  shall be published.

To publish management services to the CCF, the following steps occur :

-    The management service shall be mapped into a service API, since CAPIF publishes service APIs and not management services. The management service information is mapped into service API information using the "ServiceAPIDescription" data type (see clause 8.2.4.2.2 in 3GPP TS 29.222 [Z]). The "ServiceAPIDescription" data type describes a service API, and the network operator can decide to publish a single management service as one or more service APIs at the CCF.

-	The management services now mapped into service APIs are published to the CCF using the Publish_Service_API (see clause 5.3.2.2 of TS 29.222 [Z]) over the CAPIF-4 interface. The APF of the MSED performs the publishing functionality.

### X.Y.2 Requirements

REQ-MExpo-Pub-R1: The 3GPP management system shall have the capability to map management service information into service API information.

REQ-MExpo-Pub-R2: The 3GPP management system shall have the capability provide the APF functionality.

End of Changes