3GPP TSG-SA5 Meeting #159	S5-250873

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:                         Rakuten Mobile

# Title:	pCR TR 28.869 LCM changes for declarative descriptor

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

For approval

# 2	References

- 3GPP TR 28.869 v1.1.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add more details to Section 5.2.2.3.2 to describe CRD-based callflow for declarative descriptor

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.869 [1].

1st Change

##### 5.2.3.3.2	Use of deployment management reference point based on declarative descriptor

In this solution the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3 for creation of a NF deployment instance. The deployment requirements for creating a workload of a NF are conveyed from the 3GPP management system to the orchestration and management system via a declarative descriptor.

Figure 5.2.3.3.2-1 depicts a high-level view of proposed procedure for creation of a NF deployment instance based on declarative descriptor.

Figure 5.2.3.3.2-1: Interaction between 3GPP management system and orchestration and management system using deployment management reference point  based on declarative descriptor

The declarative descriptor provides a declaration in high-level on what to be achieved by the orchestration and management system rather than how to achieve it. The orchestration and management system converts the 3GPP requirements specified in the declarative descriptor to a NF deployment.

If the orchestration and management system is ETSI NFV MANO, the interactions over deployment management reference point are as specified in clause 7.10 of 28.531 [7]. For the case of NFV-MANO, the declarative descriptor is VNFD,as per ETSI NFV specifications (see ETSI GS NFV-IFA 011 [22]).

NOTE:	In case the orchestration and management system is ETSI NFV MANO, VNFD is defined to convey the deployment requirement information. For other industry solutions, currently there is no standardized descriptor.

If the orchestration and management system is Kubernetes-based and the API used between NFMS_P and orchestration system is K8S-based, the various implementation options for interactions over deployment management reference point are illustrated in Figure 5.2.3.3.2-2.

Figure 5.2.3.3.2-2. interaction between 3GPP management system and orchestration and management system for NF instance creation

1)	Network Function Management Service Provider (NFMS_P) receives a createMOI, for ManagedFunction IOC, request from Network Function Management Service Consumer (NFMS_C) with network function related requirements.

NOTE:	The network function related requirements see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6].

2)	If NF instance to be created contains cloud-native parts, NFMS_P derives the requirements for NF deployment instance based on the network function related requirements.

3)	If corresponding NF Package needs to be on-boarded or changed, the NFMS_P invoke corresponding NF Package management procedures. For the interaction with ETSI NFV MANO, the procedure is described in clause 7.7 in ETSI GS NFV-IFA013 [3].

4 and 5)	The NFMS_P invokes NF deployment lifecycle management. There are three different implementations possible here.

Case A:

4A) Use native K8S objects for lifecycle management of NF deployment.

Case B:

4B) Use K8S-based vendor-specific CR objects for lifecycle management of NF deployment.

5B) A vendor-specific K8S operator monitors the CR for changes, derives needed K8S objects and updates the API server accordingly

Case C:

4C) Use 3GPP-defined K8S CR for lifecycle management of NF deployment.

5C) A vendor-specific K8S operator monitors the 3GPP-based CR for changes, derives needed K8S objects and updates the API server accordingly.

6)	The NFMS_P creates the MOI for the ManagedFunction to be created. If the ManagedFunction contains virtualized part, the NFMS_P may send the request of creating the MOI to the NFMS_P in the ManagedFunction.

7)	The NFMS_P configures the new created MOI with corresponding configuration information (see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[6]).

8)	The NFMS_P sends the createMOI response to NFMS_C with DN of MOI for ManagedFunction.

End of Changes