3GPP TSG-SA5 Meeting #159	S5-250911

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.572 Improve clause Validation

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.3	Validation of planned configurations

### 6.3.1	Definition

Planned configurations may have problems and it may be impossible to activate them. Validation is the process to unveil potential problems with planned configurations before activation of the planned configuration.

The validation function has two input arguments: the operation set representing the planned configuration and a snapshot of the current configuration. The planned configuration is applied to the snapshot of the current configuration. The resulting configuration data node tree must comply to the (stage 2) NRM definitions and (stage 3) NRM schema definitions.

### 6.3.2	Potential problems

Validation discovers problems of planned configurations. These problems may be classified as follows:

- Information model problems

  - Problems of the planned configuration itself

  - Problems of the planned configuration in relationship to the current configuration

- Application layer problems

Examples for problems of the planned configuration itself:

- Update of an attribute with a value that does not match the data type defined for the value of the attribute

- Creation of an attribute name value pair that is not defined

- Update of an existing attribute that is defined as read-only

- Creation of an object with an object class that is not defined

- Creation of an object with a representation does not match the object class defined for the object

- Creation of an object under an object whose class does not support this containment

Examples for problems of the planned configuration in relationship to the current configuration:

- Creation of objects under parents that do not exist

- Deletion of leaf objects that do not exist

- Deletion of objects that are no leaves

- Violation of the cardinality property of an object

- Violation of the multiplicity property of an attribute

Examples for application layer problems

- An attribute value in the planned configuration is in the value range allowed by the attribute properties but the value is not allowed in the current state of the application.

### 6.3.3	Validation process

The purpose of validation is to detect problems of the planned configuration. As described in the previous clause, problems can be categorized into three groups.

Problems of the operations itself are detected by analyzing the operations together with the NRM schema. The current configuration is not involved in this process. Problematic operations are always invalid. If an atomic operation set contains at least one invalid operation, the complete planned configuration is invalid. If a non-atomic operation set contains a bad operation, only the operation is invalid. The operation set with the other operations may still be valid, depending on the validation result for the other operations.

Problems of the planned configuration in relationship to the current configuration are detected by applying the operations of the operation set one by one to a snapshot of the current configuration. This may result in intermediate states that do not comply to the constraints defined by the NRM schema. For example, an object of class A shall contain always exactly five objects of class B. To replace an object of class B, the object to be replaced may be deleted in the first step. In the second step the new object is added. The other possibility is to add the new object first and delete the object to be removed afterwards. In both cases the cardinality has a temporary invalid state. The validation process needs to take this into account: Problems of the planned configuration in relationship to the current configuration may be temporary only and be healed later when more operations are processed. A final assessment on the validity of operations can be made only after processing all operations.

In addition to problems of the planned configuration in relationship to the current configuration, problems of the planned configuration at application layer may occur. These problems are detected by evaluating the updates specified by the operations in the context of the configuration that is produced by applying the planned configuration against the current configuration. As for problems of the planned configuration in relationship to the current configuration, a final assessment on the validity of operations can be made only after processing all operations.

In the atomic mode for validation all operations must be valid for the planned configuration to be valid. If only one operation is invalid the complete planned configuration is invalid. The former case is also referred to as a successful validation and the latter as a failed validation.

In the best effort mode for validation all operations must be valid for the planned configuration to be valid. If at least one operation is invalid (but not all), the planned configuration is partially valid (or partially invalid). If all operations are invalid the planned configuration is invalid. These three cases are also referred to as successful validation, partially successful validation (or partially failed validation), and failed validation.

All constraint violations shall be reported.

A successful validation does not guarantee a successful activation.

### 6.3.4	Validation of planned configuration groups

The planned configuration group members are validated in the order specified. If no order is specified, they can be validated in any order. Each member is validated as specified in the previous clause. The validation of atomic members concludes with either validated or failed. The validation of best effort members concludes with either validated, partially validated or failed.

An atomic group is considered as valid, if atomic members are validat, and best effort members are validat or partially valid. If the validation of one (atomic or best effort) member fails, the validation of the complete group shall fail.

A best effort group is considered as valid, if atomic members are valid, and best effort members are valid or partially valid. If the validation of at least one (atomic or best effort) member fails, the group is considered as partially valid. If the validation of all members fails, the validation of the complete group shall fail.

### 46.3.5	Validation control parameters

Control parameters for validation are not specified.

End of modifications