3GPP TSG-SA5 Meeting #160	S5-251263

Goteborg, Sweden, 07 - 11 April 2025

Source:	Rakuten Mobile, Inc.

Title:	Pseudo-CR on LCM changes for declarative descriptor

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	1.3.0

Work Item:	FS_Cloud_OAM/Study on Cloud Aspects of Management and Orchestration

Comments

The contribution proposes to add more details to Section 5.2.2.3.2 to describe CRD-based callflow for declarative descriptor.

Proposed Changes

* * * First Change * * * *

##### 5.2.3.3.2	Use of deployment management reference point based on declarative descriptor

In this solution the 3GPP management system interacts with an orchestration and management system using the deployment management reference point as described in clause 5.2.1.3 for creation of a NF deployment instance. The deployment requirements for creating a workload of a NF are conveyed from the 3GPP management system to the orchestration and management system via a declarative descriptor.

Figure 5.2.3.3.2-1 depicts a high-level view of proposed procedure for creation of a NF deployment instance based on declarative descriptor.

Figure 5.2.3.3.2-1: Interaction between 3GPP management system and orchestration and management system using deployment management reference point  based on declarative descriptor

The declarative descriptor provides a declaration in high-level on what to be achieved by the orchestration and management system rather than how to achieve it.

If the orchestration and management system is ETSI NFV MANO, the interactions over deployment management reference point are as specified in clause 7.10 of 28.531 [7]. For the case of NFV-MANO, the declarative descriptor is VNFD,as per ETSI NFV specifications (see ETSI GS NFV-IFA 011 [22]). If another orchestration and management entity is considered, updates are needed in TS 28.531 [7] and possibly other specifications to describe the interactions over the deployment management reference point and to define the declarative descriptor.

NOTE:	In case the orchestration and management system is ETSI NFV MANO, VNFD is defined to convey the deployment requirement information. For other industry solutions, currently there is no standardized descriptor.

If the orchestration and management system is Kubernetes-based and the API used between NFMS_P and orchestration system is K8S-based, the various implementation options for interactions over deployment management reference point are illustrated in Figure 5.2.3.3.2-2.

Figure 5.2.3.3.2-2. Interaction between 3GPP management system and orchestration and management system for NF instance creation

- Network Function Management Service Provider (NFMS_P) receives a createMOI, for ManagedFunction IOC, request from Network Function Management Service Consumer (NFMS_C) with network function related requirements and any CNF-specific resource requirements. The NF related requirements can include application parameters, performance requirements and resource criteria.

NOTE:	Performance requirements may include parameters such as user-plane throughput and subscriber capacity. Resource criteria may include parameters such as compute/memory/storage footprint and allowed energy consumption.

NOTE:	For network function related requirements, see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[17]. These may need to be enhanced to accommodate CNF requirements.

If NF instance to be created contains cloud-native parts, NFMS_P can follow two paths.

Case A (Microservice-level instantiation):

2A)		NFMS_P derives the resource requirements for NF deployment instance at microservice-level.

3A) 	NFMS_P uses K8S API to deploy the NF deployment objects

Case B (NF requirements-based instantiation):

2B) 	NFMS_P transparently passes the resource requirements to the K8S cluster using a NF CR.

3B) 	A K8S operator monitors the CR for changes and derives needed K8S objects. This is similar to step 2A above.  It then updates the API server accordingly.

4)	K8S node deploys the workloads requested.

5)	The NFMS_P creates the MOI for the ManagedFunction to be created.

6)	The NFMS_P configures the newly created MOI with corresponding configuration information (see information model definition for NR NRM in clause 4 and information model definition for 5GC NRM in clause 5 in TS 28.541[17]).

7)	            The NFMS_P sends the createMOI response to NFMS_C with DN of MOI for ManagedFunction.

* * * End of Changes * * * *