3GPP TSG-SA5 Meeting #160	S5-251268

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0071 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.310 Multi-carrier RAN energy saving optimization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce a new use case for energy-saving optimization for scenarios where there may be multiple capacity booster cells are partially or fully overlaid by one or more candidate cell(s). In such scenarios, the capacity booster cells need to be matched to the appropriate candidate cells and the deactivation of the capacity booster cells need to be sequenced. This CR is related to Energy_OAM_Ph3, WT-8 Cell proximity-based energy saving. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses to be introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proximity-based energy saving optimization for multi-layer RAN scenarios not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.X (new), 5.2.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.1.X	Energy saving optimization for multi-carrier RAN scenarios with several partially or fully overlaid capacity booster cells

NG-RAN can be multi-carrier, characterized by several capacity booster cells, overlaid by several candidate cells, e.g. in different carriers or RATs. Capacity booster cells are the cells that can be deactivated when the load is low while candidate cells are the cells which can replace coverage when the capacity booster cells are deactivated. As illustrated in Figure 5.1.X.1-1, the RAN may involve:

-	A cell carrier where for several candidate cells, a candidate cell may overlap several capacity booster cells of different characteristics, e.g., the capacity booster cells may be small macro or multi-sector cells.

-	A capacity booster cell’s coverage is partially or completely overlaid by the candidate cells’ coverage i.e. candidate cells provide complete coverage which the capacity booster cells complement with extra capacity.

NOTE 1: Deactivating a cell is equivalent to transitioning the cell into energy saving state which may include a total switching off of the cell or any disabling of a part of the cell, e.g., a disabling of the cell's power amplifier.

Figure 5.1.X.1-1: An example multi-layer (possibly multi-RAT) network with macro cells A and B, and small cells C, D and E.

Since multiple cells may overlap with several other cells, there needs to be a pairing or grouping of cells that should be considered together for energy saving. The MnS consumer should be enabled to configure such cell groupings for energy saving. Within the group some cells should be marked as capacity booster cells that can be deactivated when the load is low and others as candidate cells are the cells which support coverage even when the capacity booster cells are deactivated.

Within the group, there can be several candidate cells which need to be deactivated at different times as the offered load in the network reduces. The management system should support capability to configure or derive a cell deactivation sequence defining the order in which the cells may be deactivated.

NOTE 2: The activation sequence is the opposite order to the deactivation sequence.

Next Change

### 5.2.Y	Requirements for Energy saving optimization for multi-carrier RAN scenarios with several partially or fully overlaid capacity booster cells

REQ-NES-01:	The 3GPP management system should support a capability enabling an energy saving function to be configured with or to derive an energy saving cell group.

REQ-NES-02:	The 3GPP management system should support a capability enabling an energy saving function to be configured with or to derive the categorization of cells within an energy saving cell group as either candidate or capacity booster cells.

REQ-NES-03:	The 3GPP management system should support a capability enabling an energy saving function to be configured with or to derive the cell deactivation indicating the capacity booster cell’s rank in the deactivation and reactivation process

End Change