SA WG5 Meeting #160	S5-251388

# Stor-Goteberg, SWEDEN, 07-11 April 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.423 | CR | 0201 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional NFs added in Trace Record contents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Jio Platforms Ltd(JPL) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is to reflect the inclusion of Tracing requirements at BSF,CHF,NWDAF,DCCF,ADRF,LMF and GMLC in TS 32.423-j20. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new section on Tracing to include the NF Trace record contents under section heading 4. Trace record contents on NF interfaces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the tracing records at these NFs, tracing cannot be enabled on these NFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 4.XX |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change* * * *

********Section before is same**********

##### 4.XX	BSF Trace Record Contents

The following table shows the trace record content for BSF.

The trace record is the same for management-based activation and for signalling based activation.

BSF shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| Nbsf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like PCF. BSF ID of the traced BSF |
|  |  | O | O | X | IE extracted from Nbsf messages between the traced BSF and the PCF. |
|  | Encoded* | X | X | M | Raw Messages: Nbsf messages between the traced BSF and the PCF. The encoded content of the message is provided. |
| Nnrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like NRF. BSF ID of the traced BSF |
|  |  | O | O | X | IE extracted from Nnrf messages between the traced BSF and the NRF. |
|  | Encoded* | X | X | M | Raw Messages: Nnrf messages between the traced BSF and the NRF. The encoded content of the message is provided. |

##### 4.XX	CHF Trace Record Contents

The following table shows the trace record content for CHF.

The trace record is the same for management-based activation and for signalling based activation.

CHF shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| N40 | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected SMF CHF ID of the traced CHF |
|  |  | O | O | X | IE extracted from Nchf messages between the traced CHF and the SMF. |
|  | Encoded* | X | X | M | Raw Messages: Nchf messages between the traced CHF and the SMF. The encoded content of the message is provided. |
| N28 | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected PCF CHF ID of the traced CHF |
|  |  | O | O | X | IE extracted from Nchf messages between the traced CHF and the PCF. |
|  | Encoded* | X | X | M | Raw Messages: Nchf messages between the traced CHF and the PCF. The encoded content of the message is provided. |
| Nnrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like NRF. CHF ID of the traced CHF |
|  |  | O | O | X | IE extracted from Nnrf messages between the traced CHF and the NRF. |
|  | Encoded* | X | X | M | Raw Messages: Nnrf messages between the traced CHF and the NRF. The encoded content of the message is provided. |

##### 4.XX	DCCF Trace Record Contents

The following table shows the trace record content for DCCF.

The trace record is the same for management-based activation and for signalling based activation.

DCCF shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| Ndccf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF i.e. GMLC, NEF, NWDAF, LMF, AMF etc. DCCF ID of the traced DCCF |
|  |  | O | O | X | IE extracted from Ndccf messages between the traced DCCF and the NF. |
|  | Encoded* | X | X | M | Raw Messages: Ndccf messages between the traced DCCF and the NF. The encoded content of the message is provided. |
| Nnrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like NRF. DCCF ID of the traced DCCF |
|  |  | O | O | X | IE extracted from Nnrf messages between the traced DCCF and the NRF. |
|  | Encoded* | X | X | M | Raw Messages: Nnrf messages between the traced DCCF and the NRF. The encoded content of the message is provided. |
| Nnwdaf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NWDAF DCCF ID of the traced DCCF |
|  |  | O | O | X | IE extracted from Nnwdaf messages between the traced DCCF and the NWDAF |
|  | Encoded* | X | X | M | Raw Messages: Nnwdaf messages between the traced DCCF and the NWDAF. The encoded content of the message is provided. |
| Nudm | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected UDM DCCF ID of the traced DCCF |
|  |  | O | O | X | IE extracted from Nudm messages between the traced DCCF and the UDM |
|  | Encoded* | X | X | M | Raw Messages: Nudm messages between the traced DCCF and the UDM. The encoded content of the message is provided. |
| Nbsf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected BSF DCCF ID of the traced DCCF |
|  |  | O | O | X | IE extracted from Nbsf messages between the traced DCCF and the BSF |
|  | Encoded* | X | X | M | Raw Messages: Nbsf messages between the traced DCCF and the BSF. The encoded content of the message is provided. |

##### 4.XX	NWDAF Trace Record Contents

The following table shows the trace record content for NWDAF.

The trace record is the same for management-based activation and for signalling based activation.

NWDAF shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| Nnwdaf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like PCF, CHF, UPF, SMF, NRF etc. NWDAF ID of the traced NWDAF |
|  |  | O | O | X | IE extracted from Nnwdaf messages between the traced NWDAF and the NF. |
|  | Encoded* | X | X | M | Raw Messages: Nnwdaf messages between the traced NWDAF and the NF. The encoded content of the message is provided. |
| Nnrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like NRF. NWDAF ID of the traced NWDAF |
|  |  | O | O | X | IE extracted from Nnrf messages between the traced NWDAF and the NRF. |
|  | Encoded* | X | X | M | Raw Messages: Nnrf messages between the traced NWDAF and the NRF. The encoded content of the message is provided. |

##### 4.XX ADRF Trace Record Contents

The following table shows the trace record content for ADRF.

The trace record is the same for management-based activation and for signalling based activation.

ADRF shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| Nadrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF i.e. GMLC, NEF, NWDAF, LMF, AMF etc. ADRF ID of the traced ADRF |
|  |  | O | O | X | IE extracted from Nadrf messages between the traced GMLC and the NF. |
|  | Encoded* | X | X | M | Raw Messages: Nadrf messages between the traced GMLC and the NF. The encoded content of the message is provided. |
| Nnrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like NRF. ADRF ID of the traced ADRF |
|  |  | O | O | X | IE extracted from Nnrf messages between the traced ADRF and the NRF. |
|  | Encoded* | X | X | M | Raw Messages: Nnrf messages between the traced ADRF and the NRF. The encoded content of the message is provided. |

##### 4.XX	LMF Trace Record Contents

The following table shows the trace record content for LMF.

The trace record is the same for management-based activation and for signalling based activation.

LMF shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| Nlmf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF i.e. AMF, GMLC and NWDAF etc. LMF ID of the traced LMF |
|  |  | O | O | X | IE extracted from Nlmf messages between the traced LMF and the NF. |
|  | Encoded* | X | X | M | Raw Messages: Nlmf messages between the traced LMF and the NF. The encoded content of the message is provided. |
| Nnrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like NRF. LMF ID of the traced LMF |
|  |  | O | O | X | IE extracted from Nnrf messages between the traced LMF and the NRF. |
|  | Encoded* | X | X | M | Raw Messages: Nnrf messages between the traced LMF and the NRF. The encoded content of the message is provided. |

##### 4.XX	GMLC Trace Record Contents

The following table shows the trace record content for GMLC.

The trace record is the same for management-based activation and for signalling based activation.

GMLC shall support at least one of the following trace depth levels – Maximum, Medium or Minimum.

| Interface (specific messages) | Format | Level of details |  |  | Description |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Med | Max |  |
| Ngmlc | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF i.e. GMLC, NEF, NWDAF, LMF, AMF etc. GMLC ID of the traced GMLC |
|  |  | O | O | X | IE extracted from Ngmlc messages between the traced GMLC and the NF. |
|  | Encoded* | X | X | M | Raw Messages: Ngmlc messages between the traced GMLC and the NF. The encoded content of the message is provided. |
| Nnrf | Decoded | M | M | O | Message name |
|  |  | O | O | O | Record extensions |
|  |  | M | M | X | NF ID of the connected NF like NRF. GMLC ID of the traced GMLC |
|  |  | O | O | X | IE extracted from Nnrf messages between the traced GMLC and the NRF. |
|  | Encoded* | X | X | M | Raw Messages: Nnrf messages between the traced GMLC and the NRF. The encoded content of the message is provided. |

*********Section after is same**********

# * * * End of changes * * * *