3GPP TSG-SA5 Meeting #160	S5-251697

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add use case and requirements for registration of the MSED into the CCF

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add a use case and requirements for registration of the MSED into the CCF.

# 4	Detailed proposal

It is proposed to add a use case and requirements for registration of the MSED into the CCF to TS 28.579[1].

First Change

### 5.2.X Registration of the MSED into the CCF

#### 5.2.X.1 Definition

For MSED to enable the exposure of the management services using CAPIF, MSED must be registered as a recognized API provider domain at the CCF. The MSED registration procedure ensures that the constituent API provider domain functions (i.e., API Exposing Function (AEF), the API Publishing Function (APF) and the API Management Function (AMF) are recognized users of the CCF and can support the use cases and functionalities described in clause 5.2.

Traceability: REQ-MEXPO-REG-01, REQ-MEXPO-REG-02, REQ-MEXPO-REG-03

Second Change

## 5.3 Requirements

REQ-MEXPO-FUN-01: MSED shall have the capability to provide the API provider domain functions as defined by CAPIF.

Editor’s Note: This requirement can be revised as the requirement shall be on 3GPP Management System.

REQ-MEXPO-FUN-02: 3GPP management system shall have the capability to provide the required mappings between the management service information and the CAPIF-defined information elements to enable the exposure of management services to external MnS consumers using CAPIF.

REQ-MEXPO-REG-01: The 3GPP management system shall provide the capability to register MSED to the CCF.

REQ-MEXPO-REG-02: The 3GPP management system shall provide the capability to deregister MSED from the CCF.

REQ-MEXPO-REG-03: The 3GPP management system shall provide the capability to update the registration details of MSED at the CCF.

End of Changes