3GPP TSG-SA5 Meeting #160	S5-252094

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0540 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.622 Update Attribute constraints for TimeWindow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TimeWindow <<choice>> was originally defined for ManagementDataCollection IOC in R18, and extended for other usages (e.g., intent NRM, AIML NRM). However, the description for attribute constraints is limited to ManagementDataCollection IOC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description for attribute constraint for TimeWindow <<choice>> to be generic to applied for other usages (e.g., intent NRM, AIML NRM). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for attribute constraint for TimeWindow <<choice>> is specific for ManagementDataCollection IOC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.47.1, 4.3.48.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.3.47	ManagementDataCollection

4.3.47.1	Definition

This IOC represents a management data collection request job. The requested data could be of kind Trace, MDT (Minimization of Drive Test), RLF (Radio Link Failure) report, RCEF (RRC Connection Establishment Failure) report, RRC report, PM (performance measurements), KPI (end-to-end key performance indicators) or a combination of these.

The attribute managementData defines the management data which shall be reported. This may either include a list of data categories or a list of management data identified with their name. For further details see clause 4.3.50. The targetNodeFilter attribute can be used to target object instance(s) producing the required management data. It is assumed that the consumer may not have detailed knowledge of the network and hence may not identify the exact object instance producing the required management data. In this case consumer can request management data, specified by 3GPP, produced by certain object instance (s) based on a particular location, the domain (CN or RAN) of theobject instances, and the handled traffic (CP or UP) of the object instances.

To activate the production of the requested data, a MnS consumer has to create a ManagementDataCollection object instance on the MnS producer.

The MnS producer may derive multiple jobs PerfMetricJob, TraceJob) from a single ManagementDataCollection job for collecting the required management data. If the MnS producer receives the collected data from multiple sources, it consolidate the data into a set of management data for reporting.

The attribute collectionTimeWindow specifies the time window (including CHOICE_1.1 startTime and CHOICE_1.2 endTime) for which the management data should be reported.

The attribute reportingCtrl specifies the method and associated control parameters for reporting the produced management data to MnS consumers. Three methods are available: file-based reporting with selection of the file location by the MnS producer, file-based reporting with selection of the file location by the MnS consumer and stream-based reporting.

The attribute dataScope configures, whether the management data should be reported per S-NSSAI or per 5QI or per PLMN, if applicable.

4.3.47.2	Attributes

The ManagementDataCollection IOC includes the attributes inherited from Top IOC (defined in clause 4.3.29) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| managementData | M | T | T | T | N/A |
| targetNodeFilter | M | T | T | T | N/A |
| collectionTimeWindow | M | T | T | T | N/A |
| reportingCtrl | M | T | T | T | N/A |
| dataScope | O | T | T | T | N/A |

4.3.47.3	Attribute constraints

None.

4.3.47.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC. In addition, the following set of notifications is also valid.

| Name | S | Notes |
| --- | --- | --- |
| notifyFileReady | M | -- |
| notifyFilePreparationError | M | -- |

2nd Change

4.3.48	TimeWindow <<choice>>

4.3.48.1	Definition

This choice defines a time window.

It is a choice between the control parameters required to define the time window as follows:

When startTime and endTime is present (CHOICE_1), the time window starts when startTime is reached and ends when endTime is reached.

When only the startTime attribute is present (CHOICE_2), the time window starts when startTime is reached and runs until deletion of the managed object instance including this timeWindow.

When only the endTime attribute is present (CHOICE_3), the time window starts when the managed object instance including this TimeWindow is created and ends when endTime is reached.

4.3.48.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 startTime | CM | T | T | T | T |
| CHOICE_1.2 endTime | CM | T | T | T | T |
| CHOICE_2.1 startTime | CM | T | T | T | T |
| CHOICE_3.1 endTime | CM | T | T | T | T |

4.3.48.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CHOICE_1.1 startTime CHOICE_1.2 endTime | These attributes shall be supported, when support MnS consumer to configure the start and end time of the time window. |
| CHOICE_2.1 startTime | This attribute shall be supported, when support MnS consumer to configure only the start time of a time window and the end time is defined by the deletion of the managed object instance. |
| CHOICE_3.1 endTime | This attribute shall be supported, when support MnS consumer to configure only the end time of a time window and the start time is defined by the creation of the managed object instance. |

4.3.48.4	Notifications

The clause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

End of Changes