3GPP TSG-SA5 Meeting #160	S5-251244

Goteborg, Sweden, 07 - 11 April 2025

Source:	Nokia, Samsung

Title:	Pseudo-CR on CCL Trigger conflicts

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for CCL Trigger and action-execution conflicts handling as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.A	 CCL Conflict management Capability - CONF

### 5.A.2	Use Cases

#### 5.A.2.Y	CCL Trigger-time conflicts handling - TTRTM

Typically, a CCL needs to be triggered to run at a specific time and terminate when certain conditions are met, to run when a certain performance threshold is crossed. If triggered independently, there may be conflicts among the CCLs. The triggers for different CCLs to be executed need to be coordinated to avoid conflicts among the CCLs.

The management system should support the capability for avoidance of action-execution-time conflicts. Since each CCL focuses on a smaller scope of the network problem space, several CCLs may need to be executed.  For actions in a given network scope, the CCLs can be explicitly scheduled by the management system. Where the scopes overlap, the CCLs need to align the action plans, for example, which action plan to execute and when. There is a need to assess each plan and choose the most appropriate combination of action plan(s) based on the selection policy and then notify the selected action plan(s) to the related CCLs.

### 5.A.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- CONF-TTRTM-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL trigger-time conflicts. | CONF-TTRTM |
| REQ-CONF-TTRTM-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL trigger-time conflict after it is confirmed. | CONF-TTRTM |
| REQ-CONF-TTRTM-03 | The 3GPP Management System should enable authorized MnS Consumer to provide information that can be used to support a capability to avoid or resolve a CCL trigger-time conflict. | CONF-TTRTM |

* * * End of Changes * * * *