3GPP TSG-SA5 Meeting #160	S5-251258

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR TS28.105 Coordination AIML inference capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR to introduce the agreed use case and requirements in TR28.858 on Coordination AIML inference capabilities |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the use case and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proposed use case of Coordination AIML inference capabilities is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.2.Y (new), 6.5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 6.5.3	AI/ML inference capabilities management

#### 6.5.3.2	Use cases

##### 6.5.3.2.Y	Coordinating decisions of AI/ML inference functions

AIML inference functions acting on a network may differ in source vendors and behavioural characteristics. For security and operability reasons, the operator may not grant network control rights to all the AIML inference functions. A higher-hierarchy end-to-end AIML inference coordination functionality can take responsibility for the end-to-end performance. The coordination functionality evaluates the state of the network and based on the registrerd capabilities of different AIML inference functions, it decides which of the AIML inference functioncs should be activated to propose a response to the observed state. The coordination functionality evaluates the recommendations of the AIML inference functions and their likely effects to decide the changes that should be executed on the network (e.g. to minimize concurrent changes on the same network resources). It informs the AIML inference functions of the respective feedback related to their recommended actions. The feedback may be in form of a dimensionless Action Quality Indicator that describes how acceptable or not acceptable the actions are.

The coordination function may also (re)configure the AIML inference functions based on the observed effects of their actions (e.g. to redefine the control parameter space of the individual AIML inference functions). In either cases, the coordination function may rely on network states analytics functions which provide insights that characterize the state of the network into specific states.

#### 6.5.3.3	Requirements for AI/ML inference capabilities management

Table 6.5.3.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_CAP-01 | The AI/ML inference MnS Producer shall have a capability allowing an authorized MnS consumer to request the capabilities of existing ML entities available within the AI/ML inference producer. | Identifying capabilities of ML entities (clause 6.5.3.2.1) |
| REQ- ML_CAP-02 | The AI/ML inference MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as a decision described as a triplet <object(s), parameters, metrics> with the entries respectively indicating: the object or object types for which the ML entity can undertake optimization or control; the configuration parameters on the stated object or object types, which the ML entity optimizes or controls to achieve the desired outcomes; and the network metrics which the ML entity optimizes through its actions. | Identifying capabilities of ML entities (clause 6.5.3.2.1) |
| REQ-ML_CAP-03 | The AI/ML inference MnS Producer shall have a capability to report to an authorized MnS consumer the capabilities of an ML entity as an analysis described as a tuple <object(s), characteristics> with the entries respectively indicating: the object or object types for which the ML entity can undertake analysis; and the network characteristics (related to the stated object or object types) for which the ML entity produces analysis | Identifying capabilities of ML entities (clause 6.5.3.2.1) |
| REQ-ML_CAP-04 | The AI/ML inference MnS Producer shall have a capability allowing an authorized MnS consumer to request a mapping of the consumer's inference targets to the capabilities of one or more ML entities. | Mapping of the capabilities of ML entities (clause 6.5.3.2.2) |
| REQ-ML_COORD-1 | The MnS producer of AI/ML inference or AI/ML inference coordination should have a capability to inform an authorized consumer of its actions that should be evaluated. | Coordinating decisions of AI/ML inference functions (clause 6.5.3.2.2) |
| REQ-ML_COORD-2 | The MnS producer of AI/ML inference or AI/ML inference coordination should support the capability to report to the MnS consumer an index that describes the impacts of one or more actions undertaken by AI/ML inference producer. | Coordinating decisions of AI/ML inference functions (clause 6.5.3.2.2) |

End of changes