3GPP TSG-SA5 Meeting #160	S5-251320

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0194 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 add solutions for edge application deployment location analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AsiaInfo, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is proposed to add the solutions for Edge application deployment location analysis in TS 28.104. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the solutions of Edge application deployment location analysis. Add corresponding MDA type in clasue 8.6.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case of Edge application deployment location analysis will not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.Y(New), 8.4.2.Y.1(New), 8.4.2.Y.2(New), 8.4.2.Y.3(New), 8.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Void

[3]	3GPP TS 28.535: "Management and orchestration; Management services for communication service assurance; Requirements".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "Management and orchestration;5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication managemen; Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[15]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[16]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[18]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire".

[19]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[20]	3GPP TS 28.511: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures".

[21]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[22]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[23]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[24]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/Machine Learning (AI/ML) management".

[25]	3GPP TS 32.160: "Management and orchestration; Management service template".

[26]	ETSI GS NFV-IFA 011 (V3.3.1): "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; VNF Descriptor and Packaging Specification".

[27]	Recommendation ITU-T X.733: "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[28]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[29]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[z]	3GPP TS 28.538: " Management and orchestration; Edge Computing Management (ECM)".

Next change

8.4.2.Y	 Edge application deployment location analysis

8.4.2.Y.1	MDA type

The MDA type for Edge application deployment location analysis is: SLSAnalysis.EdgeApplicationDeploymentLocationAnalysis.

8.4.2.Y.2	Enabling data

The enabling data for SLSAnalysis.EdgeApplicationDeploymentLocationAnalysis MDA type are provided in table 8.4.2.Y.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.2.Y.2-1: Enabling data for edge application deployment location

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Packet Delay on air-interface | Average delay DL air-interface (clause 5.1.1.1.1 TS 28.552 [4]); (Average delay UL on over-the-air interface (clause 5.1.1.1.3 TS 28.552 [4]) |
|  | Round-trip Packet Delay | Round-trip packet delay between PSA UPF and NG-RAN (clause 5.4.8 TS 28.552 [4]). |
|  | Registered subscribers measurement | Registered subscribers measurement as defined in clause 5.2.1 in Ts 28.552[4] |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (clause 6.3.1.2 in TS 28.554 [5]); Integrated uplink delay in RAN (clause 6.3.1.7 in TS 28.554 [5]). |
|  | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance as defined in clause 6.3.2 in TS 28.554 [5]; Downstream throughput for Single Network Slice Instance as defined in clause 6.3.3 in TS 28.554 [5]. |
|  | RAN UE Throughput | RAN UE Throughput as defined in clause 6.3.6 in TS 28.554 [5]. |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface as defined in clause 6.3.4 in TS 28.554 [5]; Downstream Throughput at N3 interface as defined in clause 6.3.5 in TS 28.554 [5]. |
| UE location reports | UE location information provided by the LMF services which can be used to correlate with the MDT reports. | The UE location information provided by LMF via service-based interface (see TS 23.273 [14]). |
| QoE Data | The QoE data of the different services | QoE data (TS 26.247 [22] and TS 26.114 [23] can be acquired through the procedures defined in TS 28.405 [8]). |
| EDN Identifier | It defines the identifier of the edge data network | ednIdentifier as defined in clause 6.3.10 in TS 28.538[z] |
| EDN Connection Information | It defines the set of information needed to connect to an EDN. | eDNConnectionInfo as defined in clause 6.3.10 in TS28.538 [z] |
| Available Edge Virtual Resources | It defines the available edge virtual resources managed by an EDN | availableEdgeVirtualResources as defined in clause 6.3.10 in TS28.538 [z] |

8.4.2.Y.3	Analytics output

The specific information elements of the analytics output for edge application deployment location analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.Y.3-1.

Table 8.4.2.Y.3-1: Analytics output for edge application deployment location

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| recommendedEDN | Edge Data Network where the EAS is recommended to be deployed | M | Properties: type: DN, multiplicity: 1, isOrdered: N/A, isUnique: N/A, defaultValue: None, isNullable: False |

Next change

8.6	Enumerations

8.6.1	MDAType <<enumeration>>

Table 8.6.1-1: <<enumeration>> MDAType

| Enumeration value | Description |
| --- | --- |
| "COVERAGEANALYTICS_COVERAGEPROBLEMANALYSIS" | Indicates that the MDA type for the Coverage problem analysis defined in clause 8.4.1.1 |
| "COVERAGEANALYTICS_PAGINGOPTIMIZATION" | Indicates that the MDA type for the Paging Optimization defined in clause 8.4.1.2 |
| "SLSANALYSIS_SERVICEEXPERIENCEANALYSIS" | Indicates that the MDA type for the Service experience analysis defined in clause 8.4.2.1 |
| "SLSANALYSIS_NETWORKSLICETHROUGHPUTANALYSIS" | Indicates that the MDA type for the Network slice throughput analysis defined in clause 8.4.2.2 |
| "SLSANALYSIS_NETWORKSLICETRAFFICANALYSIS" | Indicates that the MDA type for the Network slice traffic prediction defined in clause 8.4.2.3 |
| "SLSANALYSIS_E2ELATENCYANALYSIS" | Indicates that the MDA type for the E2E latency analysis defined in clause 8.4.2.4 |
| "SLSANALYSIS_NETWORKSLICELOADANALYSIS" | Indicates that the MDA type for the Network slice load analysis defined in clause 8.4.2.5 |
| "MDAASSISTEDFAULTMANAGEMENT_FAILUREPREDICTION" | Indicates that the MDA type for the MDA assisted failure prediction defined in clause 8.4.3.1 |
| "MDAASSISTEDENERGYSAVING_ENERGYSAVINGANALYSIS" | Indicates that the MDA type for the Energy saving analysis defined in clause 8.4.4.1 |
| "MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS" | Indicates that the MDA type for the Mobility performance analysis defined in clause 8.4.5.1 |
| "MOBILITYMANAGEMENTANALYTICS_HANDOVEROPTIMIZATION" | Indicates that the MDA type for the Handover Optimization analysis defined in clause 8.4.5.2 |
| "MAINTENANCE_MAINTENANCEANALYTICS" | Indicates that the MDA type for the Maintenance.MaintenanceAnalytics defined in clause 8.4.6.1 |
| "RESOURCEANALYTICS_VIRTUALIZEDRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Virtualized resource utilization analysis defined in clause 8.4.7.1.1 |
| "RESOURCEANALYTICS_PHYISCALRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Physical resource utilization analysis defined in clause 8.4.7.1.2 |
| "RESOURCEANALYTICS_5GCCONTROLPLANECONGESTIONANALYSIS" | Indicates that the MDA type for the 5GC Control plane congestion analysis defined in clause 8.4.7.1.3 |
| "PREDICTIONS_PMDATA" | Indicates that the MDA type for the MDA assisted PM predictions defined in clause 8.4.8.1 |
| "UETHROUGHPUTANALYTICS_UETHROUGHPUTPROBLEMANALYSIS" | Indicates that the MDA type for the UE throughput analysis defined in clause 8.4.2.6 |
| "SLSANALYSIS_EDGEAPPLICATIONDEPLOYMENTLOCATIONANALYSIS" | Indicates that the MDA type for the Edge application deployment location analysis defined in clause 8.4.2.Y |

End of change