3GPP TSG-SA5 Meeting #160	S5-251342

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.271 | CR | 0030 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend basic priciples, CDR generation and handling for 5G LCS converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no basic priciples, CDR generation and handling for 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend basic priciples, CDR generation and handling to support 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G LCS converged charging will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.4.1	Basic principles

5.4.1.1	General

Converged charging may be performed by the GMLC interacting with CHF using Nchf specified in TS 32.290 [55] and TS 32.291 [56]. In order to provide the data required for the charging activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the GMLC shall be able to perform converged charging for 5G LCS defined in TS 23.273 [202] and Ranging based services and Sidelink Positioning defined in TS 23.586 [206].

The GMLC shall be able to perform converged charging by interacting with CHF, for charging data related to 5G LCS. The Charging Data Request and Charging Data Response are exchanged between the GMLC and the CHF, based on PEC scenarios specified in TS 32.290 [55]. The Charging Data Request is issued by the GMLC towards the CHF when certain conditions (chargeable events) are met.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following sub-clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [56].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

5.4.1.2	Applicable Triggers in the GMLC

5.4.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. SUPI).

Each trigger condition (i.e. chargeable event) defined for the LCS converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.4.1.2.1.1 summarizes the set of default trigger conditions and their category which shall be supported by the GMLC when charging is active for the corresponding GMLC functionality related to 5G LCS. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from GMLC towards the CHF.

Table 5.4.1.2.1.1: Default Trigger conditions in GMLC

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Receive User Location from AMF | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| Send Ranging/SL Positioning Service Respond to trusted AF or LCS Client | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |

Next change

5.4.3	CDR generation

5.4.3.1	Introduction

The CHF CDRs for 5G LCS charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in details the conditions for generating, opening and closing the CHF CDR, which shall be supported by the CHF.

5.4.3.2	Triggers for CHF CDR

5.4.3.2.1	General

A CHF CDR is used to collect charging information related to 5G LCS chargeable events for PEC.

5.4.3.2.2	Triggers for CHF CDR generation

A CHF CDR is generated by the CHF for each received Charging Data Request[Event].

End of changes