3GPP TSG-SA5 Meeting #160	S5-251355

Goteborg, Sweden, 07 - 11 April 2025

# Source:	ZTE Corporation, NEC, Intel

# Title:	Input to DraftCR Rel-19 TS 28.105 Add Procedure Diagram for FL Server selection and determination

# Document for:	Approval

Agenda Item:	6.19.1.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

-

3	Rationale

The Use case for FL is proposed and agreed in SA5 #159. This contribution proposes to add the procedure diagram for FL to explicitly demonstrate the interaction between FL consumer and FL producer under the SBMA context. The selection and determination of the FL server can be taken by the consumer or the producer. Procedure for these two scenario are both provided.

4	Detailed proposal

Start of the First Change

7.x	Procedures for FL management

7.x.1	Introduction

This clause describes the procedures for FL management.

7.x.2	Consumer initiated FL Server selection and determination

Figure 7.x.2-1 illustrates the procedure for consumer initiated FL Server Selection and determination.

Figure 7.x.2-1: Procedure for consumer initiated FL Server selection and determination

1.	MnS Consumer for FL sends a request to the Registration & Discovery MnS Producer to query the DN(s) of the ML Training Function(s).

2.	Registration & Discovery MnS Producer responses with the DN(s) of the ML Training Function(s).

3.	Based on the received DN(s) in Step 2, MnS Consumer for FL sends a request to query the FL information of the ML Training Function.

4.	ML Training Functions response with the FL information of the ML Training Function.

NOTE 1:	The FL information should align with the agreed solution of FL.

5.	MnS Consumer for FL selects and determines the ML Training Function acting as the FL server according to the received FL information of each ML Training Function.

6.	MnS Consumer for FL selects ML Training Function to act as the FL Server for the future training request for an ML model.

NOTE 2: the procedure should be enhanced to align with the agreed SOL.

7.x.3	Producer initiated FL Server selection and determination

Figure 7.x.3-1 illustrates the procedure for producer initiated FL Server Selection and determination.

Figure 7.x.3-1: Procedure for producer initiated FL Server selection and determination

1.	MnS Consumer for FL sends a request to the Registration & Discovery MnS Producer to query the DN of the ML Training Function.

2.	Registration & Discovery MnS Producer responses with the DN of the ML Training Function.

3.	Based on the received DN in Step 2, MnS Consumer for FL sends a request to the ML Training MnS Producer to perform FL.

4.	ML Training MnS Producer evaluates if it can act as the FL Server.

If the ML Training Function can act as the MnS Producer for FL, Step 5 is performed.

5.	ML Training MnS Producer responses to the FL Consumer with the DN of the FL Server, i.e., its own DN.

If the ML Training Function cannot act as the MnS Producer for FL, Step 6-11 is performed.

6.	ML Training MnS Producer sends a request to the Registration & Discovery MnS Producer to query the DN(s) of the ML Training Function(s).

7.	Registration & Discovery MnS Producer responses with the DN(s) of the ML Training Function(s).

8.	Based on the received DN(s) in Step 7, ML Training MnS Producer sends a request to query the FL information of each ML Training Function.

9.	ML Training Functions response with the FL information of the ML Training Function.

10. ML Training MnS Producer selects and determinates the FL Server.

11.	ML Training MnS Producer responses to the MnS Consumer for FL with the DN of the FL Server.

NOTE: the procedure should be enhanced to align with the agreed NRM solution.

End of the First Change

Start of the Second Change

Annex A (informative):
PlantUML source code

A.1	Procedures for FL management

A.1.1	Consumer initiated FL Server selection and determination

@startuml

title "[Procedure for Consumer initiated FL Server selection and determination]"

actor "MnS Consumer" as MnS_Consumer

participant "Registration & Discovery \n MnS Producer" as RD_Producer

collections "ML Training \n Function" as MLT

participant "ML Training Function \n (FL Server)" as MLT_Server

MnS_Consumer -> RD_Producer: 1.Request to query the DN(s) of ML Training Function(s)

RD_Producer -> MnS_Consumer: 2. Response with the DN(s) of ML Training Function(s)

MnS_Consumer -> MLT: 3.Request to query the FL Information of the ML Training Function

MnS_Consumer -> MLT_Server:

MLT -> MnS_Consumer: 4. Response with the FL Information of the ML Training Function

MLT_Server -> MnS_Consumer:

MnS_Consumer -> MnS_Consumer: 5. Select and determine the FL Server

MnS_Consumer <-> MLT_Server: 6. Request the ML Training Function to act as the FL Server

hide footbox

@enduml

A.1.2	Producer initiated FL Server selection and determination

@startuml

title "[Procedure for Producer initiated FL Server selection and determination]"

actor "MnS Consumer" as MnS_Consumer

participant "Registration & Discovery \n MnS Producer" as RD_Producer

participant "ML Training MnS Producer \n (ML Training Function)" as MnS_Producer

collections "ML Training \n Function" as MLT

MnS_Consumer -> RD_Producer: 1.Request to query the DN of ML Training Function

RD_Producer -> MnS_Consumer: 2. Response with the DN of ML Training Function

MnS_Consumer -> MnS_Producer: 3.Request to perform FL

MnS_Producer -> MnS_Producer: 4. Evaluate if the ML Training Function \n can act as the FL Server

alt The ML Training Function can act as the FL Server

MnS_Producer -> MnS_Consumer: 5.Response the DN of the FL Server

else The ML Training Function cannot act as the FL Server

MnS_Producer -> RD_Producer: 6. Request to query the DN(s) of ML Training Function(s)

RD_Producer -> MnS_Producer: 7. Response with the DN(s) of ML Training Function(s)

MnS_Producer -> MLT: 8. Query the FL information of the ML Training Function(s)

MLT -> MnS_Producer: 9. Response with the FL information of each ML Training Function(s)

MnS_Producer -> MnS_Producer: 10. Select and Determinate the FL Server

MnS_Producer -> MnS_Consumer: 11.Response the DN of the FL Server

end

hide footbox

@enduml

End of the Second Change