3GPP TSG-SA5 Meeting #160	S5-251356

Goteborg, Sweden, 07 - 11 April 2025

# Source:	ZTE Corporation, NEC, Intel

# Title:	Input to DraftCR Rel-19 TS 28.105 Add Procedure Diagram for FL Client selection

# Document for:	Approval

Agenda Item:	6.19.1.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

-

3	Rationale

The Use case for FL is proposed and agreed in SA5 #159. This contribution proposes to add the procedure diagram for FL to explicitly demonstrate the interaction between FL consumer and FL producer under the SBMA context. The selection of the FL clients is taken by the FL server as the FL producer.

4	Detailed proposal

Start of the First Change

7.x	Procedures for FL management

7.x.1	Introduction

This clause describes the procedures for FL management.

7.x.2	Procedure for FL Client Selection

Figure 7.x.2-1 illustrates the procedure for FL Client Selection.

Figure 7.x.2-1: Procedure for FL Client Selection

Precondition: the ML Training Function acting as the FL Server is determined already.

If the consumer initiates the FL client selection procedure, Step 1a-2a are performed.

1a. The MnS producer for FL (FL server) receives a request from the MnS consumer for FL to create an MLTrainingRequest MOI for a training an ML model that needs to be trained by FL on the producer.

2a. The MnS producer for FL (FL server) creates an MLTrainingRequest MOI

If the producer initiates the FL client selection procedure, Step 1b-2b are performed.

1b. The MnS producer for FL (FL server) decides to train an ML model by FL, without the request from consumer.

2b. The MnS producer for FL (FL server) retrieves the pre-defined fLClientSelectionCriteria.

3. MnS producer for FL (FL Server) sends a request to the Registration & Discovery MnS Producer to query the DN(s) of the ML Training Function(s).

4.	Registration & Discovery Producer responses with the DN(s) of the ML Training Function(s).

5.	Based on the received DN(s) in Step 4, The MnS producer for FL (FL Server) sends a request to query the FL information of the ML Training Function.

6.	ML Training Functions response with the FL information of the ML Training Function.

7.	The MnS producer for FL (FL Server) selects and determines the ML Training Functions acting as the FL clients according to the fLClientSelectionCriteria.

8.	The MnS producer for FL (FL Server) requests the ML Training Function to join the FL process as the FL Client.

9.	ML Training Function responses to the MnS producer for FL (FL Server) to confirm joining the FL process as the FL Client.

10.  The MnS producer for FL (FL Server) creates the MLTrainingProcess MOI that containing the DN of the participating FL clients, and sends the notification about creation of this MOI to the consumer.

NOTE: the procedure should be enhanced to align with the agreed NRM Solution.

End of the First Change

Start of the Second Change

Annex A (informative):
PlantUML source code

A.1	Procedures for FL management

@startuml

title "[Procedure for FL Client selection]"

actor "MnS Consumer for FL" as MnS_Consumer

participant "MnS Producer for FL \n (ML Training Function, FL Server)" as FL_Producer

participant "Registration & Discovery \n MnS Producer" as RD_Producer

collections "ML Training Function \n (FL Clients)" as MLT

alt Consumer initiated FL Client selection

MnS_Consumer -> FL_Producer: 1a.Request to create MLTrainingRequest MOI \n including fLClientSelectionCriteria

FL_Producer -> FL_Producer: 2a. Create the MLTrainingRequest MOI

else Producer initiated FL Client selection

FL_Producer -> FL_Producer: 1b.The MnS producer for FL decides to train an ML model by FL

FL_Producer -> FL_Producer: 2b. Retrieve the pre-defined fLClientSelectionCriteria

end

FL_Producer -> RD_Producer: 3. Request to query the DN(s) of ML Training Function(s)

RD_Producer -> FL_Producer: 4. Response with the DN(s) of ML Training Function(s)

FL_Producer -> MLT: 5.Request to query the FL Information of the ML Training Function(s)

MLT -> FL_Producer: 6.Response with the FL Information of the ML Training Function(s)

FL_Producer -> FL_Producer: 7. Select and determine the FL Clients based on the FLClientCriterion

FL_Producer -> MLT: 8. Request the ML Training Function to join the FL process as the FL Client

MLT -> FL_Producer: 9. Response that the ML Training Function joins the FL process as the FL Clinet

FL_Producer -> MnS_Consumer: 10. Notify the DN(s) of FL Clients

hide footbox

@enduml

End of the Second Change