3GPP TSG-SA5 Meeting #160	S5-251360

Goteborg, Sweden, 07 - 11 April 2025

# Source:	ZTE Corporation

# Title:	Input to DraftCR Rel-19 TS 28.105 Enhancement on AIMLSupportedFunction to support inference in LMF

# Document for:	Approval

Agenda Item:	6.19.1.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

-

3	Rationale

As specified in clause 4.3.8 in TS 23.273, it supports requesting ML model provision from NWDAF containing MTLF for LMF-based AI/ML Positioning, i.e., the inference for Positioning can be performed in the LMF. Management support for LMF based AI/ML Positioning is missing.

4	Detailed proposal

Start of the First Change

7.3a.4	Information model definitions for ML inference

7.3a.4.1	Class diagram

7.3a.4.1.1	Relationships

Figure 7.3a.4.1.1-1: NRM fragment for ML update

NOTE 1:	The ManagedEntity and AIMLSupportedFunction shall not represent the same MOI.

NOTE 2:	For AnLFFunction, DMROFunction, DLBOFunction, and DESManagementFunction see [18] and for MDAFunction see [2].

Figure 7.3a.4.1.1-2: NRM fragment for AI/ML inference function

End of the First Change

Start of the Second Change

7.3a.4.2.5	AIMLInferenceFunction

7.3a.4.2.5.1	Definition

This IOC represents the common properties of the AI/ML inference function.

This AIMLInferenceFunction instance can be created by the system or pre-installed.

The AIMLInferenceFunction MOI may be associated with one or more MOIs that represent the functions/functionalities (Note) provided by the subject AIMLInferenceFunction MOI.

The AIMLInferenceFunction MOI can be only created by the MnS producer but not consumer.

The MOI of AIMLInferenceFunction or the MOI of the IOC inheriting from the AIMLInferenceFunction IOC contains one or more MOI(s) of MLModel.

NOTE:	The IOCs representing the functions/functionalities (Note) that use the AI/ML inference function include MDAFunction, AnLFFunction, DMROFunction, DLBOFunction, LMF, and DESManagementFunction.

The AIMLInferenceFunction MOI may be contained by either a SubNetwork MOI, a ManagedElement MOI, or an MOI of ManagedFunction’s subclass, and it is allowed for an MnS producer to support multiple AIMLInferenceFunction MOIs contained in different superordinated MOIs among SubNetwork, ManagedElement and the ManagedFunction’s subclass.

The generation of inference outputs is based on the configuration of inference, e.g., to start a stated time, or to be executed at all times. The observations of the inference function and information on derived Outputs is registered in the inference report.

End of the Second Change

Start of the Third Change

A.12	PlantUML code for Figure 7.3a.4.1.1-2: NRM fragment for AI/ML inference function

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

skinparam nodesep 60

class AIMLInferenceFunction <<InformationObjectClass>>

class AIMLInferenceReport <<InformationObjectClass>>

class MLModel <<InformationObjectClass>>

class ManagedEntity <<ProxyClass>>

class AIMLSupportedFunction <<ProxyClass>>

ManagedEntity  "1" *-- "*" AIMLInferenceFunction : <<names>>

AIMLInferenceFunction  "*" <-l-> "*" AIMLSupportedFunction

MLModel  "*" <-r-> "*" AIMLSupportedFunction

MLModel  "*" <-r-> "*" AIMLInferenceFunction

MLModel  "1..*" <-r-> "*" AIMLInferenceReport

AIMLInferenceFunction "1" *-- "*" AIMLInferenceReport : <<names>>

note right of ManagedEntity #white

Represents the IOCs:

ManagedElement or

SubNetwork or

ManagedFunction

end note

note top of AIMLSupportedFunction #white

Represents the IOCs:

DMROFunction or

DLBOFunction or

DESManagementFunction or

MDAFunction or

AnLFFunction or

LMF

end note

@enduml

End of the Third Change