3GPP TSG-SA5 Meeting #160	S5-251455

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0684 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS28.552 Enhance RRC connection resuming related measurements for RedCap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_OAM |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This submission implements the solution for “REQ-RedCap-Perf-RRCNum-3” as recommended in TR 28.876 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE type as subcounter for: Number of RRC connection resuming attempts Successful RRC connection resuming |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.18.1, 5.1.1.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 5.1.1.18.1	Number of RRC connection resuming attempts

a)	This measurement provides the number of RRC connection resuming attempts.

b)	CC.

c)	On Receipt of the RRCResumeRequest message or RRCResumeRequest1 from UE.Each RRCResumeRequest is added to the relevant subcounter per resume cause. The measurement is filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

d)	Each subcounter is an integer value.

e)

RRC.ResumeAtt， or
RRC.ResumeAtt.Cause，where Cause indicates the resume cause defined in clause 6.2.2 of TS 38.331 [20], or
RRC.ResumeAtt_RedCapUE, or
RRC.ResumeAtt.Cause_RedCapUE, where RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the number of RRC connection resuming attempts of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

2nd change

##### 5.1.1.18.2	Successful RRC connection resuming

a)	This measurement provides the total successful number of RRC connection resuming.

b)	CC.

c)	On Receipt of a RRCResumeComplete message from UE for RRC connection resuming. Each successful RRC connection resuming is added to the relevant subcounter per resume cause. The measurement is filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

d)	Each subcounter is an integer value.

e)	RRC.ResumeSucc, or

RRC.ResumeSucc.Cause

, Where Cause indicates the resume cause defined in clause 6.2.2 of TS 38.331 [20], or

RRC.ResumeSucc_RedCapUE, or

RRC.ResumeSucc.Cause_RedCapUE, where RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the total successful number of RRC connection resuming of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

End of changes