3GPP TSG-SA5 Meeting #160	S5-251456

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0685 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS28.552 Enhance PRB Usage related measurements for RedCap UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_OAM |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This submission implements the solution for Use case #6: Performance evaluation on resource load of RedCap network service in TR 28.876 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE type as subcounter for: DL/UL Total PRB Usage DL/UL PRB Usage per SSB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.1,5.1.1.2.2, 5.1.1.2.15,5.1.1.2.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 5.1.1.2.1	DL Total PRB Usage

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the downlink for any purpose. The measurement is optionally is filterable per PLMN ID and per supported S-NSSAI and per QoS resource type (Non-GBR, GBR, Delay-critical GBR, as specified in TS 23.501[4]) and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	SI

c)	This measurement is obtained as: , where is the DL PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for DL traffic transmission per PLMN ID and per supported S-NSSAI and per QoS resource type shall be included; is total number of PRBs available for DL traffic transmission during time period ; and is the time period during which the measurement is performed.

d)	A single integer value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN and the number of supported S-NSSAIs and the number of supported S-NSSAIs multiplied by the number of QoS resource types.

e)	The measurement name has the form RRU.PrbDl, which indicates the DL PRB Usage for all traffic, or optionally RRU.PrbDl.PLMN, where the PLMN identifies the PLMN ID, or optionally RRU.PrbDl.SNSSAI, where the SNSSAI identifies the S-NSSAI, or optionally RRU.PrbDl.SNSSAI.ResType, where the ResType identifies the resource type of 5G QoS characteristics, or optionally RRU.PrbDl_RedCapUE, where RedCapUE is the filter with value 1 indicating  (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the usage (in percentage) of physical resource blocks (PRBs) on the downlink of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

next change

##### 5.1.1.2.2	UL Total PRB Usage

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the uplink for any purpose. The measurement is optionally is filterable per PLMN ID and per supported S-NSSAI and per QoS resource type (Non-GBR, GBR, Delay-critical GBR, as specified in TS 23.501[4]) and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	SI

c)	This measurement is obtained as: , where is the UL PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of full physical resource blocks and all PRBs used for UL traffic transmission per PLMN ID and per supported S-NSSAI and per QoS resource type shall be included; is total number of PRBs available for UL traffic transmission during time period ; and is the time period during which the measurement is performed

d)	A single integer value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN and the number of supported S-NSSAIs and the number of supported S-NSSAIs multiplied by the number of QoS resource types.

e)	The measurement name has the form RRU.PrbUl, which indicates the UL PRB Usage for all traffic, or optionally RRU.PrbUl.PLMN, where the PLMN identifies the PLMN ID, or optionally RRU.PrbUl.SNSSAI, where the SNSSAI identifies the S-NSSAI, or optionally RRU.PrbUl.SNSSAI.ResType, where the ResType identifies the resource type of 5G QoS characteristics, or optionally RRU.PrbUl_RedCapUE where RedCapUE is the filter with value 1 indicating  (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the usage (in percentage) of physical resource blocks (PRBs) on the uplink of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

next change

##### 5.1.1.2.15	DL PRB Usage per SSB

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) per SSB beam of the NR cell on the downlink for any purpose. The measurement is optionally  is filterable per QoS resource type (Non-GBR, GBR, Delay-critical GBR, as specified in TS 23.501[4]) and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	SI

c)	This measurement is obtained as: , where is the DL total PDSCH PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of all PDSCH PRBs used for DL traffic transmission for the SSB beam per QoS resource type; is the cell total PDSCH PRB number during time period ; and is the time period during which the measurement is performed.

d)	A single float value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of QoS resource types.

e)	The measurement name has the form RRU.PrbDlSSB, which indicates the DL PRB Usage for all traffic,

or optionally RRU.PrbDlSSB.ResType, where the ResType identifies the resource type of 5G QoS characteristics, or optionally RRU. PrbDlSSB_RedCapUE where RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the usage (in percentage) of physical resource blocks (PRBs) per SSB beam on the downlink of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for radio resource status, which may be used as the input for AI/ML model training at OAM, such as load balacing.

next change

##### 5.1.1.2.16	UL PRB Usage per SSB

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) per SSB beam of the NR cell on the uplink for any purpose. The measurement is optionally is filterable per QoS resource type (Non-GBR, GBR, Delay-critical GBR, as specified in TS 23.501[4]) and filterable based on whether UE is (e)RedCap UE(See procedure in TS 38.473 clause 8.4.1 Initial UL RRC Message Transfer or TS 38.331 clause 5.6.1 UE capability transfer).

b)	SI

c)	This measurement is obtained as: , where is the UL total PUSCH PRB usage, which is percentage of PRBs used, averaged during time period  with value range: 0-100%; is a count of all PUSCH PRBs used for UL traffic transmission for the SSB beam per QoS resource type; is the cell total PUSCH PRB number during time period ; and is the time period during which the measurement is performed.

d)	A single float value from 0 to 100. If the optional measurements are performed, the number of measurements is equal to the number of QoS resource types.

e)	The measurement name has the form RRU.PrbTotUlSSB, which indicates the UL PRB Usage for all traffic,

or optionally RRU.PrbUlSSB.ResType, where the ResType identifies the resource type of 5G QoS characteristics, or optionally RRU. PrbUlSSB_RedCapUE where RedCapUE is the filter with value 1 indicating (e)RedCapUE (see in 3GPP TS 38.306 clause  4.2.21.1 Definition of RedCap UE and 4.2.22.1 Definition of eRedCap UE), it represents the usage (in percentage) of physical resource blocks (PRBs) per SSB beam on the uplink of (e)RedCap UEs. Value 0 indicates non-(e)RedCapUE.

f)	Beam

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for radio resource status, which may be used as the input for AI/ML model training at OAM, such as load balacing.

End of changes