3GPP TSG-SA5 Meeting #160	S5-251497

Goteborg, Sweden, 07 - 11 April 2025

# Source:	China Mobile

# Title:	Input to DraftCR Rel-19 TS 28.105 Add UC for AI explainability

# Document for:	Approval

Agenda Item:	6.19.1.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal..

2	References

None.

3	Rationale

This contribution proposes to add the use case of AI explainability agreed in TR 28.858.

4	Detailed proposal

Start of the First Change

6.5	AI/ML inference

6.5.x	ML explainability

6.5.x.1		Description

Explainable ML refers to a process that enables the consumers (e.g., human operator) to understand and trust the outputs provided by ML models. In essence, explainable ML is about making the decision-making of ML comprehensible to its consumers. .

6.5.x.2	Use cases

6.5.x.2.1	Explanation report in AI/ML inference

The explanation is an additional information that the MnS consumer can use to analyze the ML model’s potential impact in terms of network performance gain and make informed decisions regarding deactivation of AI/ML inference or fallback to previous version of the ML model.

In this case, the MnS consumer may indicate the need for explainability so that the MnS producer can preprocess the training data or employ suitable techniques to generate the explanations. The generated explanations by the AI/ML inference MnS producer may need to be reported to the AI/ML inference MnS consumer. The MnS consumer can utilize this explanations information to analyze the ML model’s potential impact on network performance. By considering network performance PM/KPIs, model performance, and the explanations, the MnS consumer can effectively identify the ML model contributing to network performance degradation when several ML models are active in the network.

6.5.x.3	Requirements for ML explainability

Table 6.5.x.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_EXP-01 | The 3GPP management system should provide the capability for an authorized consumer to indicate the support for explanations in ML model training. | Local explanation in AI/ML inference (clause 6.5.x.2.1) |
| REQ- AI/ML_EXP-02 | The 3GPP management system should provide the capability for an authorized consumer to receive from the AI/ML inference MnS producer the generated explanations for inference. | Local explanation in AI/ML inference (clause 6.5.x.2.1) |

End of the First Change