3GPP TSG SA WG5 Meeting #160	S5-251542

Goteborg, SWEDEN 7 - 11 April 2025

# Source:	Samsung

# Title:	NDT Inducement Requirements

# Document for:	Approval

Agenda Item:	6.19.5.3

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

The requirements for Inducement are missing.

# 4	Detailed proposal

Start of First change

## 5.2		NDT support for network automation

### 5.2.1	Description

NDTs can be used to support automation use cases (e.g. MDA, SON, etc.). An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it could be possible for the network automation function to configure the NDT and the scenario that could be modeled and simulated by the NDT. NDTs do not execute any actions in form of configuration of the live network but support decision-making.

### 5.2.2	Use cases

#### 5.2.2.1	General capabilities on NDT support for network automation

An NDT, depending upon the network or service management use case and scenario to be modeled and simulated, might need data originating from various sources (network data, environment data, analytic, UEs data) and suitable hardware/software resources to function properly. MnS consumers would need to specify needed NDT characteristics or configurations to the NDT tailored to fulfil consumer specific needs i.e. to define the consumer preference for the specific NDT. For example, consumer preferences may be related to environment data sources e.g. weather, synthetic data etc, data characteristics (e.g. robustness, data granularity, maximum tolerable latency), required NDT output latency, characteristics of the service to be twinned, resource constraints (HW/SW), etc. Furthermore, in the case that consumer's preference on NDT characteristics or configuration may change over time and MnS consumer may update the NDT with the needed changes.

The achieved performance by NDT may depend on NDT characteristics such as e.g. load; the simulation results would change based on the network load, time; the simulation results would change based on the target time of the day, energy saving state; the simulation results would change considering whether the simulated node is running in energy saving mode.

The automation scenarios that the NDT might support include those in following sub-clauses.

#### 5.2.2.2	Network failure and risk prediction

Each operations for network optimization and maintenance on mobile network may cause potential network failures and risks, especially high-risk operations, such as potentially dangerous configuration modification, policy modification, software version upgrade, and board switching, which may cause network congestion and network breakdown. Operators cannot carry out the high-risk network operations in the physical network directly network optimization. Therefore, utilization of NDT is a better way for evaluation of high-risk network operations.

Using NDT, operations causing potential network failures can be identified . NDT can also be utilized for optimization and verification of network policies and solutions for the further risk avoidance. After simulating/emulating a scenario, NDT can report the results back to the consumer. Based on results, The consumer can evaluate and verify possible network policies and solutions to minimize the impact of high-risk operations.

As an example, SLA degradation and failure of single node in mobile network can also be predicted using the NDT. When it is predicted that the network resources in the network domain are not enough to maintain the SLA or hardware resources failure in the future, the NDT results helps the consumer to act appropriately to avoid  network failure.

#### 5.2.2.3	Support to prediction of failure events including Signaling storm

A failure event refers to the situation where users in the network are completely unable to get service form the network. An example failure events is a signaling storm, which is the situation where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm. When a signaling storm occurs on the network, Network Functions (NFs) such as AMF, SMF, and UDM in the 5G network might all be potential impact points during a signaling storm . The NDT can be used to validate whether the proposed solution can resolve the signaling storm issue.

The NDT can validate and report the network’s ability to defend against a possible signaling storm.

The NDT can also model network behaviors and provide information on potential impact of network failures including on surges in signaling requests on the network. For a signaling storm, for example, the information can that enable to identify the optimal flow control parameters for each signaling impact point.

#### 5.2.2.4	Network issue inducement capability - INDUC

This use case describes how a network issue can be induced using NDT. In order to develop a resilient network, the behavior and performance of the network should be monitored during certain network failure issue e.g. node/functionality failure, service degradation etc. In order to plan for the optimal network configuration in case of such network failure issue, the scenario itself need to be induced in the network. It is desirable, to use NDT for such an inducement process. A particular issue can be induced in the NDT instead of real network. After a particular issue is induced, the performance of the network can be monitored, other degradation/faults/failure can be identified and the mitigation actions can be decided. The following are some of the examples of the issues that can be induced.

The network slice performance degradation in terms of low PDU session establishment success rate or in terms of high latency can be induced, in a NDT, to see how the related network functions will behave when the PDU session establishment success rate is degraded. The remedial actions can be decided to mitigate the problem.

The coverage hole can be induced, in a NDT, to see how the related services are getting effected. The remedial actions can be decided to mitigate the problems arising due to the induced coverage hole.

NDT can be used for fault injection experiments avoiding impact on the physical network while measuring and monitoring the impact of each injected fault in the NDT simulation. This could be leveraged, to build a training data set for enhancing and enriching detection and diagnosing systems capabilities. In addition, NDT could be leveraged for improving root causes analysis.

5.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTAUTO-INDUC-01: | The 3GPP management system should support a capability enabling an authorized MnS consumer to request inducement of a network issue by an NDT. | NDTVER-GEN |

End Change