3GPP TSG-SA5 Meeting #160	S5-251547

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | InputToDraftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 InputToDraftCR TS 28.105 Update the description for managing ML models in use in a live network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description is not clear about how to use these KPIs to manage ML models in use for network functions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the description for managing ML models in use in a live network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance monitoring of Network Functions with ML trained models in live networks can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.x.1, 6.5.x.2.2, 6.5.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

### 6.5.x	Managing ML models in use in a live network

#### 6.5.x.1	Description

These use cases deal with managing ML models that are in-use or deployed in the live network. These use cases will apply to all supported AI/ML inference capabilities.

##### 6.5.x.2 			Use cases

##### 6.5.x.2.1	Handling of underperforming ML trained models in live networks

Actions may need to be taken by a network operator once trained ML models has been identified that is contributing towards non-optimal running of the network. These actions may involve for example, without service interruptions, reverting to running of the network without ML based optimizations (i.e deavtivating the AIMLInferenceFunction) or replacing current ML model with an earlier model one that was performing better (i.e deploying a new MLmodel).

##### 6.5.x.2.2	Performance monitoring of Network Functions with ML trained models in live networks

Several trained ML models maybe in use in an operator network with each one of them influencing network performance. The Network Functions with these ML models need to be monitored to ensure network is running optimally with these models in use. KPIs for evaluating runtime performance of Network Functions using ML models should be provided for this purpose as part of the model. These KPIs correspond to specific network functions of 5GC AI/ML supported functions and NG-RAN AI/ML supported functions, and would need to be collectable by the network operator.

#### 6.5.x.3	Requirements for managing ML models that are in-use or deployed in the live network

Table 6.5.x.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AI/ML_INF-LIVES-01 | The 3GPP management system should have a capability enabling an authorized consumer to identify ML Model(s) that caused performance measurement and/or KPI degradation. | Handling of underperforming ML trained models in live networks |
| REQ-AI/ML_INF-LIVES-02 | The 3GPP management system should have a capability to recommend remedial actions to address performance measurement and/or KPI degradation caused by ML model(s). | Handling of underperforming ML trained models in live networks |
| REQ-AI/ML_INF-LIVES-03 | The 3GPP management system should be able to collect network related performance data pertaining to Network Functions actively utilizing ML models. | Performance monitoring of Network Functions with ML trained models in live networks |

End of change