3GPP TSG-SA5 Meeting #160	S5-251583

Goteborg, Sweden, 07 - 11 April 2025

# Source:                         NTT DOCOMO

# Title:	pCR TR 28.869 Cloud-native VNF policy management evaluation

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

For approval

# 2	References

- 3GPP TR 28.869 v1.2.0 Study on cloud aspects of management and orchestration.

# 3	Rationale

The contribution proposes to add an evaluation to clause 5.1.2.4 about Cloud-native VNF policy management.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.869 [1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[2]	ETSI GS NFV-IFA 049: "Network Functions Virtualisation (NFV) Release 5; Architectural Framework; VNF generic OAM functions and other Paas Services specification".

[3]	ETSI GR NFV-EVE 019: "Network Functions Virtualisation (NFV); Architectural Framework; Report on VNF generic OAM functions".

…

[xx]	3GPP TS 28.311: "Technical Specification Group Services and System Aspects; Management and orchestration; Network policy management for mobile networks based on Network Function Virtualization (NFV) scenarios"

[yy]	3GPP TS 28.556: "Technical Specification Group Services and System Aspects; Management and orchestration; Network policy management for 5G mobile networks; Stage 2 and stage 3"

[zz]	Open Policy Agent: https://github.com/open-policy-agent

2nd Change

### 5.1.2	Use case #2：Cloud-native VNF policy management

#### 5.1.2.1	Description

3GPP TS 28.311 [xx] contains the architecture, requirements, use cases, procedures and definitions of interfaces for policy management for 4G networks. 3GPP TS 28.555 [12] specifies the concepts, requirements and use cases for network policy management in 5G networks. It is suitable for use cases where 3GPP and NFV-MANO interact for policy management (see the policy categories in ETSI GR NFV-IFA 023 [13] Table 6.2.2-1 for details). However it  lacks support for policy management for  cloud native VNFs.

The 3GPP management system needs to implement policy management for cloud-native VNFs, which might be implemented by a Microservice-based architecture. This architecture can split a single application into multiple small services, each of them can run independently. However, it will also bring many challenges, such as a large number of services will have complex dependencies, resulting in complex deployment. In this case, policy management is needed to help improve efficiency.

#### 5.1.2.2	Potential requirements

REQ-policy-1 The 3GPP management system should have the capability to manage policies for the cloud-native VNFs.

REQ-policy-2 The reference point between 3GPP management system and external OAM entity should support the capability enabling the 3GPP management system to interact with external (non-3GPP) policy management entities to perform the policy management for cloud-native VNF.

#### 5.1.2.3	Potential solutions

##### 5.1.2.3.1	Policy Agent

As shown in figure 5.1.2.3.1-1, this solution introduces a platform entity that interacts with 3GPP management system for policy management of cloud-native VNFs via a new PaaS reference point.

This solution proposes using the Policy Agent function defined in ETSI GS NFV-IFA 049 [2], which can interact with the VNF generic OAM function, other PaaS Service, and VNFs for assisting on the execution and decision-making of policies.

Figure 5.1.2.3.1-1: Potential solution for Cloud-native VNF policy management using the Policy Agent

The solution improves the efficiency in handling policies associated to various entities by interacting with VNF generic OAM functions and other PaaS Services and enables the 3GPP management system having the capability to manage policies for the cloud-native VNFs with or without interaction with NFV-MANO functions.

The present solution addresses the potential requirement REQ-policy-1 and REQ-policy-2.

This potential solution related to option 2 in clause 5.1.5.

##### 5.1.2.3.2	Use of existing 3GPP provisioning management service and ETSI NFV MANO

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for policy management purposes.

- The 3GPP management system manages VNF application specific policies according to TS 28.556 [yy] and by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532[10]).

- The 3GPP management system interacts with NFV-MANO to configure VNF non-application parameters (as defined in clause 5.1.18 of TS 28.531[7]).

The VNF policies (non-application aspects) are decided either by the MnS producer or by a system outside the 3GPP management system.

This solution is not using the VNF Generic OAM functions described in ETSI ISG NFV-IFA049 [2].

This potential solution related to option 1 in clause 5.1.5.

#### 5.1.2.4	Evaluation of solutions

The solution described in 5.1.2.3.1 about the Policy Agent is a feasible solution and can be used to support policy management for cloud native VNFs.

In more detail, the MnS producer can execute the policy related to VNF application aspects (as described in TS 28.555). Nevertheless, the cloud native VNF policy is evaluated in the Policy Agent (i.e., outside the 3GPP management system). An example realization of the Policy Agent can follow the Open Policy Agent (OPA) model for cloud-native environments [zz]). Regarding policy enforcement, the Policy Agent provides the evaluation output to an entity which enforces the decision (i.e. executes the right action based on the output provided by the Policy Agent). This means that the MnS producer needs to notify policy enforcement results learned by the Policy Agent to the MnS consumer. However, the notification mechanism described in TS 28.556 [yy] is only about reporting policy conflicts for new or updated policy(ies) and not about policy enforcement results.

The solution described in 5.1.2.3.1 is also a feasible solution however the same issue about policy results reporting is also applicable.

Updates are expected in TS 28.555 [12] and TS 28.556 [yy] for the management of policies for cloud native VNFs.

End of Changes