3GPP TSG-SA5 Meeting #160	S5-251695

Goteborg, Sweden, 7- 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.314 | CR | 0004 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Use case IAB-node connects to management system |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MOBILE_IAB_OAM |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is nomative work for Rel-19 study on management of IAB nodes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add use case for IAB-node to connect to management system |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 6.1, 6.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[3]	IETF RFC 4210: "Internet X.509 Public Key Infrastructure Certificate Management Protocol".

[4]	IETF RFC 4211: "Internet X.509 Public Key Infrastructure Certificate Request Message Format (CRMF)".

[x]	3GPP TS 38.401: "Technical Specification Group Radio Access Network; NG-RAN; Architecture description".

[y]	3GPP TS 28.532: "Technical Specification; Management and orchestration; Generic management services".

2nd Change

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Plug and Connect: The procedure by which a NE gets basic connectivity information after it is powered up and gets connected to its management system.

Software and Configuration Server (SCS): A server that provides software and configuration functions for each connected network element.

IAB-node: as defined in TS 38.401 [x].

3rd Change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CA	Certification Authority

CMP	Certificate Management Protocol

DHCP	Dynamic Host Configuration Protocol

DNS	Domain Name System

FQDN	Fully Qualified Domain Name

NAT	Network Address Translation

NE	Network Element

PnC	Plug and Connect

RA	Registration Authority

SCS	Software and Configuration Server

SeGW	Security Gateway

VLAN	Virtual LAN

IAB	Integrated Access and Backhaul

4th Change

## 6.1.x Use case IAB-node connects to management system

### 6.1.x.1 Overview

IAB architecture is specified in 3GPP TS 38.401[x]. The IAB-node connects to an upstream IAB-node or an IAB-donor-DU via a subset of the UE functionalities of the NR Uu interface (named IAB-MT function of IAB-node). IAB-node differs from static NE in term of its wireless connection with IAB-donor-node and its mobility behaviours. Mobile IAB can move to different geographic areas or traverse to different PLMNs that require to connect to different management system based on its location.

IAB-node connects to management system at power up and during mobiltiy by following PnC steps as described in clause 6.1.1. At step of IP autoconfiguration, IAB-node differs from static NE in term of ontaining IP configuration for OAM connectivity. When IAB-node powers up, IAB-node may get the initial IP configuration via PDU session that setup between IAB-MT and 5GC, using this connectivity IAB-node can obtain the initial IP configuration provided by IP autoconfiguration service (e.g. DHCP server), and additionally may perform DNS query depends on the operator's deployment scenario. The management system that IAB-node connects to may change due to IAB-node mibility behaviours. To support IAB-node connecting to a management system based on its location, IAB-node should obtain IP configuration for OAM connectivity that is associated to its location via notification operations as described in clause 6.1.x.2. With the IP configuration obtained IAB-node performs the subsequent PnC procedures, satisfies security aspects including CA/RA enrolment and connectivity to SeGW before connecting to SCS.

### 6.1.x.2 Obtain IP configuration for OAM connectivity via notifications

3GPP management system should support IAB-node to use notification mechanism to obtain IP configuration for OAM connectivity. When notification mechanism is used, 3GPP management system shall support the mapping of IP configuration for OAM connectivity with IAB-node location, see 6.1.x.3.

When IAB-node powers up and after connecting to 3GPP management system for the 1st time, the MnS producer for IAB-node shall provide location information with IAB-node indication via notifications to 3GPP management system as specified in 3GPP TS 28.532 [y] (e.g. notifyMOIChanges) to indicate its current location. Upon the reception of notifications, 3GPP management system shall provision the IP configuration for OAM connectivity (e.g. via createMOI operation) as specified 3GPP TS 28.532 [y] based on its current location.

When IAB-node moves to a new location which may need connects to new management system, the MnS producer for IAB-node shall provide new location information via "notifyMOIChanges" or "notifyMOIAttributeValueChanges" or other notifications as specified in 3GPP TS 28.532 [y] to 3GPP management system. Upon the reception of notifications of new location, 3GPP management system shall provision the new IP configuration for OAM connectivity via "modifyMOIAttributes" or other operations as specified in 3GPP TS 28.532 [y] to the MnS producer for IAB-node based on its new location.

### 6.1.x.3 IP configuration for OAM connectivity with location information

The IP configuration for OAM connectivity should include one or more the following depending on the deployment scenarios:

-	Basic IP configuration information (e.g. IP address, netmask, default gateway, domain name, IP address(es) of DNS servers, time servers)

-	FQDN or IP address of the initial OAM SeGW

-	FQDN or IP address of the CA/RA servers. If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s).

-	   FQDN or IP address of the initial SCS. If FQDNs need to be resolved, corresponding IP address(es) at the DNS server(s)

The location information should include one or more of the following depending on IAB-node physical location or the mobility scenarios as specified in 3GPP TS 38.401 [x] clause 8.12 and clause 8.23:

-	TAC or TAI pertaining to the cells where IAB-MT is connected.

-	Serving cell ID that IAB-MT is connected to.

-	The gNBId of the IAB-donor-CU and PLMN info that target IAB-DU connects to.

-	The gNBId of the IAB-donor-CU and PLMN info that serves the IAB-MT.

End of change