3GPP TSG-SA5 Meeting #160	S5-251723

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0244 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.531 Add requirements to support solution-agnostic request for service provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson GmbH, Eurolab |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the rationale elaborated in S5-251720 “DP on Solution-agnostic request for service provisioning”, the aim is that network slice provisioning service providers can fulfil the requirements issued by any consumer, no matter whether the consumer understands slice semantics or not (i.e. slice-agnostic consumers). Slice-agnostic consumers expresses service requirements without any slice parameter. This would require changes on requirements captured in clause 5.2.1 and clause 5.2.2 Clause 5.2.1: - Add a new requrirement that expresses the ability of the producer to derive the SST. This ability is needed for when having slice-agnostic consumers, since they do not specify value of “sST” attribute in the request. The SST is needed by the producer to form the “pLMNInfoList” attribute, present in both ServiceProfile and SliceProfile (see change #4 in S5-251720 “DP on Solution-agnostic request for service provisioning”). - Changes to existing requirements, to reflect that what the MnS consumer sends is service requirements rather than network slice related requirements (see change #1 in S5-251720 “DP on Solution-agnostic request for service provisioning”). This will allow the provider to accommodate both slice-agnostic and slice-aware consumers. These service requirements will be fulfilled with a network slice instance, as per 5G network features. Clause 5.2.2: - Changes to existing requirements, to reflect that what the MnS consumer sends is network slicing requirements, rather than network slice subnet related requirements (see change #3 in S5-257120 “DP on Solution-agnostic request for service provisioning”). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the requirements for network slice provisioning service provider: - Add a new requirement (REQ-PRO_NSI_FUN-XX) to express the ability for the producer to derive the SST value, if not provided by the consumer. - Reword FUN-2, NOTE 1 and NOTE 2, to reflect that what the consumer sends is service requirements to be fulfilled with network slicing. - Remove NOTE3, as neither SLA nor serviceprofile are specified in REQ-2. Furtherfore, SLA requirements are outside 3GPP management system domain, and their relationship with GST is not clear. - Minor changes on REQ-PRO_NSI_FUN-01 (allocation wording is missing), REQ-PRO_NSI_FUN-07 (authorized is missing) and REQ-PRO_NSI_FUN-09 (too strict). In the requirements for network slice provisioning service provider, mirror of the above changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.541 CR1524 |  |  |
| Other comments: |  | S5-251720 “DP on Solution-agnostic request for service provisioning” is feeding this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 5.2	Requirements

### 5.2.1	Requirements for network slice provisioning service

REQ-PRO_NSI-FUN-1	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request the allocation of a network slice instance.

REQ-PRO_NSI-FUN-2	The network slice provisioning service provider shall have the capability allowing its authorized consumer to send  communication service related requirements to be fulfilled with a network slice instance.

NOTE 1: Communication service related requirements include requirements such as area traffic capacity, coverage area, isolation/sharing, end-to-end latency, mobility, overall user density, priority, service availability, service reliability, UE speed; see TS 22.261 [5] where these parameters are defined for end user services.

NOTE 2: Communication service related requirements also include requirements derived from the Generic network Slice Template (GST) defined by GSMA in [9].

NOTE 3:  Void

NOTE 4: Void

REQ-PRO_NSI-FUN-3	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request the deallocation of a network slice instance.

REQ-PRO_NSI–FUN-4	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request activation of a network slice instance.

REQ-PRO_NSI–FUN-5	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request deactivation of a network slice instance.

REQ-PRO_NSI-FUN-6	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request the modification of a network slice instance.

REQ-PRO_NSI-FUN-7	The network slice provisioning service provider shall have the capability allowing its authorized consumer to obtain the network slice management data.

REQ-PRO_NSI-FUN-8	The network slice provisioning service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning the requested network slice instance at a particular point of time.

REQ-PRO_NSI-FUN-9	The network slice provisioning service provider shall have the capability targeting to satisfy the consumer’s requested requirements using shared or dedicated managed resources for a network slice instance.

REQ-PRO_NSI-FUN-XX	The network slice provisioning service provider should have the capability to ascertain the SST value that satisfies the consumer’s requested requirements.

Next change

### 5.2.2	Requirements for network slice subnet provisioning service

REQ-PRO_NSSI-FUN-1	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request the allocation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-2	The network slice subnet provisioning service provider shall have the capability of interaction with NFVO via the NS lifecycle management interface.

REQ-PRO_NSSI-FUN-3	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to send network slicing related requirements.

REQ-PRO_NSSI-FUN-4	The network slice subnet provisioning service provider shall have the capability to create a new NSSI or use an existing NSSI based on the consumer’s requested requirements.

REQ-PRO_NSSI-FUN-5	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request to associate the NS instance with corresponding NSSI.

REQ-PRO_NSSI-FUN-6	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request the configuration of the RAN NSSI constituents with the RRM policy information for simultaneous support of multiple NSIs.

REQ-PRO_NSSI-FUN-7	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain network slice subnet instance information.

REQ-PRO_NSSI-FUN-8	The network slice subnet provisioning service provider shall have the capability of allowing its authorized consumer to request the deallocation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-9	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request activation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-10	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request deactivation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-11	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request modification of a network slice subnet instance.

REQ-PRO_NSSI-FUN-12	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain information regarding available network slice subnet resources.

REQ-PRO_NSSI-FUN-13	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning a network slice subnet instance at a particular point of time.

REQ-PRO_NSSI-FUN-14	The network slice subnet provisioning service provider shall have the capability to satisfy the request to consume the NF provisioning service.

REQ-PRO_NSSI-FUN-16	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to provide slice specific operation information for the (re)configuration to a NSSI.

REQ-PRO_NSI-FUN-17	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to assign priority of a network slice subnet.

REQ-PRO_NSSI-FUN-18	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain network slice subnet provider capability information.

REQ-PRO_NSSI-FUN-19	The network slice subnet provisioning service provider shall have the capability to satisfy the consumer’s requested requirements using shared or dedicated managed resources for a network slice subnet instance.