3GPP TSG-SA5 Meeting #160	S5-251797

# Goteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0199 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel19 CR TS28.104 add use case and requirement on management data correlation analytics for threshold assessment and adjustment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use cases of “Measurement data correlation analytics for threshold assessment and adjustment” have been discussed and agreed upon in the study (clause 7.2.8.2 in TS28.104 ). This CR is to normatively propose the solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the solution for the agreed use case and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for the use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.8.1.2.x(new), 8.4.8.1.3, 8.5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS28.104 CR0212 |  |  |
| Other comments: |  | The CR0212 introduced the TS28.111 as a new reference. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251442 |  |  |  |  |  |  |  |  |

Start of First change

###### 8.4.8.1.2.x	Threshold assessment related statistics and predictions

The enabling data for threshold assessment and adjustment are provided in the table 8.4.8.1.2.x-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.8.1.2.x-1: Enabling data for assessment and adjustment related thresholds

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The performance measurements, KPI related to certain performance threshold configurations. 3GPP management system may monitor a set of performance measurements, KPIs and their thresholds, to support the analytics of threshold assessments and recommend threshold adjustments. | Performance metrics include measurements defined in TS 28.552 [4] and KPIs defined in TS 28.554 [5]. For non-3GPP specified measurements the name is defined elsewhere. |
| Alarm notifications | Alarm information, e.g. the alarm notification of a cross threshold of measurement or KPI for a network functions. | Alarm information and notifications as per TS 28.111 [x] |
| Configuration data | MOIs of 5GC NFs, RAN NFs, Network Slice etc. | TS 28.541 [15] |

Next change

##### 8.4.8.1.3	Analytics output

The specific information elements of the analytics output for predictions, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.8.1.3-1.

Table 8.4.8.1.3-1: Analytics output for predictions

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| pmPredictions | This information element defines the predicted values for the below performance measurements. - Mobility related performance measurements listed in table 8.4.5.1.2-1 and 8.4.5.2.2-1 in the current document. - Coverage related performance measurements listed in table 8.4.1.1.2-1 and 8.4.1.2.2-1 in the current document. - SLS related performance measurements listed in table 8.4.2.1.2-1, 8.4.2.2.2-1, 8.4.2.3.2-1, 8.4.2.4.2-1 and 8.4.2.5.2-1 in the current document. - Energy saving related performance measurements listed in table 8.4.4.1.2-1 in the current document. - Maintenance management related performance measurements listed in table 8.4.6.1.2-1 in the current document. | M | type: PmPrediction multiplicity: 1..* isOrdered: False isUnique: True defaultValue: none isNullable: False |
| thresholdAssessment | It indicates a threshold assessment from the management data correlation analysis for the provisioned threshold. allowedValue: N/A | O | type: ThresholdAssessment multiplicity: * isOrdered: False isUnique: True defaultValue: none isNullable: False |
| thresholdAdjustmentRecommendations | It indicates the recommendations of threshold adjustment. allowedValue: N/A | O | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: none isNullable: False |

Next change

### 8.5.x	ThresholdAssessment <<dataType>>

#### 8.5.x.1	Definition

This data type specifies the threshold assessment report.

#### 8.5.x.2	Information elements

Table 8.5.x.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| performanceMetrics | It indicates list of performance metrics with threshold configuration issue. allowedValues: Performance metrics include measurements defined in TS 28.552 [4] and KPIs defined in TS 28.554 [5]. For non-3GPP specified measurements the name is defined elsewhere. | M | type: String multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| timeWindow | It indicates the time window that threshold may not be defined properly. allowedValues: N/A | O | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: N/A isNullable: False |
| confidenceScore | It indicates the confidence of the analysis result. The numerical range is from 0 to 1, with higher values approaching 1 indicating greater confidence. allowedValues: numerical range is from 0 to 1 | O | type: Float multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |

End of changes