3GPP TSG-SA5 Meeting #160	S5-251836

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0616 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend Data Type for 5G LCS converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no Data Type for 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend Data Type to support 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G LCS converged charging will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.19, 6.1.6.3.4, 6.1.8, 7.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.271 CR 0034 TS 32.298 CR 1034 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.6.2.19	5GS LCS Specified Data Type

6.1.6.2.19.1	Type ChargingDataRequest

This clause specifies additional attributes of the type ChargingDataRequest defined in clause 6.2a.1.2.1 5GS LCS charging described in 3GPP TS 32.271 [38].

Table 6.1.6.2.19.1-1: 5GS LCS Specified attribute of type ChargingDataRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| rangingSLChargingInformation | RangingSLChargingInformation | OC | 0..1 | This field holds the Ranging and Sidelink Positioning specific information. | RangingSL |
| lCSInformation | LCSInformation | OC | 0..1 | This field holds the 5GS LCS specific information. | LCS |

6.1.6.2.19.2	Type ChargingDataResponse

This clause specifies additional attributes of the type ChargingDataResponse defined in clause 6.2a.1.2.2 5GS LCS charging described in 3GPP TS 32.271 [38].

Table 6.1.6.2.19.2-1: 5GS LCS specified attribute of type ChargingDataResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

6.1.6.2.19.3	Type RangingSLChargingInformation

Table  6.1.6.2.19.3-1: Definition of type RangingSLChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| targetUEID | Supi | OC | 0..1 | The identity of Target UE in Ranging/Sidelink positioning |  |
| sLReferenceUEID | Supi | OC | 0..1 | The identity of SL Reference UE in Ranging/Sidelink positioning |  |
| sLPositioningServerUEID | Supi | OC | 0..1 | The identity of SL Positioning Server UE in Ranging/Sidelink positioning |  |
| locatedUEID | Supi | OC | 0..1 | The identity of Located UE in Ranging/Sidelink positioning |  |
| locationType | LocationType | OC | 0..1 | This field holds the type of location information being requested. |  |
| locationEstimate | LocationEstimate | OC | 0..1 | This field denotes the location of a Target UE and the requested accuracy of the estimate. |  |

6.1.6.2.19.4	Type LocationEstimate

Table 6.1.6.2.19.4-1: Definition of type LocationEstimate

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| userLocationInformation | UserLocation | OC | 0..1 | provides information on the location |  |
| horizontalAccuracy | OctetString | OC | 0..1 | This field indicates the required horizontal accuracy of the location estimate described in TS 29.002 [261]. |  |
| verticalAccuracy | OctetString | OC | 0..1 | This field indicates the required vertical accuracy of the location estimate described in TS 29.002 [261]. |  |

6.1.6.2.19.x	Type LCSInformation

Table  6.1.6.2.19.x-1: Definition of type LCSInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lCSClientID | String | OC | 0..1 | This field holds the ID of the LCS client that invoked the LR, if available. |  |
| locationType | LocationType | OC | 0..1 | This field holds the type of location information being requested. |  |
| locationEstimate | LocationEstimate | OC | 0..1 | This field denotes the location of a Target UE and the requested accuracy of the estimate. |  |
| positioningData | String | OC | 0..1 | This field indicates the positioning method that was attempted to determine the location estimate for MO-LR, if available. |  |
| targetUEID | Supi | OC | 0..1 | The identity of Target UE in LCS |  |

Next change

6.1.6.3.4	Enumeration: NodeFunctionality

Table 6.1.6.3.4-1: Enumeration NodeFunctionality

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SMF | This field identifies that NF is a SMF. |  |
| AMF | This field identifies that NF is a AMF. |  |
| SMSF | This field identifies that NF service consumer is a SMSF. |  |
| PGW_C_SMF | This field identifies that NF is a SMF+PGW-C. |  |
| NEF | This field identifies that NF is a NEF. |  |
| SGW | This field identifies that node is an SGW, only applicable for interworking with EPC. |  |
| I_SMF | This field identifies that node is an I-SMF, only applicable for PDU session served by SMF + I-SMF. | ETSUN |
| ePDG | This field identifies that node is an ePDG, only applicable for interworking with EPC/ePDG. | 5GIEPC_CH |
| CEF | This field identifies that NF is a CEF. |  |
| MnS_Producer | This field identifies that NF is a MnS Producer |  |
| SGSN | This field identifies that node is an SGSN, only applicable when SMF+PGW-C serves GERAN/UTRAN access. | TEI17_NIESGU |
| V_SMF | This field identifies that node is a V-SMF, may be used instead of SMF in roaming scenarios. |  |
| 5G_DDNMF | This field identifies that NF is a 5G DDNMF | 5G ProSe |
| IMS_Node | This field identifies that NF is an IMS Node. A further breakdown of IMS Node type may be available in IMS Charging Information | IMS |
| EES | This field identifies that NF is an EES. | EdgeComputing |
| PCF | This field identifies that NF is PCF. Only applicable for API Target Network Function |  |
| UDM | This field identifies that NF is UDM. Only applicable for API Target Network Function |  |
| UPF | This field identifies that NF is UPF. Only applicable for API Target Network Function |  |
| TSN AF | This field identifies that NF is a TSN AF. | TSN |
| TSCTSF | This field identifies that NF is a TSCTSF. | TSN |
| MB_SMF | This field identifies that NF is a MB-SMF. |  |
| CHF | This field identifies that NF is a CHF. | INTER_CHF |
| GMLC | This field identifies that NF is a GMLC. | RangingSL, LCS |

Next change

6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nchf_ConvergedCharging API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [299].

Table 6.1.8-1: Supported Features

| Feature number |  | Feature Name |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | CHFCQM |  | CHF-controlled quota management i.e. support for temporary offline. |  |
| 2 |  | AF_Charging_Identifier |  | Indicates the support of long character strings as charging identifiers. |  |
| 3 |  | 5GIEPC_CH |  | 5GS interworking with EPC. |  |
| 4 |  | ATSSS |  | This feature indicates support of Access Traffic Steering, Switching, Splitting (ATSSS). |  |
| 5 |  | ETSUN |  | This feature indicates support of Enhancing Topology of SMF and UPF in 5G Networks (ETSUN). |  |
| 6 |  | EnhancedDiagnostics |  | Support the enhanced diagnostics. |  |
| 7 |  | AMF_subs_PRA |  | PRA(s) subscription by CHF in AMF. |  |
| 8 |  | FilterRuleList |  | Support of multiple filter rules in the final unit indication. |  |
| 9 |  | TEI17_NIESGU |  | This feature indicates support of GERAN/UTRAN access. |  |
| 10 |  | IMS |  | This feature indicates support of IMS. |  |
| 11 |  | QoSMonitoring |  | This feature indicates support of QoS Monitoring. |  |
| 12 |  | Announcement |  | This feature indicates support of announcements. |  |
| 13 |  | 5GLAN |  | This feature indicates support of 5G LAN-type services. |  |
| 14 |  | URLLC |  | This feature indicates support of URLLC. |  |
| 15 |  | NotifyInfoResponse |  | This feature indicates support of response with information for a notification. |  |
| 16 |  | ES4xx |  | Extended Support of HTTP 400, 403, 404 allowing use of either ChargingDataResponse or ProblemDetails in the response. |  |
| 17 |  | ES3xx |  | Extended Support of HTTP 307 and 308 redirections, an NF that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15 and 16. |  |
| 18 |  | EdgeComputing |  | This feature indicates support of edge computing domain charging. |  |
| 19 |  | 5GSCIoT |  | This feature indicates support of 5GS control plane CIoT optimization. |  |
| 20 |  | SMF_Charging_Id |  | Indicates the support of strings as SMF charging identifiers. |  |
| 21 |  | SNPN |  | This feature indicates support of Stand-alone Non-Public Network. |  |
| 22 |  | IDC_CH |  | This feature indicates support of IMS Data Channel charging. |  |
| 23 |  | 5MBS_CH |  | This feature indicates 5G multicast-broadcast services charging. |  |
| 24 |  | SatelliteAccess |  | This feature indicates support of NR satellite access. |  |
| 25 |  | NSREP |  | This feature indicates support of Network slice replacement charging. |  |
| 26 |  | TSN |  | This feature indicates support of time sensitive networking. |  |
| 27 |  | 5GSATB |  | This feature indicates support of satellite backhaul. |  |
| 28 |  | NSAC_CH |  | This feature indicates support of Network slice admission control charging |  |
| 29 |  | NSSAA |  | This feature indicates support of Network slice-specific authentication and authorization charging |  |
| 30 |  | ProSe |  | This feature indicates support of 5G ProSe. |  |
| 31 |  | INTER_CHF |  | This feature indicates support of inter-CHF communication. |  |
| 32 |  | RangingSL |  | This feature indicates support of Ranging and Sidelink Positioning. |  |
| 33 |  | EE_NS_CH |  | This feature indicates support of energy information for network slice. |  |
| 34 |  | LCS |  | This feature indicates support of 5GS LCS. |  |  |

Next change

7.17	Bindings for 5GS LCS charging

Table 7.17-1: Bindings of CDR field, Information Element and Resource Attribute for 5GS LCS charging

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
|  |  | ChargingDataRequest |
| Ranging and Sidelink Positioning Charging Information | Ranging and Sidelink Positioning Charging Information | /rangingSLChargingInformation |
| Target UE ID | Target UE ID | /rangingSLChargingInformation/targetUeId |
| SL Reference UE ID | SL Reference UE ID | /rangingSLChargingInformation/sLReferenceUeID |
| SL Positioning Server UE ID | SL Positioning Server UE ID | /rangingSLChargingInformation/sLPositioningServerUeId |
| Located UE ID | Located UE ID | /rangingSLChargingInformation/locatedUeId |
| Location Type | Location Type | /rangingSLChargingInformation/locationType |
| Location Estimate | Location Estimate | /rangingSLChargingInformation/locationEstimate |
| LCS Information | LCS Information | /lCSInformation |
| LCS Client ID | LCS Client ID | /lCSInformation/lCSClientId |
| Location Type | Location Type | /lCSInformation/locationType |
| Location Estimate | Location Estimate | /lCSInformation/locationEstimate |
| Positioning Data | Positioning Data | /lCSInformation/positioningData |
| Target UE ID | Target UE ID | /lCSInformation/targetUeId |
|  |  | ChargingDataResponse |
| - | - | - |

End of changes