3GPP TSG SA WG5 Meeting #160	S5-251904

Goteborg, SWEDEN 7 - 11 April 2025

# Source:	Samsung, Nokia, China Unicom, DTAG

# Title:	Triggered CCL UC and Requirements

# Document for:	Approval

Agenda Item:	6.19.4.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

# 4	Detailed proposal

First Change

## 5.x		Triggered CCL Capability

### 5.x.1						Description

#### 5.x.2		Use Cases

5.x.2.a						CCL instantiation based on conditions

The MnS consumer may want to request for a CCL to be instantiated not immediately but when certain conditions are met. For example, the MnS consumer may want that for a CCL of a stated type or that matches a set of stated characteristics (e.g. goal) to be instantiated under certain conditions and another with variations in goals to be instantiated under other conditions. The MnS consumer should be enabled to define those conditions so that the CCL is instantiated when the stated conditions are met. The MnS Producer monitors the conditions to check if they are met.

The conditions can be related to events based on management data (e.g., performance, fault, configuration).

Performance events are defined related with performance measurements and KPIs that need to be monitored by the producer to see if an CCL is to be initiated. For example, if the value of a particular performance measurement goes beyond a particular value, a CCL should be instantiated to keep the value of the same performance measurement below a defined value.

Fault events are defined by related information (e.g alarm type, alarm severity) that need to be monitored by the producer to see if a CCL is to be initiated. For example, if the total number of alarm with type QUALITY_OF_SERVICE_ALARM and perceivedSeverity MAJOR goes beyond a particular value, a CCL should be instantiated.

Provisioning events (e.g CreateMOI) are defined that need to be subscribed by the producer to see if a CCL is to be instantiated. For example, the creation on an Intent MOI can be a trigger to instantiate a CCL. For another example, when a pre-defined system time specified by operators can be a trigger to instantiated a CCL.

Note: the use case requires to set the conditions and then the conditions need to be continuously monitored and tracked.

#### 5.x.2.b					CCL action execution based on conditions

For the CCLs that have been instantiated, the MnS consumer may want to define conditions under which a CCL may execute actions on the network, e.g. when the performance on a certain threshold is crossed, or when the confidence on the decision is above a stated threshold. The consumer does not need to be aware of all decisions, but by providing conditions under which decisions may be activated or not, it is able to have supervision over the CCL without having to continuously track the decisions. The MnS consumer should be enabled to define those conditions for executing the CCL actions. Otherwise, the consumer should be enabled to define alternative actions, e.g. to notify the consumer of the decision that is not executed.

By supporting this, the execution can be affected by producer based on consumer's conditions or requirements. To ensure oversight or accountability by the MnS consumer, the MnS consumer may be notified by the CCL about the executed action for any conditionally execution. The MnS consumer can intervene as needed.

Note: the use case requires to set the conditions and then the conditions need to be continuously monitored and tracked.

### 5.x.3						Requirements

Table 5.x.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-TRI-FUN-01 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger CCL instantiation. | Conditional trigger of a CCL |
| REQ-TRI-FUN-02 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger CCL update. | Conditional trigger of a CCL |
| REQ-TRI-FUN-03 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data that can be monitored and used to trigger CCL deletion. | Conditional trigger of a CCL |
| REQ-TRI-FUN-04 | The 3GPP management system should enable authorized consumers to define conditions related to performance, fault and configuration data under which a CCL may execute its actions. | Conditional execution of CCLs network changes |

End of Change