3GPP TSG-SA5 Meeting #160	S5-251909

Goteborg, Sweden, 07 - 11 April 2025 	Revision of S5-251252

Source:	Nokia

Title:	Pseudo-CR on informative annex on CCL conflict handling approach

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add an informative description for the options on CCL conflict handling

Proposed Changes

* * * First Change * * * *

Annex D (informative):
Selection of conflict coordination approach

# D.1	Alternative CCL coordination Approaches for conflicts handling

To address the conflicts, coordination interactions are required either among CCLs or between the CCLs and one or more higher hierarchy coordination functions to avoid or detect and resolve the conflicts. This is required when CCL are actuating in the same set of resources.

The coordination of CCLs could be accomplished via one of three approaches illustrated by Figure D.1-1:

-	Distributed coordination with distributed execution (Figure D.1-1 a), where the CCLs directly coordinate with one another, and each manages execution of its decisions. The CCL exchange information with each other avoid, detect or resolve conflicts. The information may for example include notifications of executed actions or observed impacts.

-	Hierarchical coordination with distributed execution (Figure D.1-1 b), where the CCLs coordinate through a separate coordination layer, say via a CCL coordination entity, but each manages execution of its coordinated decisions. The CCL exchange information with the CCL coordination entity to avoid, detect or resolve conflicts. A CCL may send notifications of its executed actions or observed impacts which the CCL coordination entity may relay to other CCLs. The CCL coordination entity may configure the CCLs but each CCL executes its action based the CCL coordination entity’s configuration.

-	Hierarchical coordination and execution (Figure D.1-1 c), where the CCLs coordinate through a separate coordination layer, say via a coordination entity that besides coordination also manages execution of the coordinated decisions. The CCL exchange information with the CCL coordination entity to avoid, detect or resolve conflicts including notifications of their executed actions or observed impacts which the CCL coordination entity may relay to other CCLs. The CCL coordination entity may configure the CCLs and the CCL execute their actions through the CCL coordination entity.

Figure D.1-1: Closed Control Loop Coordination approaches

Distributed coordination can lead to too many exchanges between the CCLs which may unnecessarily clog the system. On the other hand, "Hierarchical coordination and execution" implies that too much responsibility is concentrated in a single CCL. A desired behavior is that the individual CCLs are responsible for their own decision execution, so it is recommended that to follow the "hierarchical coordination with distributed execution" approach. In this approach, the CCLs are responsible for making their decisions and executing actions, but they coordinate with the CCL coordinator before, during or after execution.

# D.2	Hierarchical CCL-coordination-interactions for conflicts handling

To address the conflicts, coordination interactions are required between the CCLs and one or more higher hierarchy coordination functions to avoid or detect and resolve the conflicts among goals and their targets, control scopes or actions of the CCLs. The 3GPP management system includes at least one entity called the Coordination entity that undertakes the role of CCL coordination. The Coordination entity can be implemented as a CCL, an AIML inference engine or any other functionality that is found appropriate. The coordination entity may support coordination for conflict management for different conflicts described in clause 5.A.2 including Goal- and Goal-targets- Conflicts; scope conflicts, CCL-Trigger-time and CCL-action-execution-time conflicts, Direct actions conflicts as well as indirect targets conflicts.

The coordination of CCLs could be required at different execution points of the CCL translating into different CCL coordination use cases with corresponding CCL coordination services required at those points as illustrated by example Figure D.2-1. The coordination of CCLs could be achieved via direct interaction among the CCLs or via a third-party entity, say called the CCLs coordination Function (or simply CCL Coordinator).

Figure D.2-1: Exemplary Closed Control Loop Coordination interaction points

NOTE:	The terms at the top indicate general naming of the groupings of coordination interactions at the different execution points during the execution of the CCL. Action-space coordination implies coordinating the sets of actions that the different CCL can apply. Concurrency control implies coordinating the times at which different CCLs can execute actions. Action-impact assessment indicates interactions and processes on the evaluation of the impacts of the different CCLs.

The coordination purpose attributes contain the information and data needed or used by the coordination entity for interacting with the CCL when handling conflicts.

* * * End of Changes * * * *