3GPP TSG-SA5 Meeting #160	S5-251915

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0332 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.312 Clarifications on negotiation of intent fulfillment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Canada Inc., ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some clarifications to the intent negotiation procedures are required. Some requirements can be removed (e.g. duplicates) and others combined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications to the intent negotiation text. Added paragraph in Introduction to clarify that intent negotiation can be triggered by MnS Consumers. Req-07, Req-08 and Req-09 are clarified. Req-13 and Req-15 are combined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficult to understand the negotiation procedures and their relationship to other content in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.9.1, 5.3.9.2, 5.3.9.3, 5.3.9.4, 5.3.9.5, 5.3.9.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.3.9	Negotiation on fulfilment of intents

#### 5.3.9.1	Introduction

For a given intent, an MnS Consumer may express a feasible intent, but the producer may have multiple ways to fulfil it. One of these solutions might be better in one aspect and another solution might be better in another aspect. Although this intent is feasible, the producer may not have the knowledge and capability to determine which solution is preferable from the MnS Consumer´s point of view.

Example, the MnS Consumer may express intent for energy saving but only specify the target for energy consumption reduction without more constraints, e.g. reduce 20 % energy consumption. There will be numerous solutions to fulfil the energy consumption target with various impacts, e.g. one solution may reduce the coverage and one may reduce the number of users.

The MnS consumer may have concerns on the possible impact of the selected solutions chosen by the MnS Producer, which means that negotiation with the MnS Consumer is necessary. In such cases, the MnS Consumer and MnS Producer may negotiate on the best way to fulfil the intent.

Intent negotiation is a set of procedures which are applicable throughout the life cycle of an intent.  Several negotiations are possible for a feasible intent, many employing interactions that are similar.

NOTE 1:	Some of these negotiations can also be applicable during the feasibility check proces.

Intent negotiation procedures can be initiated by MnS Consumers. Consequently, if an MnS Consumer does not support intent negotiation capabilities, it does not trigger such procedures.

2nd Change

#### 5.3.9.2	Checking for fulfillable outcomes

The MnS consumer wants to know possible fulfillable outcomes for a given intent. The MnS consumer creates an intent MOI that should be evaluated by the MnS producer to see options which the MnS producer can deliver.

Figure 5.3.9.2-1: MnS consumer requests and receives a list of possible fulfillable outcomes on an intent.

Many dynamic factors impact possible outcomes.  As a result, it is unrealistic to expect all potential outcomes to be reported.   The list of possible outcomes an MnS Producer may provide is left to implementation.

Subsequently, the MnS producer provides a report indicating what is fulfillable for each intent aspect (intentExpectation and expectationTarget) within that intent. Since different possible fulfillable outcomes may have different impacts, the report should include the relative impacts of each outcome. The relative impact is defined as the MnS Producer´s computed impact to existing expectations of fulfilling one or more expectations within the requested intent on the expectations within another intent from the same MnS Consumer. For example, an energy intent expectation can be fulfilled causing a degradation on the coverage intent expectation. Relative impact from different MnS Consumers shall not be reported since different MnS Consumers are not to be aware of each others intents.

NOTE 1:  the mechanism by which MnS Consumer can indicate a preference for a specific possible outcome is out of scope of this TS.

NOTE 2: How the Producer would report “the relative impacts of each outcome” is FFS

3rd Change

#### 5.3.9.3	Checking for best possible outcome on an intent, intent expectation, or expectation target

The MnS consumer wants to know the best possible outcome for a given intent or intent expectation or expectation target. This could be prior to or during fulfilment.

Figure 5.3.9.3-1: Checking for best possible outcome on intent or
intent expectation or expectation target

The MnS consumer creates an intent that should be evaluated by the MnS producer with a request to provide the best possible outcome. Subsequently, the MnS producer provides a report indicating that best possible outcome. The best possible outcome is defined in 2 possible scenarios as follows:

-	The request is to evaluate specific expectation target(s) in an intent with multiple expectation targets (e.g. multiple expectations or one expectation with multiple expectation targets): The best possible outcome is the best value on specific expectation target(s) that maintains the other expectation targets to within the ranges specified in the intent.

-	The request is to evaluate all expectation targets in an intent with multiple expectation targets (e.g. multiple expectations or one expectation with multiple expectation targets): The best possible outcome is the best value on each expectation target that maintains the other expectation targets to within the ranges specified in the intent.

The MnS producer should support a possible outcomes report that lists the possible outcomes for any of the two scenarios, the report including the relative impact on other targets in the intent or on other metrics and contexts.

4th Change

#### 5.3.9.5	MnS consumer advises on preferred alternatives

The MnS consumer wants an intent to be fulfilled. The intent is feasible, but the MnS producer has multiple alternatives related to fulfilling the intent. The MnS producer wants the MnS consumer to advise on their (the MnS consumer's) preference among these alternatives.

NOTE 1:	An alternative is the combination a set of expectation target values that the MnS producer can achieve together with their (expected) impacts on the network (objects). E.g. for an expectation target on energy consumption, the impact may include coverage area as a result of the energy consumption.

Figure 5.3.9.5-1: MnS consumer advises the MnS producer
on the preferences among alternatives at the MnS producer

After the MnS consumer creates an intent to be fulfilled, the MnS producer determines that there are multiple alternatives, so the MnS producer provides a report to the MnS consumer so that the MnS consumer may help chose the best alternative.

The report to the MnS consumer may include:

-	The list of available/fulfillable expectation target values that the MnS producer is able to apply/achieve.

-	The expected relative impacts of the different alternatives - on aspects of the submitted intent or their  intents and intent expectations.

-	A request to the MnS consumer to select one among the alternatives.

Given the alternatives, the MnS consumer takes any combination of these actions:

-	Chooses and indicates the preferred alternative.

-	Defines the relative importance of their expectation Targets (in the form of a utility function) so that the MnS producer may consider these in deciding upon the solution/ solution approach/ closed loops/ action/ outcome to be applied/deployed/achieved.

-	Provides the relative satisfaction level that each of the alternatives achieves. The satisfaction level is the computation of the utility achieved by each alternative according to the MnS consumer’s utility function. It is an integer in the range  [0,100] . The highest possible value indicates that the solution provided by the MnS producer achieves the best possible outcomes that the MnS consumer expected, e.g., that it achieves the highest range of a target whose desired values were defining as falling in a range. Correspondingly, the values lower than 100 indicate how far from the maximum satisfaction the outcome is.

5th Change

#### 5.3.9.6	Requirements

REQ-Intent_NEGOT_01: The intent-driven MnS producer should have the capability to provide possible outcomes to the MnS Consumer.

REQ-Intent_NEGOT_02: The intent-driven MnS producer should have the capability enabling MnS Consumer to  decide appropriate outcome for fulfilment based on possible outcomes.

NOTE 1:	The definition for outcome is For Further Study.

REQ_INT_NEGOT-03: The intent driven MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide information on the alternative fulfillable outcomes for an intent.

NOTE 2:	An alternative is the combination of a set of expectation target values that the MnS producer achieves together with their (expected) impacts.

NOTE 3:	The impacts refer to information about the changes and outcomes on the expectation objects of the intent and on other related intents from the same intent MnS consumer. The impact refers to outcomes of the contexts and targets that MnS consumers have not explicitly pointed out. The exact characterization of what is reported is For Further Study. E.g. for an expectation target on energy consumption, the impact may include coverage area as a result of the energy consumption.

NOTE 4:	Example changes could include: omitting certain intentExpectations and/or expectationTarget(s) or changing the properties of intentExpectations and/or expectationTarget(s).

REQ_INT_NEGOT-04: The intent driven MnS producer should support a capability to provide to an MnS consumer an intent report indicating the alternatives that the MnS producer supports for the provided intent, intent expectations, or expectation Targets and the expected relative impacts of the different alternatives.

REQ_INT_NEGOT-05: The intent driven MnS producer should support a capability to request an MnS consumer to indicate its preference among a set of alternatives that the MnS producer supports for the provided intent, intent expectations, or expectation Targets.

REQ_INT_NEGOT-06: The intent driven MnS producer should support a capability enabling an authorized MnS consumer to provide information on a description of its preferences (e.g. in form of a utility function or satisfaction index) that should be used by the MnS producer to select among the alternatives available at the MnS producer.

REQ_INT_NEGOT-07: The intent driven MnS producer should support a capability to request the MS consumer to provide an evaluation of the selected alternative among the MnS producer's alternatives based on the expected relative impacts of the different alternatives.

REQ_INT_NEGOT-08: The intent driven MnS producer should support a capability enabling an MnS consumer to provide an evaluation of the MnS producer's alternatives.

REQ_INT_NEGOT-09: The intent driven MnS producer should support a capability to inform an authorized MnS consumer that their selected alternative has been applied

REQ_INT_NEGOT-10: The intent driven MnS producer should support a capability to inform an authorized MnS consumer that no more improvement to intent fulfillment is possible with the applied alternative and request the MnS consumer to provide extra information which helps improve satisfaction.

REQ_INT_NEGOT-11: The intent driven MnS producer should support a capability enabling an authorized consumer to inform the producer that the alternative selected by the MnS producer was not satisfactory and another alternative should be applied.

REQ_INT_NEGOT-12: The intent driven MnS producer should support a capability enabling an authorized MnS consumer to provide information on the degree of satisfaction which the MnS producer uses to differently attempt the fulfillment.

INT_NEGOT_REQ 13: The MnS producer should support a capability to provide an intent report including information on what is the best possible outcome for each intentExpectation or expectationTarget within that intent and the relative impact of achieving that outcome.

INT_NEGOT_REQ 14: The MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide the best possible outcome for selected intent expectations and expectation targets, while keeping other intent expectations unmodified.

INT_NEGOT_REQ-15: The MnS producer should support a capability enabling an MnS consumer to provide an intent with a request for the MnS producer to provide information on what changes could be made to the intent properties or to properties of other of their intents to make the intent fulfillable.

NOTE 6: Further discussion is needed for this requirement.

INT_NEGOT_REQ-16: The MnS producer may support a capability to provide a report indicating the changes, which if applied to the intent, would make the intent fulfillable

NOTE 7: The definition of outcomes is to be clarified

INT_NEGOT_REQ-17: The MnS producer may support a capability to provide implicit intent, i.e., the additional intent expectation target and contexts, which includes the information that MnS consumers have not explicitly pointed out in its intent.