3GPP TSG-SA5 Meeting #160 	S5-251961

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0674 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Cell Unavailable Time performance measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Cell Unavailable performance measurement needs to be defined in NRCellDU based on state attributes (Administrative, operational) aligning it with E-UTRAN spec approach |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a Cell Unavailable performance measurement in NRCellDU |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lacking of performance measurement for Cell Unavailable in NRCellDU |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.4.1.X 	Cell Unavailable Time

a)	This measurement provides the length of time during which the NRCellDU part of the cell is unavailable to provide service to UEs within the granularityPeriod (as defined in 3GPP TS 28.622 [65]). It is optional to perform this measurement for each cause.

b)	CC

c)	This measurement is obtained by accumulating the number of seconds for which the cell is unavailable, or optionally per cause. The cumulative value shall be equal the total time period of cell unavailability. Possible causes are:

- "manual intervention": when the administrativeState is LOCKED.

- "automatic intervention": when the administrativeState is UNLOCKED and operationalState is DISABLED.

- any vendor specified causes.

d)	Each measurement is an integer value (in seconds).

e)	OEU.CellUnavailableTime, or
OEU.CellUnavailableTime.Cause, where Cause identifies the cause that resulted in cell being unavailable, it has enumerated value for "manual intervention”, "automatic intervention", or any vendor specified cause.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	This measurement may be relevant in the context of monitoring for network and service operations for energy utilities as described in appendix A.118

End of changes