3GPP TSG-SA5 Meeting #160	S5-252024

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0037 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28540 Add NTN regenerative concepts and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NTN_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new concepts/background and requirements for management of NTN regenerative mode which is defined in RAN TS 38.300 and studied in SA5 TR 28.874. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new concepts/background and requirements for management of NTN regenerative mode |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing key requirements for NTN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.10.X (new), 5.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 4.10.2	Management of Backhaul feature

The AMF report the satellite backhaul category and indicates the satellite backhaul category change to SMF as outlined in TS 23.501 [2]. And the AMF can determine the Satellite backhaul category based on the information within Global RAN Node IDs associated with satellite backhaul. The 3GPP management system should provide a capability to allow the MnS consumer to configure satellite backhaul information for NTN node.

### 4.10.X	Management of regenerative mode feature

Fig. 4.10.X-1 illustrates an NTN system with regenerative mode, where the gNB is located onboard the satellite. In this case, the ground segment Core Network (CN) will serve the same beams all the time, while the space segment gNB on different satellites (satellite 1, 2 and 3) will serve the beam in different time periods as the satellites are approaching and leaving the coverage of the beam over time.

Figure 4.10.X-1:		Non-geosynchronous satellites in NTN with regenerative gNB processed satellite payload

As defined in TS 38.300 [3], with Non-Geosynchronous orbit (NGSO) satellites, the gNB can provide either quasi-Earth-fixed cell coverage or Earth-moving cell coverage.

One consequence of non-geosynchronous satellites is that the associations between the entities on ground segment and entities in space segment are changing frequently, typically with a period of one to several minutes.

Another issue is the topology between space segment Managed Element (MnS producer) and the ground based Management System (MnS consumer): With long distances in between, disturbances (e.g. bad weather conditions), and partial reachability issues (when satellites fly over oceans with no gateway coverage), the latency, availability and reliability of the interface between them (feeder link + Inter-satellite link) are impacted.

Summary:

For the deployment scenario of RAN nodes on-board satellites, this would result in the following scenario: a LEO or MEO satellite with an onboard RAN node leaves the coverage area of a CN and then returns to the coverage area of that CN after cycling around the earth one or several times seen from the operator's perspective, it is necessary to investigate how to efficiently manage 1) The cell configuration of satellite gNBs when quasi-Earth-fixed cell coverage is applied, as the moving satellite gNB is serving different coverage areas over time. 2) The connections between RAN nodes and CN to avoid errors in CN due to stale connections, e.g. AMF sending paging requests or AMF configuration updates to an unavailable RAN node. For example, 3GPP management system configures AMF and/or gNB to add necessary information to support their awareness of when connectivity between a RAN node and a CN NF is available or unavailable.

2nd Change

### 5.9.4	Management of regenerative mode feature

REQ-NTN_REGEN-001: The NR NRM definitions shall have the capability to allow the MnS consumer to configure NTN neighbour cells considering the satellite movement.

REQ-NTN_REGEN-002: The 3GPP Management System should have the capability to allow the configuration of  the connections between RAN nodes on-board satellite and 5GC considering that the availability of the management interface to the RAN nodes may be intermittent.

REQ-NTN_REGEN-003: The 3GPP Management System should have the capability to allow the configuration of the (quasi-Earth-fixed) cell configurations of RAN nodes where, due to satellite movement, the configuration requires continuous update while considering that the availability of the management interface to the RAN nodes may be intermittent.

End of Changes