3GPP TSG-SA5 Meeting #161	S5-252519

Fukuoka, Japan, 19 - 23 May 2025

# Source:	Moderator (Huawei)

# Title:	New WID on Study on Management Data Analytics (MDA) – Phase 4

# Document for:	Approval

# Agenda Item:	5.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Study on Management Data Analytics (MDA) – Phase 4

Acronym:	FS_eMDAS_Ph4

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_MDAS | SA5 | 850028 | Study on enhancement of Management Data Analytics Service |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940078 | Rel-17 SA5: Enhancements of Management Data Analytics Service (eMDAS) | Earlier phase of the MDA specifications |
| 970031 | Rel18 SA5: Management Data Analytics phase 2 (eMDAS_Ph2) | Earlier phase of the MDA specifications |
| 1060014 | Rel19 SA5: Management Data Analytics phase 3 (eMDAS_Ph3) | Earlier phase of the MDA specifications |

# 3	Justification

As the field of network management evolves towards programmable and autonomous networking, new opportunities are opening up to enhance the usefulness of the 3GPP Management Data Analytics Service (MDAS).

The study for phase 4 will focus on investigating the following topics for potential improvements to MDAS:

Improvements to interoperability

In order to support business-level insights and automation, it is important that external consumers and stakeholders can effectively consume the MDAS capabilities. To ensure that MDAS capabilities can be consumed in a multi-vendor ecosystem, the MDAS interfaces must be defined with no ambiguity. Some issues combine to make it difficult to design and implement external consumers which can consume the MDAS capabilities. For example, it is unclear which input parameters are valid to consume each MDAS capability. Also, some MDAS outputs are not clearly defined and are expressed simply as strings. The stage 3 specification is overly flexible, and therefore a deep understanding of the stage 2 specification is necessary to create and test a stage 3 solution.

Improvements to MDA framework

To provide an analysis of the status/performance of a distributed multi-technology network, MDAS should be able to consume and build value upon 3GPP analytics providers (e.g. NWDAF, domain-specific MDAS), other non-3GPP analytics providers, and AI/ML functions. However, the MDAS framework has not been updated to reflect recent changes in the surrounding ecosystem, for example in non-3GPP analytics providers. The MDAS framework does not clearly describe the relationship between MDAS and AI/ML functions, and this needs to be updated and elaborated. The MDAS framework does not describe the relationship between MDAS and Network Digital Twin, and this needs to be added. Currently, the enabling data for MDAS analytics is limited to 3GPP-specified data sources, this may need to be expanded.

New and enhanced analytics capabilities

Currently, many of the MDAS capabilities focus on predicting the future status of the network. But to support the trend towards automation, MDAS should add capabilities to analyse the current status/performance/usage of the network to enable more responsive management. New capabilities are needed in proactive network performance/fault monitoring to allow problems to be detected, diagnosed and resolved as quickly as possible. To support automated decision-making, MDAS needs to analyse the current status of the network and recommend optimization or repair actions.

To ensure the safety of automated decision-making, MDAS needs to analyse potential impact of proposed changes.

Currently, analytics focuses on the status and performance of the managed network, more intelligent data insights are needed to analyse how the network is being used, thus allowing real-time optimization of KPIs for network resource utilization and quality of service delivery.

Currently, there is limited ability for a consumer to influence the behaviour of an MDAS producer, for example a consumer can specify that the MDAS producer should not recommend changes to specific managed objects. But there are other situations where MDAS analytics or MDAS recommendations may be irrelevant for the consumer, and therefore represent wasted effort. To make MDAS reports more relevant for the consumer, improvements are needed to allow a consumer to specify behavioural requirements, for example whether selected analytics should have higher/lower precision, or how to limit how aggressively MDAS should recommend optimizations or resolutions.

# 4	Objective

The objectives of the study on Management Data Analytics (MDA), Phase 4 is to investigate and discuss use cases along with the corresponding potential requirements and solutions for the following topics:

WT-1 Improvements to interoperability

WT-1.1 Study challenges for MDAS consumers to understand how to consume MDAS capabilities. Specification may need to be improved or examples added to assist consumers to discover the available MDAS capabilities and to invoke each MDAS capability.

WT-1.2 Study challenges for interoperability in a multi-vendor environment. Specification may need to be improved to clarify ambiguous parameters and generic schemas.

WT-2 Improvements to MDA framework

WT-2.1 Study possible updates to MDAS framework to describe potential MDAS interactions with 3GPP-defined entities. This may include, but not limited to, NWDAF, domain-specific MDAS, AI/ML, Network Digital Twin.

WT-2.2 Study possible updates to MDAS framework to describe potential MDAS interactions with non-3GPP entities.

WT-3 New and enhanced analytics capabilities

WT-3.1 Study possible new MDAS capabilities to provide analytics on the current status/performance/usage of the managed network.

WT-3.2 Study possible new MDAS capabilities to help detect, diagnose and resolve problems in the managed network.

WT-3.3 Study possible new MDAS capabilities to evaluate the possible impact of proposed changes to the managed network.

WT-3.4 Study possible new MDAS capabilities to analyse network resource utilization and quality of service delivery.

WT-3.5 Study enhancements MDAS capabilities to allow MDAS consumers to influence the MDAS analytics and recommendations to align with the needs of the MDAS consumer.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | SA Dependency (Yes/No/Maybe) |
| --- | --- | --- | --- | --- |
| WT-1 | 1.0 | 1.5 | No | No |
| WT-2 | 0.5 | 0.5 | No | No |
| WT-3 | 1.5 | 2.5 | No | No |

Total TU estimates for the study phase: 3.0

Total TU estimates for the normative phase: 4.5

Total TU estimates: 7.5

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 28.8xx | Study on Management Data Analytics (MDA) – Phase 4 | TSG#111 (Mar. 2026) | TSG#111 (Mar. 2026) | TBD |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

# 6	Work item Rapporteur(s)

# 7	Work item leadership

SA5

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

Supporting IM name

AT&T ?

DTAG ?

Docomo ?

CMCC ?

China Telecom ?

Fibercop ?

Huawei ?

NEC ?

Orange ?

Verizon ?

ZTE ?