| 3GPP TS 28.561 V0.3.0 (2025-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management aspects of Network Digital Twins (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	9

4	Concepts and overview	9

4.1	Introduction and Overview	9

4.1.1	Digital Twins and Network Digital Twins	9

4.1.2	Utilizing emulation and/or simulation	9

4.2	NDT Management Service	9

5	NDT Management capabilities	10

5.1	Control and life cycle management of NDTs - DTLCM	10

5.1.1	Description	10

5.1.2	Use cases	10

5.1.2.1	Life cycle management of NDT instances - DTLCM	10

5.1.2.2	Control of NDT instances - NDTCTR	10

5.1.2.3	Synchronization with network - DTSYNC	10

5.1.3	Requirements	11

5.2	NDT support for network automation	11

5.2.1	Description	11

5.2.2	Use cases	11

5.2.2.1	General capabilities on NDT support for network automation	11

5.2.2.2	Support for evaluation of high-risk network operations	12

5.2.2.3	Support for evaluation of failure events including signalling storm	12

5.2.2.4	Network issue inducement	12

5.2.3	Requirements	13

5.3	NDT support for verification - NDTVER	13

5.3.1	Description	13

5.3.2	Use cases	13

5.3.2.1	General verification – NDTVER_01	13

5.3.2.2	Verification of network response to events – NDTVER_02	14

5.3.2.3	Verification of network configurations- NDTVER_03	14

5.3.2.4	Verification of automation-function configurations – NDTVER_04	14

5.3.3	Requirements	14

5.4	NDT support for data generation	15

5.4.1	Description	15

5.4.2	Use cases	15

5.4.2.1	General use case on NDT support for data generation	15

5.4.2.2	Using NDT to generate ML training data	15

5.4.2.3	Using NDT to generate user experience data	15

5.4.3	Requirements	16

5.5	Advanced NDT capabilities - NDTADV	16

5.5.1	Description	16

5.5.2	Use Cases	16

5.5.2.1	Collaboration between NDTs - NDTADV_01	16

5.5.3	Requirements	17

6	Management service for NDT (stage 2)	17

6.1	Management operation for NDT (MnS component type A)	17

6.2	Information model for NDT (MnS component typeB)	17

6.2.1	Information model definition	17

6.2.1.1	Imported information entities and local labels	17

6.2.1.2	Class diagram	18

6.2.1.2.1	Relationship	18

6.2.1.2.2	Inheritance	18

6.2.1.3	Class definition	18

6.2.1.3.1	NDTFunction <<InformationObjectClass>>	18

6.2.1.3.1.1	Definition	18

6.2.1.3.1.2	Attributes	18

6.2.1.3.1.3	Attribute constraints	19

6.2.1.3.1.4	Notifications	19

6.2.1.3.2	NDTJob <<InformationObjectClass>>	19

6.2.1.3.2.1	Definition	19

6.2.1.3.2.2	Attributes	19

6.2.1.3.2.3	Attribute constraints	20

6.2.1.3.2.4	Notifications	20

6.2.1.3.3	NDTFunctionScope << dataType >>	20

6.2.1.3.3.1	Definition	20

6.2.1.3.3.2	Attributes	20

6.2.1.3.3.3	Attribute constraints	20

6.2.1.3.3.4	Notifications	20

6.2.1.3.4	NDTInputDescription  <<dataType>>	20

6.2.1.3.4.1	Definition	20

6.2.1.3.4.2	Attributes	21

6.2.1.3.4.3	Attribute constraints	21

6.2.1.3.4.4	Notifications	21

6.2.1.3.5	NDTOutputDescription <<dataType>>	21

6.2.1.3.5.1	Definition	21

6.2.1.3.5.2	Attributes	21

6.2.1.3.5.3	Attribute constraints	21

6.2.1.3.5.4	Notifications	21

6.2.1.3.6	ScopeDefinition <<choice>>	21

6.2.1.3.6.1	Definition	21

6.2.1.3.6.2	Attributes	22

6.2.1.3.6.3	Attribute constraints	22

6.2.1.3.6.4	Notifications	22

6.2.1.3.7	NdtJobExecutionReqts <<dataType>>	22

6.2.1.3.7.1	Definition	22

6.2.1.3.7.2	Attributes	22

6.2.1.3.7.3	Attribute constraints	22

6.2.1.3.7.4	Notifications	22

6.2.1.3.8	NDTReport <<InformationObjectClass>>	22

6.2.1.3.8.1	Definition	22

6.2.1.3.8.2	Attributes	23

6.2.1.3.8.3	Attribute constraints	23

6.2.1.3.8.4	Notifications	23

6.2.1.3.9	NDTOutputDataPoint <<dataType>>	23

6.2.1.3.9.1	Definition	23

6.2.1.3.9.2	Attributes	23

6.2.1.3.9.3	Attribute constraints	23

6.2.1.3.9.4	Notifications	23

6.3	Attribute definitions	23

6.4	Procedure for consuming NDT management service	29

6.5	Procedure for delete an NDT job	29

6.6	Procedure for query an NDT job	30

7	Management service for NDT (stage 3)	30

Annex A (informative): PlantUML source code	31

A.1	Information model definition for NDT	31

A.1.1	Relationship UML diagram for NDT management	31

A.1.2	Inheritance UML diagram for NDT management	31

A.2	Procedure for NDT management	31

A.2.1	Procedure for delete an NDT job	31

A.2.2	Procedure for query an NDT job	32

Annex B (informative): 	32

B.1	NDT function in RAN domain	32

B.2	NDT function in cross domain	32

Annex C (informative): Change history	33

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document specifies the terms and concepts of Network Digital Twin, as well as the use cases, requirements, and solutions for Network Digital Twins within the 3GPP management system.

The present document applies to the service based management architecture.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[4]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[5]	3GPP TS 28.111: "Management and orchestration; Fault Management (FM)".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Network Digital Twin (NDT): virtual replica of mobile network or part of one, that captures its attributes, behaviour and interactions

NOTE 1:	Mobile network includes both RAN and Core.

NOTE 2: 	NDTs can interact with other NDTs during simulation or emulation activities, and dynamically adapt their internal behavior based on such interactions.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

NDT	Network Digital Twin

S-NSSAI	Single Network Slice Selection Assistance Information

4	Concepts and overview

4.1	Introduction and Overview

4.1.1	Digital Twins and Network Digital Twins

A digital twin is a representation of an object that models the characteristics and behaviours of a real-world object or system. The digital twin provides support to network management and operations by creating a virtual representation of the corresponding physical network process(es). A digital twin can be created for any physical object, including any objects in communication networks. The digital twin may also be created for a group of objects, e.g. for the sets of network objects that form the RAN segment or the NFs in Core network.

Accordingly, a digital twin modelling of an object of a communication network is called Network Digital Twin.

Network Digital Twin (NDT) is used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, NDT needs to model the behaviour of the mobile network, so that the result of the operations on the virtual replica are good approximations to similar operations on the actual network. The implementation of an NDT can rely on simulation, emulation, AI-based modelling, or any other technique that enables the NDT to mimic the behaviour of the network.

Thus, NDT contributes to efficient management of mobile networks, helps building resilient networks, enables the early deployment of new services, and enhances network quality. For example, with NDT, network operators can verify network behaviour before they apply to real network to prevent unintended behaviour, which contributes to resilient networks and enhancement of network quality. Additionally, network automation function can use NDT to analyze network behaviour, which can contribute to reduce operator’s manual operation and improve management efficiency. NDT may also utilize network automation functions to deliver NDT reports.

4.1.2	Utilizing emulation and/or simulation

The implementation of NDT modelling relies on simulation, emulation or other modelling technologies that enables the NDT to mimic the behaviour of the network.

Simulation implies that the NDT uses a mathematical model to characterize the behavior of the system. This is used e.g. to model the traffic generated by users in the RAN to model the movement of users, or a simplified behaviour of a NF. Emulation implies that the NDT uses a realizable object such as software that replaces the live object to characterize the behavior of at least one aspect of the system. This is used e.g. to replicate or mimic the functionalities of core network functions which are implemented using the real software of NFs.

It is possible to combine emulation and simulation to create an integrated solution. The choice depends on implementation and deployment considerations, thus out of scope of this specification.

4.2	NDT Management Service

The NDT MnS can be requested by the NDT MnS consumer to model the behaviour of the network and generate the simulation/emulation output. The simulation/emulation capabilities provided by NDT MnS Producer can be invoked by the network intelligence and automation management functions (e.g. MDAF, AIML Inference Function, Intent Handling Function, etc.) playing the role of NDT MnS Consumer to support their intelligence and automation functionality.

Figure 4.2-1

5	NDT Management capabilities

5.1	Control and life cycle management of NDTs - DTLCM

5.1.1	Description

This describes uses on control and life cycle management of NDT instances.

5.1.2	Use cases

5.1.2.1	Life cycle management of NDT instances - DTLCM

NDTs can be used for many application use cases. In each case, the NDT needs to implement a defined network scenario, run a simulation/emulation of that scenario, and subsequently provide an output representing the statuses of different network for the simulated network scenario. The lifecycle management of NDT refers to the management processes that control and transition an NDT instance through the processes of creation, activation, operation, deactivation, deletion and any updates during these processes. At each step, it is critical to ensure that the NDT instance accurately reflects the status of physical entities and support efficient simulation/emulation.

-	For creation, the NDT MnS Consumer may provide requirements to request the NDT MnS producer to trigger the creation of an NDT instance based on its capabilities. NDT MnS producer may create a new NDT instance from scratch, updating existing NDT models or to combining existing NDT models to generate a new NDT.

-	At any point during the life of an NDT instance, the MnS consumer may provide physical network data to refine and adjust the NDT instance to accurately reflect the status of the physical entity and adapt it to specific application scenarios. For example, the consumer may trigger data collection from the network and synchronization of the state with the NDT.

5.1.2.2	Control of NDT instances - NDTCTR

The 3GPP management system should support capabilities enabling consumers to define the network scenario, to control (start, suspend, resume or stop) the simulation/emulation of that network scenario and to provide outcomes representing measurements and counters from metrics in the simulated/emulated network scenario.

An NDT may have dependencies on other NDTs or DTs, each of which implements a specific network scope. The MnS consumer should also be enabled to provide input to NDT MnS Producer to create relationship among NDT instances, i.e., configure or modify the dependencies (e.g., to select which environment or radio propagation DT to be applied).

5.1.2.3	Synchronization with network - DTSYNC

The NDT needs to obtain information from physical network including network functions and entities. The collected information includes the following:

-	Performance data: Performance measurement or the KPI as defined in 3GPP TS 28.552 [2] and 3GPP TS 28.554 [3].

-	MDT/Trace data: MDT measurements as defined in 3GPP TS 32.422 [4].

-	Fault management data: Fault management data as defined in 3GPP TS 28.111[5]

-	Configuration data: Configuration data defined in 3GPP TS 28.541 [6] and 3GPP TS 28.622 [7], any of the available MOIs;

Mobile networks are generally composed of numerous network elements. The data about these numerous network elements can potentially be vast. When collecting this vast amount of data to NDT, the collection might cause congestion in the network bandwidth used for data collection. NDT MnS Producer may create an NDT instance according to data provided by the NDT MnS Consumer or collect the data from the physical network for synchronization. The MnS producer should enable the MnS consumer to control the frequency of data to be collected by the NDT to minimize unnecessary collection of data and minimal congestion.

5.1.3	Requirements

Table 5.1.3-1: Control and life cycle management of NDTs

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-DTLCM-DTLCM-01: | The 3GPP management system should support a capability enabling an authorized MnS consumer to define the network scenario that should be modelled and simulated/emulated. | DTLCM-DTLCM |
| REQ-DTLCM-DTLCM-02: | The 3GPP management system should support a capability enabling an authorized MnS consumer to control the simulation/emulation of a network scenario, e.g. to start, or stop the simulation/emulation. | DTLCM-DTLCM |
| REQ-DTLCM-DTLCM-03: | The 3GPP management system should support a capability to provide to an authorized MnS consumer an output of a modelling or simulation process, the output representing the status of the network for the simulated scenario. | DTLCM-DTLCM |
| REQ-DTLCM-NDTCTR-01: | The 3GPP management system should support a capability to allow the MnS consumer to provide input to NDT MnS Producer to create relationship among NDT instances | DTLCM-NDTCTR |
| REQ-DTLCM-DTSYNC-02: | The 3GPP management system should have the capability enabling an authorized MnS consumer to control the frequency of network data collection. | DTLCM-DTSYNC |

5.2	NDT support for network automation

5.2.1	Description

NDTs can be used to support automation use cases (e.g. MDA, SON, etc.). An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it could be possible for the network automation function to configure the NDT and the scenario that could be modelled by the NDT. NDTs do not execute any actions in form of configuration of the live network but support decision-making.

5.2.2	Use cases

5.2.2.1	General capabilities on NDT support for network automation

An NDT, depending upon the network or service management use case and scenario to be modelled, might need data originating from various sources (network data, environment data, analytic, UEs data) and suitable hardware/software resources to function properly. MnS consumers would need to specify needed NDT characteristics or configurations to the NDT tailored to fulfil consumer specific needs i.e. to define the consumer preference for the specific NDT. For example, consumer preferences may be related to environment data sources e.g. weather, synthetic data etc, data characteristics (e.g. robustness, data granularity, maximum tolerable latency), required NDT output latency, characteristics of the service to be twinned, resource constraints (HW/SW), etc. Furthermore, in the case that consumer's preference on NDT characteristics or configuration may change over time and MnS consumer may update the NDT with the needed changes.

The achieved performance by NDT may depend on NDT characteristics such as load, time or energy saving state. This means that the simulation results would change based on the network load, the target time of the day, or on whether the simulated node is running in energy saving mode. The NDT report should indicate the performance for the respective configured NDT characteristics.

The automation scenarios that the NDT might support are described in following sub-clauses.

5.2.2.2	Support for evaluation of high-risk network operations

Each operation for network optimization and maintenance on mobile network may cause potential network failures, including network congestion and network breakdown. High-risk operations are the operations with increased risk of failure, such as potentially dangerous configuration modification, policy modification, software version upgrade, and board switching. Operators should not carry out the high-risk network operations or perform direct optimization in the physical network. The NDT can be used for evaluation of high-risk network operations.

Using NDT, high-risk operations causing potential network failures can be evaluated. NDT MnS Producer can also be requested to create an instance for optimization and verification of network policies and solutions for the further risk avoidance. After simulating/emulating a scenario, NDT MnS Producer can generate the results and report them back to the MnS consumer. Based on results, the MnS consumer can generate possible network policies and solutions to minimize the impact of high-risk operations.

As an example, the NDT can predict SLA degradation and single node failures in mobile networks. When the prediction indicates insufficient network resources to maintain SLA compliance or potential hardware resource failures, the NDT results help operators to take appropriate preventive actions to avoid network outages.

5.2.2.3	Support for evaluation of failure events including signalling storm

The NDT can be used to support the evaluation of failure event refers to the situation where users in the network are completely unable to get service from the network. A typical example of a failure event is signalling storm, where a large number of signalling messages suddenly surge in the mobile communication network, overloading the network's processing capacity and consequently degrading the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signalling messages surge suddenly, causing signalling storm. When a signalling storm occurs on the network, network Functions (NFs) such as AMF, SMF, and UDM in the 5G network might all be potential impact points. Therefore, when received the request with network object information from MnS consumer for signalling storm analysis, the NDT can execute the modelling and validation based on network object related information (network performance, KPIs, S-NSSAIs, etc.), which synchronized from the network resources, and provide the report with the modelling results to MnS consumer.

An automation function may propose a solution to a signalling storm. The NDT can be used to evaluate the appropriateness of the proposed solution (e.g.,update the configuration of network flow control parameters) based on the analysing of failure events for resolving, predicting and preventing the signalling storm issue.

NDT models network behaviors and provides information on potential impacts of network failures, including surges in signalling requests. For instance, in case of the signalling storm, the information can enable identification of optimal flow control parameters for each signalling impact point.

5.2.2.4	Network issue inducement

This use case describes how a network issue can be induced using NDT. A resilient network requires that the behaviour and performance of the network are monitored during certain network failure issue e.g. node/functionality failure, service degradation etc. To plan for the optimal network configuration in case of such network failure issue, NDT can be used for issue inducements. For a particular issue that is induced, the NDT measures performance identifies degradation/faults/failures and the mitigation actions can be decided. The following are some of the examples of the issues that can be induced.

Scenario 1: Service degradation - The network slice performance degradation in terms of low PDU session establishment success rate or in terms of high latency can be induced, in a NDT, to see how the related network functions will behave when the PDU session establishment success rate is degraded. The remedial actions can be decided to mitigate the problem.

Scenario 2: Coverage issue - The coverage issue can be induced, in a NDT, to see how the related services are getting effected. The remedial actions can be decided to mitigate the problems arising due to the induced coverage issue.

Scenario 3: Fault injection - NDT can be used for fault injection experiments avoiding impact on the physical network while measuring and monitoring the impact of each injected fault in the NDT simulation. This could be leveraged to build a training data set for enhancing and enriching detection and diagnosing systems capabilities. In addition, NDT could be leveraged for improving root causes analysis.

5.2.3	Requirements

Table 5.2.3-1: NDT support for network automation

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTAUTO-01: | The 3GPP management system should support a capability enabling an authorized MnS consumer to specify characteristics or configurations of the network scenario to be modelled in the NDT. | General capabilities on NDT support for network automation (Clause 5.2.2.1) |
| REQ-NDTAUTO-02: | The 3GPP management system should support a capability to report to the authorized MnS consumer the simulation/ emulation output by NDT. | General capabilities on NDT support for network automation (Clause 5.2.2.1) |
| REQ-NDTAUTO-03: | The 3GPP management system should support a capability enabling an authorized MnS consumer to define and request simulation/ emulation of a network optimization operations. | Support for evaluation of high-risk network operations (Clause 5.2.2.2) |
| REQ-NDTAUTO-04: | The 3GPP management system should have a capability enabling an authorized MnS consumer to configure the network scenario to be modelled for the consumer to evaluate a network failure. | Support for evaluation of high-risk network operations (Clause 5.2.2.2) |
| REQ-NDTAUTO-05: | The 3GPP management system should support a capability to model the behaviour of the signalling storm in network. | Support for evaluation of failure events including Signaling storm (Clause 5.2.2.3) |
| REQ-NDTAUTO-06: | The 3GPP management system should support a capability enabling an authorized MnS consumer to define and request simulation/ emulation of the proposed solution for resolving a set of network failure events, including signalling storm. | Support for evaluation of failure events including Signaling storm (Clause 5.2.2.3) |
| REQ-NDTAUTO-07: | The 3GPP management system should support a capability enabling an authorized MnS consumer to request inducement of a network issue by an NDT. | Network issue inducement (Clause 5.2.2.4) |
| REQ-NDTAUTO-08: | The 3GPP management system should have a capability for NDT MnS producer to report the impact of the injected issue in NDT. | Network issue inducement (Clause 5.2.2.4) |

5.3	NDT support for verification - NDTVER

5.3.1	Description

This describes NDT use in support of verification.

5.3.2	Use cases

5.3.2.1	General verification – NDTVER_01

To evaluate a network scenario, the NDT needs to be configured with data defining a network scenario, and where applicable, the configurations and functionalities to be evaluated. The MnS consumer should be enabled to configure the NDT accordingly. Given the scenario, configurations and functionalities, the consumer can then control the simulation (e.g. to start or stop the simulation) according to the LCM capabilities as described in clause 5.1. The NDT simulates the defined scenario configurations and functionalities and collects data, which is then compiled into an NDT report. The MnS consumer should be enabled to configure the information that should be included in the simulation report. The NDT MnS producer should be enabled to send the simulation report to MnS consumer to indicate the network simulation results.

5.3.2.2	Verification of network response to events – NDTVER_02

The network can experience different kinds of events, including failure and non-failure events. A failure event refers to the situation where the network does not behave as expected, e.g., where users in the network are completely unable to get service from the network. A non-failure event refers to the situation where the network experiences an unusual occurrence, but users are still able to get service. The NDT can be used to evaluate how the network responds to any of these events. The NDT can model network behaviors and provide information on potential impact of network events. The NDT can be used to evaluate if a particular event will cause a network failure or whether the network can withstand the event.

The following is an example of events that can be supported:

- 	Signaling storms: A signaling storm is a failure events where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm. For a signaling storm, the NDT provides information on potential impact of surges in signaling requests on the network. The information can enable the MnS consumer to identify the optimal flow control parameters for each signaling impact point.

5.3.2.3	Verification of network configurations- NDTVER_03

Several configurations can be provided to the network, either as modifications to single parameters or as bulk configurations, e.g., as a configuration plan. There may be multiple configurations that are iteratively executed in NDT until the network performance, e.g. energy efficiency of NG-RAN, UE throughput in gNB, etc., meets the operators’ requirements. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated as well as the input information that should be applied to network in that scenario. The information may include configurations, simulated network scenarios or simulated traffic patterns.

The configurations can be evaluated for any network scenarios including the event listed in 5.3.2.1

5.3.2.4	Verification of automation-function configurations – NDTVER_04

Besides network configurations, network automation functionality (e.g., SON functions) may also be configured., e.g. configurations of functionality say for energy savings. The NDT can be used to evaluate the configurations of the automation functionality. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be evaluated for one or more automation functionality with the corresponding input information that should be applied to automation functionality to be evaluated. The information may include configurations, simulated network scenarios or simulated traffic patterns.

An example scenario of automation configuration verification is verification of Configuration of RAN energy saving management function (RAN ES MnF). A RAN ES MnF derives and executes network management operations via provisioning MnS to maximize energy saving for a group of cells e.g. based on the degree to which the cells overlap with each other. The NDT can be used to evaluate the configurations of the RAN ES MnF, i.e., the MnS consumer may instantiate an NDT simulation/emulation to verify the RAN ES configuration. The verification can be for configuration of behavior including which cells are grouped for ES and their degree of overlap. The verification can also be for configuration of performance requirements, which is MnS consumer’s requirements for network simulation performance, e.g., energy efficiency, RAN UE throughput, precision, maximum run time, etc. NDT performs network simulation/emulation, i.e., simulating/emulating the execution of RAN ES according to the configured behavior and performance requirements and collects its impact on the simulated network.

5.3.3	Requirements

Table 5.3.3-1: NDT support for verification

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTVER_01-01: | The management system should support a capability enabling an authorized MnS consumer to define the information that should be included in the report/output on the simulation/emulation of verification of network scenarios and configurations. | NDTVER_01 |
| REQ-NDTVER_02-01: | The management system should have a capability enabling an authorized MnS consumer to define one or more network events to be modelled and evaluated by the NDT. | NDTVER_02 |
| REQ-NDTVER_02-02: | The management system should have the capability to report the simulation/emulation outcomes on the verification of one or more network events to be modelled and evaluated by the NDT | NDTVER_02 |
| REQ-NDTVER_03-01: | The management system should have a capability enabling an authorized MnS consumer to define one or more network configurations to be modelled and evaluated by the NDT. | NDTVER_03 |
| REQ-NDTVER_03-02: | The management system should have the capability to report the simulation/emulation outcomes on the verification of network configurations modelled and evaluated by the NDT. | NDTVER_03 |
| REQ-NDTVER_04-01: | The management system should have a capability enabling an authorized MnS consumer to define one or more automation functionality and configurations for the automation functionality to be modelled and evaluated by the NDT. | NDTVER_04 |
| REQ-NDTVER_04-02: | The management system should have the capability to report the outcomes on the verification of one or more automation functionality and configurations modelled and evaluated by the NDT. | NDTVER_04 |

5.4	NDT support for data generation

5.4.1	Description

NDTs can be used to support many generation capabilities for different application use cases.

5.4.2	Use cases

5.4.2.1	General use case on NDT support for data generation

The NDT should support a capability to provide a report/output on the simulation/emulation enabling generation of data and information related to network scenarios, configurations, policies, and performance outcomes.

The data generation scenarios that the NDT might support include those in following sub-clauses.

5.4.2.2	Using NDT to generate ML training data

ML training usually requires large amounts of data to guarantee good performance of the ML models. In general, the ML training data for network related use cases is obtained through historical network management data. For instance, assuming that there is a ML model supporting MDA SLS analysis described in TS 28.104 clause 7.2.2, the raw feature of training data could be the enabling data, such as UL/DL throughput, uplink/downlink delay, etc., as specified in clause 8.4.2 of TS 28.104.

However, obtaining data from the network has the following limitations:

-	The quantity of issues happened in actual mobile network is limited.

-	The variety of issues happened in actual mobile network is limited. There could be corner network issues that hardly happen in live network.

Sufficient ML training data plays a key role to a useful ML model. The more training data provided, the better the performance of ML model. To overcome these challenges, the MnS consumer can request the NDT to generate data with an indication of data requirements, e.g. data type, required data period, data sampling periods, etc. When the request is sent by the MnS consumer, the NDT can execute the simulation and generate data corresponding to the request. The NDT sends a report with the generated data to the MnS consumer, which can be used to enhance model accuracy by providing a wide range of training examples that reflect potential real network conditions.

5.4.2.3	Using NDT to generate user experience data

For operators, it is important to accurately measure user satisfaction with the network services from a network usage perspective. When the performance metrics related to customer satisfaction are low, operators are eager to identify the underlying causes and figure out ways to boost performance. Multiple factors influence user satisfaction, including user service quality, network usage experience etc. Digital twin technology can be applied here to model and simulate end - user behaviors within the network, providing valuable insights for improving network services.

The satisfaction of network service is affected by combining factors, and the digital twin technology is desirable to model and simulate the user usage experiences by integrating multi-domain data sources which include network performance and user experience data, fault prediction. This proactive approach allows CSPs to identify potential detractors who are not satisfied with services on the entire network, monitor the end user journey, gain deeper insights into the end user's needs, and perform refined experience management based on user groups.

5.4.3	Requirements

Table 5.4.3-1: NDT support for data generation

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTDG-01: | The 3GPP management system should support a capability to provide an output on the simulation/emulation enabling generation of data. | General use case on NDT support for data generation (See clause 5.4.2.1) |
| REQ-NDTDG-02: | The 3GPP management system should have the capability to allow an authorized MnS consumer to request generation of simulated network data to be used for ML training | Using NDT to generate ML training data (See clause 5.4.2.2) |
| REQ-NDTDG-03: | The 3GPP management system should support a capability to report the generated data for ML training. | Using NDT to generate ML training data (See clause 5.4.2) |
| REQ-NDTDG-04: | The 3GPP management system should support a capability to allow an authorized MnS consumer to request generation of the user experience data. | Using NDT to generate user experience (See clause 5.4.3) |
| REQ-NDTDG-05: | The 3GPP management system should support a capability to report the generated user experience data. | Using NDT to generate user experience (See clause 5.4.3) |

5.5	Advanced NDT capabilities - NDTADV

5.5.1	Description

This clause describes advanced uses of Network digital twins.

5.5.2	Use Cases

5.5.2.1	Collaboration between NDTs - NDTADV_01

A single NDT might not be able to fulfil a task by itself and may be dependent on or need to use the service or outputs of another NDT during the simulation/emulation activity. Thus, the MnS consumer should be able to configure the relation between NDTs during simulation/emulation.

The benefits of collaboration between NDTs are:

-	NDTs are able to interact with each other, supporting information exchange and coordinated behavior.

-	NDTs can dynamically adapt their behavior according to the information exchanged and collaboration in place with other NDTs.

-	NDTs collaborations enhance their capabilities and system awareness.

5.5.3	Requirements

Table 5.5.3-1: Advanced NDT capabilities

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTAUT-GEN-01 | The 3GPP management system should support a capability enabling an authorized MnS consumer to configure relation between NDTs during simulation/emulation. | NDTADV_01 |

6	Management service for NDT (stage 2)

6.1	Management operation for NDT (MnS component type A)

The operations (e.g., createMOI operations) and notifications (e.g., notifyMOIcreation) of generic provisioning MnS defined in 3GPP TS 28.532 [8] can be used for NDT management, including NDT function management, NDT job management and NDT report management. The NDTFunction, NDTJob, NDTReport instances can be treated as Managed Object instances.

Following is the IS to support NDT lifecycle management:

Table 6.1-1: IS to support NDT function management

| NDT function management | IS operation |
| --- | --- |
| Query an NDT function | getMOIAttributes operation |

Following is the IS to support NDT job management:

Table 6.1-2: IS to support NDT job management

| NDT job management | IS operation |
| --- | --- |
| Create an NDT job | createMOI operation |
| Delete an NDT job | deleteMOI operation |
| Modify an NDT job | modifyMOIAttributes operation |
| Query an NDT job | getMOIAttributes operation |

Following is the IS to support NDT report management:

Table 6.1-3: IS to support NDT report management

| NDT lifecycle management | IS operation |
| --- | --- |
| Query an NDT report | getMOIAttributes operation |
| Subscribe an NDT report | createMOI operation |
| Notify an NDT report | notifyMOIAttributeValueChanges notification |
| Unsubscribe an NDT report | deleteMOI operation |
| Query an NDT report subscription | getMOIAttributes operation |

6.2	Information model for NDT (MnS component typeB)

6.2.1	Information model definition

6.2.1.1	Imported information entities and local labels

| 3GPP TS 28.622 [7], DataType, GeoArea | GeoArea |
| --- | --- |
| 3GPP TS 28.622 [7], DataType, TimeWindow | TimeWindow |
| 3GPP TS 28.622 [7], DataType, ThresholdInfo | ThresholdInfo |
| 3GPP TS 28.622 [7], ConditionMonitor | ConditionMonitor |

6.2.1.2	Class diagram

6.2.1.2.1	Relationship

Figure 6.2.1.2.1-1: Relationship UML diagram for NDT management

6.2.1.2.2	Inheritance

Figure 6.2.1.2.2-1: Inheritance UML diagram for NDT management

6.2.1.3	Class definition

6.2.1.3.1	NDTFunction <<InformationObjectClass>>

6.2.1.3.1.1	Definition

This IOC represents the properties of an NDT Function.

The NDTFunction includes a reference to one or more NDTFunction instances which act as component NDTs contributing to the functionality of the NDTFunction.

6.2.1.3.1.2	Attributes

The NDTFunction IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

The NDTFunction has a relationship indicating that an NDT Function may be associated with one or more other NDT Functions. For example, one or more NDT Functions with small scope might support the operation of an NDT Function with a wider scope.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedNDTCapabilities | M | T | F | T | T |
| nDTFunctionScope | M | T | F | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtRef | M | T | T | F | T |

6.2.1.3.1.3	Attribute constraints

None.

6.2.1.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

6.2.1.3.2	NDTJob <<InformationObjectClass>>

6.2.1.3.2.1	Definition

This IOC represents the properties of an NDT job demand created by an MnS consumer.

The attribute "nDTJobSynchScope” indicates the scope of the network that should be synchronized into and modelled by the NDT for the specific NDT job. If the NDTjob is not meant to synchronize with the network, no cnten tis provided in the nDTJobSynchScope attribute.

The attribute "ndtJobScenario" indicates the input that is defined by MnS consumer for the characteristics of network objects that should be simulated/emulated by NDT. If the NDT is able to synchronize with an actual network, the ndtJobScenario indicates the delta between the actual network and twin network that is simulated/emulated. Otherwise, it indicates the critical features that should be modelled, allowing the NDT to use defaults for all other features. The ndtJobScenario can be network configurations or automation functionality configurations, network events, issues that are defined by MnS consumer and will be injected to NDT.

The ndtJobScenario also captures requirements to be simulated to see the network’s response to specific network events or issues. The network issue or events that ned to be simulated/emulated (including configuration, performance and fault characteristics may result in a particular network issue) are added into the networkResponseTask attribute.

The attribute "ndtJobExecutionRequirements" represents the execution-related requirements for an NDT job, e.g., maximum run time for each simulation/emulation task, precision, etc which are used to select the model parameters (e.g., simulation/emulation step and number of simulation/emulation times) for building NDT models. It is up to implementation how the NDT model is built and used to execute the simulation/emulation task.

NOTE: the model for the specific tasks can be extended as needed.

6.2.1.3.2.2	Attributes

The NDTJob IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTCapability | M | T | T | F | T |
| nDTJobSynchScope | M | T | T | F | T |
| ndtJobScenario | M | T | T | F | T |
| ndtJobExecustionRequirements | O | T | T | F | T |
| collaboratingNDT | O | T | T | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtReportRefList | M | T | T | F | T |

6.2.1.3.2.3	Attribute constraints

None.

6.2.1.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

6.2.1.3.3	NDTFunctionScope << dataType >>

6.2.1.3.3.1	Definition

This IOC represents the properties of a scope that can be modelled by an NDT

6.2.1.3.3.2	Attributes

The NDTFunctionScope IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.3.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| nDTRANScope | M | T | T | F | T |
| nDTCNScope | M | T | T | F | T |

6.2.1.3.3.3	Attribute constraints

None.

6.2.1.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.4	NDTInputDescription<<dataType>>

6.2.1.3.4.1	Definition

This dataType represents a description of the network. It may be used to describe any of the following:

-	aspects of the network that should be modelled in the NDT,

-	configurations that should be applied or have been applied by the NDT

The objects to be considered may be described in terms of the attribute networkObjectType or by specific network object instances represented by objectInstance. If a list of specific managed objects are to be modelled, they are listed in the attribute objectInstance. Otherwise, their type if listed in the attribute networkObjectType and their specific characteristics listed in objectAttributeList.

6.2.1.3.4.2	Attributes

The NDTInputDescription  <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.4.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTInputDescriptionId | M | T | T | F | T |
| simulationData | O | T | T | F | T |
| networkEventInfo | O | T | T | F | T |
| condition | M | T | T | F | T |

6.2.1.3.4.3	Attribute constraints

None

6.2.1.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.5	NDTOutputDescription <<dataType>>

6.2.1.3.5.1	Definition

This dataType represents a description of the individual outputs of the NDT modelled twin. It may be used to describe any of the following:

-	states of the network that have been modelled and are being reported by the NDT,

-	characteristics of the network that are being reported by the NDT.

The objects that have been modelled are described by specific network object instances represented by objectInstance and their specific characteristics (PMs, alarms, etc.) listed in objectAttributeList.

6.2.1.3.5.2	Attributes

The NDTInputDescription  <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.5.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTInputDescriptionId | M | T | T | F | T |
| objectInstance | M | T | T | F | F |
| objectAttributeList | M | T | T | F | T |

6.2.1.3.5.3	Attribute constraints

None

6.2.1.3.5.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.6	ScopeDefinition <<choice>>

6.2.1.3.6.1	Definition

This <<choice>> represents a definition of a scope.

The managedEntitiesScope attribute identifies the scope by the DNs of the managed entities.

The areaScope attribute identify the scope by the geographical area information.

The managedEntitiesScope attribute and areaScope attribute shall not be present at the same time.

6.2.1.3.6.2	Attributes

Table 6.2.3.6.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Choice_1 managedEntitiesScope | CM | T | T | F | T |
| Choice_2 areaScope | CM | T | T | F | T |

6.2.1.3.6.3	Attribute constraints

Table 6.2.1.3.6.3-1

| Name | Definition |
| --- | --- |
| _1 managedEntitiesScope | the MnS producer supports identifying the scope by managed entities. |
| _2 areaScope | the MnS producer supports identifying the scope by geographical area information. |

6.2.1.3.6.4	Notifications

None.

6.2.1.3.7	NdtJobExecutionReqts <<dataType>>

6.2.1.3.7.1	Definition

The NdtJobExecutionReqs <<dataType>> represents the execution requirements for NDT job.

6.2.1.3.7.2	Attributes

The NdtJobExecutionReqs includes the following attributes.

Table 6.2.1.3.7.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| maxRuntime | O | T | T | F | F |

6.2.1.3.7.3	Attribute constraints

None

6.2.1.3.7.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType.

6.2.1.3.8	NDTReport <<InformationObjectClass>>

6.2.1.3.8.1	Definition

This IOC represents the properties of an NDT report corresponding to an NDT job.

An NDT job may run more than one task at the same time, e.g.,  a network configuration task and a network response task. The NDTReport contains an output for each task that is executed by the NDT job.

The attribute "NDTJobOutputData"  specified performance metrics and/or alarm types that are collected and reported by NDT after the behaviour is modelled in NDT is put in a list for which each entry is an NDTOutputDataPoint.

6.2.1.3.8.2	Attributes

The NDTReport IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

Table 6.2.1.3.8.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| NDTJobOutputData | M | T | F | F | T |
| Attribute related roles |  |  |  |  |  |
| ndtJobRef | M | T | F | F | T |
| ndtFunctionRef | M | T | F | F | T |

6.2.1.3.8.3	Attribute constraints

None.

6.2.1.3.8.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC.

6.2.1.3.9	NDTOutputDataPoint <<dataType>>

6.2.1.3.9.1	Definition

This dataType represents a single output of the NDT.

6.2.1.3.9.2	Attributes

The nDTOutputDataPoint <<datatype>> includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [7]) and the following attributes.

The nDTOutputDataPoint indicates three different types of information about the network. The networkState indicates the state of the network at which a specific networkConfiguration (i.e. a delta of the network state or new state of the netwrok) is applied. The observations shows the outcome on the network when the stated networkConfiguration is applied in the stated networkState.

Table 6.2.1.3.9.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| networkState | M | T | F | T | T |
| networkConfiguration | M | T | F | T | T |
| observations | M | T | F | T | T |

6.2.1.3.9.3	Attribute constraints

None.

6.2.1.3.9.4	Notifications

The common notifications defined in clauses 6.1 are valid for this dataType .

6.3	Attribute definitions

6.3.1	Attribute properties

Table 6.3.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| ndtJobRef | It indicates an DN of a NDTJob Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtFunctionRef | It indicates an DN of a NDTFunction Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtReportRefList | It indicates a list of DN for NDTReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| supportedNDTCapabilities | It indicates the different types of NDT application use cases which the NDT is capable of undertaking. allowedValues: "RISKY-ACTIONS_PREDICTION", "EVENTS-IMPACTS_VERIFICATION", "FAULT_INJECTION", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML-TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY-ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS-IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "FAULT_INJECTION" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML-TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCapability | It indicates the type of application use cases that is desired to be executed. allowedValues: "RISKY-ACTIONS_PREDICTION", "EVENTS-IMPACTS_VERIFICATION", "FAULT_INJECTION", "NETWORK_EVENTS_VERIFICATION" "NETWORK_CONFIGURATIONS_VERIFICATION", "AUTOMATION_CONFIGURATION_VERIFICATION" "ML-TRAINING_DATA_GENERATION", "USER_EXPERIENCE_DATA_GENERATION" New values can be added to this list in future releases to support new use cases. The meaning of these values is as follows: "RISKY-ACTIONS_PREDICTION" means NDTFunction supports the use case described in 5.2.2.2. "EVENTS-IMPACTS_VERIFICATION" means NDTFunction supports the use case described in 5.2.2.3. "FAULT_INJECTION" means NDTFunction supports the use case described in 5.2.2.4. "NETWORK_EVENTS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.2. "NETWORK_CONFIGURATIONS_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.3. "AUTOMATION_CONFIGURATION_VERIFICATION" means NDTFunction supports the use case described in 5.3.2.4. "ML-TRAINING_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.2. "USER_EXPERIENCE_DATA_GENERATION" means NDTFunction supports the use case described in 5.4.2.3. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| nDTRANScope | It indicates the scope of the RAN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTCNScope | It indicates the scope of the CN that can be modelled by the NDT function. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| nDTJobSynchScope | It indicates the scope of the network that should be synchronized into and modelled by the NDT for the specific NDT job. | type: ScopeDefinition multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ndtJobScenario | It indicates a network scenario that should be modelled in the NDTJob as an extra beyond what is synchronized from the network. The ndtJobScenario can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. - Evaluation of high-risk network operations which are listed within the planned configuration - Verification of network configurations which are listed within the planned configuration - Using NDT to generate ML training data - Using NDT to generate user experience data | type: NDTInputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific input to be modelled in the NDTJob | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NDTInputDescriptionId | It indicates the identifier for a specific output provided as outcomes of the NDTJob | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkEveFntInfo | This defines the information related with a network event (a provisioning, performance measurement, KPI or fault/ alarm event) that can be introduced into the NDT. The NetworkEventData can be used for - Verification of network response to one or more events - evaluation of the impact of one or more failure events, e.g. a signalling storm - Evaluating one or more network issues, e.g. a coverage issue. The network issues involve one or more network events. Editor’s note: The definition and modelling of networkEventInfo is to be clarified | Type: TBD multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationData | This will define which management data is to be updated artificially in order to induce a particular network issue. The management data includes: - Performance data: The name of the performance measurement or the KPI as defined in 3GPP TS 28.552 and TS 28.554 - MDT/Trace data: The name of MDT measurements as defined in 3GPP TS 32.422 - Configuration data: The name of the attribute from any of the available MOIs. Editor’s note: The definition and modelling of simulationData is to be clarified | type: TBD multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| condition | This will define the condition that has to be satisfied in order to update the simulation data for the task that is executed by the NDT. This can be defined in terms of location and time. This will be the DN of ConditionMonitor[7]. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ndtJobExecustionRequirements | It describes the performance requirements for NDT modelling, e.g., maximum run time for each simulation/emulation job, precision, etc | type: NdtJobExecutionReqts multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| NDTJobOutputData | It indicates the list of NDTOutput(s) that are provided by the NDT function as the output for any task executed in an instantiated NDT job. | type: NDTOutputDataPoint multiplicity: 1 ..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| maxRuntime | Maximum run time for each simulation task executed in NDT Editor’s note: The unit of this attribute is to be added | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkState | It indicates a state of the twin network (the modelled network in the NDT) for which a configuration or reconfiguration is applied. The networkState is the desvription of what exists in the network at the time when the networkConfiguration is made | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| networkConfiguration | It indicates a network configuration that is executed by the NDT and being reported in the NDT report | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| observations | It indicates an impact on the network. It shows the list of network objects that are affected and the effects on the specific objects | type: NDTOutputDescription multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| collaboratingNDT | It indicates the related NDT Job contributing as a collaborator to the executed NDT Job. It describes a relationship to an NDT job, i.e. it indicates the DN of a component NDT which provides input to the NDT job | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

NDT

5.Execute the NDT job and generate corresponding reports

6. Send the NDT reports

4. Send the request for NDT job execution

1. Request the creation of an NDT

2a. Create the NDT function

3. Response for the NDT creation

MnS Consumer

2b. Synchronization

Managed entities

6.4	Procedure for consuming NDT management service

Figure 6.4-1: Procedure of consuming NDT management service

1. NDT MnS Producer receives a request from MnS consumer to create an NDT job instance (see createMOI operation defined in TS 28.532 [8]) which represents the consumer’s requirements for the NDT job. Examples of NDT job can be a signalling storm analysis, network configuration verification, data generation, etc.

2a. NDT MnS Producer creates the NDT function instance according to the request from MnS consumer. The request may include the scope of NDT job (e.g., simulated network objects such as network functions, S-NSSAI, time).

2b. NDT MnS Consumer synchronizes the simulated network objects related information from managed entities for NDT job modelling and validation.

3. NDT MnS Producer sends a response (see createMOI operation defined in TS 28.532[8]) to the MnS Consumer.

4. MnS consumer sends the request to NDT MnS Producer to start the execution of the NDT job instance.

5. NDT MnS producer executes the NDT job and creates an NDTReport instance (i.e., the instance of NDTReport IOC) and configures NDTJob MOI according to the modelling results.

6. NDT MnS Producer sends the NDT report to MnS consumer.

NOTE: The Procedure should be further updated to align with the agreed NRM.

6.5	Procedure for delete an NDT job

Figure 6.5-1: Procedure for delete an NDT job

1.	MnS Consumer sends a request to delete an NDT job instance (see deleteMOI operation defined in TS 28.532 [8]) to MnS Producer with 'objectInstance' of the NDT job MOI.

2.	Based on the request, MnS Producer deletes the NDT job MOI.

3.	MnS Producer sends response (see deleteMOI operation defined in TS 28.532 [8]) to the MnS consumer with status (OperationSucceeded or OperationFailed) and 'objectInstance' of the deleted NDT job MOI.

6.6	Procedure for query an NDT job

Figure 6.6-1: Procedure for query an NDT job

1.	MnS Consumer sends a request to query an NDT job instance (see getMOIAttributes operation defined in TS 28.532 [8]) to MnS Producer with 'objectInstance' of the existing NDT job MOI and list of attribute names of NDT job IOC. The list of attribute names identifies the attributes to be returned by this operation.

2.	Based on the request, the MnS Producer queries the concrete NDT job MOI.

3.	MnS Producer sends a response (see getMOIAttributes operation defined in TS 28.532 [8]) to the MnS consumer with 'objectClass', 'objectInstance', and list of [Attribute,Value] which is defined in clause 6.2.

7	Management service for NDT (stage 3)

Editor's note: this clause will specify the solution set for NDT capabilities and services defined in clause 6.

Annex A (informative):
PlantUML source code

A.1	Information model definition for NDT

A.1.1	Relationship UML diagram for NDT management

@startuml

hide empty members

skinparam ClassStereotypeFontStyle normal

hide circle

skinparam class {

BackgroundColor White

ArrowColor Black

BorderColor Black

}

skinparam linetype ortho

'skinparam BoxPadding 40

skinparam nodesep 2

class ManagedEntity <<ProxyClass>>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

note right of ManagedEntity

Represents SubNetwork

end note

ManagedEntity "1" *-- "*" NDTFunction: <<names>>

NDTFunction "1" *-- "*" NDTJob: <<names>>

NDTFunction "1" *-- "*" NDTReport: <<names>>

NDTJob "1" <--> "*" NDTReport

@enduml

A.1.2	Inheritance UML diagram for NDT management

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ManagedFunction << InformationObjectClass >>

class NDTFunction <<InformationObjectClass>>

class NDTJob <<InformationObjectClass>>

class NDTReport <<InformationObjectClass>>

ManagedFunction <|-- NDTFunction

Top <|-- NDTJob

Top <|-- NDTReport

@enduml

A.2	Procedure for NDT management

A.2.1	Procedure for delete an NDT job

@startuml

participant "MnS Consumer" as MNSC

participant “MnS Producer" as MNSP

MNSC -> MNSP: 1. Request to delete an NDT job instance

rnote over MNSP

2. Delete the NDT job MOI

endrnote

MNSP -> MNSC: 3. Response for delete an NDT job instance

@enduml

A.2.2	Procedure for query an NDT job

@startuml

participant "MnS Consumer" as MNSC

participant “MnS Producer" as MNSP

MNSC -> MNSP: 1. Request to query an NDT job instance

rnote over MNSP

2. Query the NDT job MOI

endrnote

MNSP -> MNSC: 3. Response for query an NDT job instance

@enduml

Annex B (informative):

NDT function in different domains

B.1	NDT function in RAN domain

The NDT function is provided by the RAN management domain to support RAN domain simulation/emulation capability. For instance, the NDT function in RAN domain is responsible for RAN energy saving configuration verification as introduced in clause 5.3.2.4. Figure B.1-1 depicts the scenario when NDT function is deployed in RAN domain.

Figure B.1-1: NDT function in RAN domain management

B.2	NDT function in cross domain

The NDT function is provided by the cross management domain to support cross-domain simulation/emulation capability. For instance, the NDT function in cross-domain is responsible for synthetic user experience data generation as introduced in clause 5.4.2.3. Figure B.2-1 depicts the scenario when NDT function is deployed in cross-domain. The cross management domain is introduced in TS 28.533.

Figure B.2-1: NDT function in cross-domain management

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-02 | SA5#159 | 1.S5-250858 2.S5-250859 3.S5-250861 4.S5-250862 5.S5-250951 6.S5-251120 7.S5-251121 |  |  |  | - pCR 28.561 Add skeleton - pCR 28.561 Add scope - pCR 28.561 Add the term and concepts of NDT - Rel-19 pCR TS 28.561 Add the Role of NDT in the Management Loop - pCR TS28.561 NDT support to Automation - pCR TS28.561 NDT LCM use case - pCR Add UC of using NDT to generate ML training data for TS 28.561 | 0.1.0 |
| 2025-04 | SA5#160 | 1.S5-251897 2.S5-251899 3.S5-251901 4.S5-251903 5.S5-252019 6.S5-252118 7.S5-252119 8.S5-252120 |  |  |  | - TS28.561 Add the NDT concept and scope - TS28.561 NDT support for network automation - Pseudo-CR on NDTs for verification - Pseudo-CR on TS 28.561 Add requirements for NDT supporting data generation - Rel-19 pCR TS 28.561 General improvements to use cases and requirements - Pseudo-CR on TS 28.561 NDT NRMs - Pseudo-CR on TS 28.561 Procedure for consuming NDT capability - Pseudo-CR on TS 28.561 Nested -NDT Definition | 0.2.0 |
| 2025-05 | SA5#161 | 1.S5-252610 2.S5-253004 3.S5-253005 4.S5-253063 5.S5-253065 6.S5-253068 7.S5-253102 |  |  |  | - Rel-19 pCR TS 28.561 Add authorization to all requirement - Pseudo-CR on TS 28.561 NDT Procedure update - Rel-19 pCR TS 28.561 Add procedures for delete and query an NDT job - Pseudo-CR on TS 28.561 NDT Use case Update - Rel-19 pCR TS 28.561 Deployment scenarios of NDT function - Pseudo-CR on NRM for Collaboration between NDTs - Pseudo-CR on General NRM for NDTs |  |