3GPP TSG-SA5 Meeting #161	S5-252839

Fukuoka, Japan, 19 - 23 May 2025

Title:	Reply LS on clarification on UE performance measurement at PDCP level

Response to:	LS in R3-252492 from RAN3 on clarification on UE performance measurement at PDCP level

Release:	Rel-19

Work Item:	AIML_MGT_Ph2

Source:	SA5

To:	RAN3

Cc:

Contact person:	Hassan Al-Kanani

Hassan.alkanani@emea.nec.com

Yizhi Yao

Yizhi.yao@intel.com

Zu Qiang

zu.qiang@ericsson.com

Bangqiu Ruan

ruan.bangqiu@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA5 would like to thank RAN3 for their LS in R3-252492 on clarification on UE performance measurement at PDCP level.

SA5 has discussed the LS from RAN3 and would like to provide the following clarifications:

DL UE Throughput Measurement at PDCP Level

SA5 would like to note that while the measurement ‘Average DL UE Buffered Throughput per DRB,’ defined in TS 28.552 clause 5.1.1.3.7, provides useful information, we would further like to inform RAN3 about the following information:

When the measurement is used for UE air interface, the following applies for the PDCP-level throughput measurements in both split and non-split gNB scenarios:

- While the measurement provides a means to estimate PDCP-layer throughput based on the volume of PDCP SDUs buffered and the achievable DRB throughput (as inferred from DDDS), it does not directly measure over-the-air data transmission.

- The precision of the measurement may vary depending on implementation-specific aspects such as:

  - The frequency and consistency of DDDS reporting from DU to CU-UP,

  - The alignment of DDDS intervals with PDCP burst dynamics, and

  - The treatment of short-duration or highly bursty traffic.

Average DL UE Buffered Throughput per DRB measurement is not a direct or precise representation of total PDCP-level throughput per UE.

UL UE Throughput Measurement at PDCP Level

SA5 would like to confirm that uplink PDCP throughput measurements are not defined in TS 28.552.

While UE Data Volume counters are defined in TS 28.558 clause 6.3.1.5 (per UE), these represent cumulative data volume, not throughput. SA5 notes that throughput requires both volume and time components. Therefore, although TS 28.558 provides a useful basis, it is not sufficient by itself to define an accurate PDCP-layer UL throughput measurement.

Packet Loss Rate at PDCP Level (UL and DL)

SA5 recognises the importance of monitoring packet loss at the PDCP level in both UL and DL directions. The following measurements are already defined:

- UL PDCP SDU Loss Rate – TS 28.552 clause 5.1.3.1.1

- DL PDCP SDU Drop Rate in gNB-CU-UP – TS 28.552 clause 5.1.3.2.1

These metrics allow packet loss monitoring in UL and packet drop monitoring in DL for PDCP-layer traffic. SA5 encourages RAN3 to review these existing definitions and provide feedback on whether they satisfy the intended use cases, or if additional definitions are required.

# 2	Actions

To: RAN3

ACTION: SA5 kindly asks RAN3 to take the above clarification into their consideration. If RAN3 has any specific requirements for SA5, we kindly request that you provide more details on those requirements and the related use cases. This information will be valuable for SA5 as input to develop the appropriate measurements.

# 3	Dates of next TSG SA WG5 meetings

SA5#162		25 August - 29 August 2025	Goteborg, Sweden

SA5#163		13 October - 17 October 2025	Wuhan, China