3GPP TSG-SA5 Meeting #161	S5-253012

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0234 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 Add EE KPIs evaluated from network quality performance dimension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Mobile, China Telecom, Orange, Verizon, ATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion for multi-dimensional network energy efficiency metrics in TR 28.880, it is proposed to add EE KPIs evaluated from network quality performance dimension. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EE KPIs evaluated from network quality performance dimension. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 6.7.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.7.X	Energy efficiency evaluated from network quality

#### 6.7.X.1	Introduction

These KPIs about energy efficiency evaluated from network quality performance perspective are defined with two variants for sub-network or network slice subnet.

#### 6.7.X.2	Energy efficiency of a sub-network based on DL UE throughput

a) EESNw,DLUeThroughput

b) A KPI that shows the energy efficiency of a sub-network based on DL UE throughput. This KPI is obtained by DL RAN UE throughput for a sub-network KPI divided by the average energy consumption per gNB of the sub-network over the same observation period. The unit of this KPI is kbit/(sec*J).

c)

EESNw,DLUeThroughput=DlUeThroughput _SNwECper gNB of sub-network

d) SubNetwork

e) The DL RAN UE throughput for a sub-network KPI is defined in clause 6.3.6.3.2. The average energy consumption per gNB of the sub-network is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the sub-network and then dividing the result by the total number of gNBs present in the sub-network. ECgNB is defined in clause 6.7.3.4.2. The use of ECper gNB of sub-network in the denominator enables this KPI to be used to compare the EE of sub-networks of different sizes/scales.

NOTE:	The average energy consumption for gNB is derived by averaging of the sum of the energy consumption of ECgNB over the number of gNBs present in the subnetwork.

#### 6.7.X.3	Energy efficiency of a sub-network based on UL UE throughput

a) EESNw,ULUeThroughput

b) A KPI that shows the energy efficiency of a sub-network based on UL UE throughput. This KPI is obtained by UL RAN UE throughput for a sub-network KPI divided by the average energy consumption per gNB of the sub-network over the same observation period. The unit of this KPI is kbit/(sec*J).

c)

EESNw,ULUeThroughput=UlUeThroughput _SNwECper gNB of sub-network

d) SubNetwork

e) The UL RAN UE throughput for a sub-network KPI is defined in clause 6.3.6.4.2. The average energy consumption per gNB of the sub-network is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the sub-network and then dividing the result by the total number of gNBs present in the sub-network. ECgNB is defined in clause 6.7.3.4.2. The use of ECper gNB of sub-network in the denominator enables this KPI to be used to compare the EE of sub-networks of different sizes/scales.

NOTE:	The average energy consumption for gNB is derived by averaging of the sum of the energy consumption of ECgNB over the number of gNBs present in the subnetwork.

#### 6.7.X.4	Energy efficiency of a network slice subnet based on DL UE throughput

a) EENss,DLUeThroughput

b) A KPI that shows the energy efficiency of a network slice subnet based on DL UE. This KPI is obtained by DL RAN UE throughput for a network slice subnet KPI divided by the average energy consumption per gNB of the network slice subnet over the same observation period. The unit of this KPI is kbit/(sec*J).

c)

EENss,DLUeThroughput=DlUeThroughput_NssECper gNB of Nss

d) NetworkSliceSubnet

e) The DL RAN UE throughput for a network slice subnet KPI is defined in clause 6.3.6.3.3. The average energy consumption per gNB of the network slice subnet is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the network slice subnet and then dividing the result by the total number of gNBs present in the network slice subnet. ECgNB is defined in clause 6.7.3.4.2. The use of ECper gNB of Nss in the denominator enables this KPI to be used to compare the EE of network slice subnets of different sizes/scales.

NOTE:	The average energy consumption for gNB is derived by averaging of the sum of the energy consumption of ECgNB over the number of gNBs present in the network slice subnet.

#### 6.7.X.5	Energy efficiency of a network slice subnet based on UL UE throughput

a) EENss,ULUeThroughput

b) A KPI that shows the energy efficiency of a network slice subnet based on UL UE throughput. This KPI is obtained by UL RAN UE throughput for a network slice subnet KPI divided by the average energy consumption per gNB of the network slice subnet over the same observation period. The unit of this KPI is kbit/(sec*J).

c)

EENss,ULUeThroughput=UlUeThroughput_NssECper gNB of Nss

d) NetworkSliceSubnet

e) The UL RAN UE throughput for a network slice subnet KPI is defined in clause 6.3.6.3.3. The average energy consumption per gNB of the network slice subnet is obtained by summing up the Energy Consumption (ECgNB) of all the gNBs that constitute the network slice subnet and then dividing the result by the total number of gNBs present in the network slice subnet. ECgNB is defined in clause 6.7.3.4.2. The use of ECper gNB of Nss in the denominator enables this KPI to be used to compare the EE of network slice subnets of different sizes/scales.

NOTE:	The average energy consumption for gNB is derived by averaging of the sum of the energy consumption of ECgNB over the number of gNBs present in the network slice subnet.

End of change