3GPP TSG-SA5 Meeting #161	S5-252321

Fukuoka, Japan, 19 - 23 May 2025

Source:	Huawei, Deutsche Telekom, Ericsson, Nokia, Samsung

Title:	pCR TR 28.869 Add potential solution for VNF configuration

Document for:	Approval

Agenda item:	6.19.6

Spec:	3GPP TR 28.869

Version:	V1.4.0

Work Item:	FS_Cloud_OAM

Comments

In clause 5.1.5, two options are proposed to configure/manage NF realized by VNF(s) with VNF application specific parameters and VNF non-application specific parameters. The justification/motivation to introduce the option#2 (Interactions between SBMA and VNF Generic OAM Functions) is to provide a unified OAM solution to support both VNF application specific parameters and VNF non-application specific parameters management. However, the option#2 introduced a new entity and several interfaces to satisfy this purpose.

The existing 3GPP MnSs (including provision MnS) support vendor extension mechanism (e.g. VsDataContainer), which can be simply used to support the management of VNF non-application parameters. So, it proposes to add potential solution to use the existing 3GPP MnS with vendor extension mechanism to satisfy the requirement of unified OAM solution for configuring/managing NF realized by VNF(s) with VNF application specific parameters and VNF non-application specific parameters.

Proposed Changes

* * * First Change * * * *

5.1.5	Relationship between 3GPP Management System (SBMA) and ETSI NFV VNF generic OAM functions and NFV-MANO

The decoupling of software and hardware in NFV gave rise to the need to delineate the concerns of application-level management versus virtualization-related management. VNF application specific parameters are related to the behavior and services provided by the NF, which in the 3GPP context are therefore 3GPP service related and defined by 3GPP, whilst the VNF non-application specific parameters (i.e. non-3GPP service related) are related to the virtualization aspects of the NF and consider the aspects on how software entities can be deployed and operated on virtualized infrastructure platform. Following are the details:

- The MO attributes, which are defined by 3GPP, can be regarded to be VNF application specific parameters.

- The VNF instance specific information defined in ETSI NFV specifications can be regarded to be VNF non-application specific parameters.

NOTE:	The VNF application specific parameters are referred as virtualization-independent items in ETSI IFA 049 [2], VNF non-application specific parameters are referred as virtualization dependent items in ETSI IFA 049 [2].

Regarding the relationship between the VNF generic OAM functions framework and 3GPP management system (SBMA) defined in 3GPP TS 28.533 [16] the following can be considered:

Like in the case of NFV-MANO, VNF generic OAM functions reside outside the 3GPP management system (SBMA).

In SBMA, a MnS producer can consume management interfaces provided by NFV-MANO and can expose a corresponding service to MnS consumers. Similarly for the case of VNF Generic OAM functions  in SBMA, a MnS producer can consume management interfaces provided by VNF generic OAM functions and can expose a corresponding service to MnS consumers.

Following are the two options to illustrate the relation between 3GPP management system (SBMA),VNF generic OAM functions and NFV-MANO.

NOTE:	The name of VNF application parameters and VNF non-application parameters needs to be revisited during normative phase.

Option#1:

Figure 1: Use of 3GPP MnS and ETSI NFV MANO to manage VNF

In this option, 3GPP management system (SBMA) configures/manages NF realized by VNF(s) with VNF application specific parameters by utilizing already defined 3GPP MnSs (e.g. provisioning MnS). 3GPP management system interacts NFV-MANO to configure/manage NF realized by VNF(s) with VNF non-application specific configurations.

Option#2:

Figure 2: Interactions between SBMA and VNF Generic OAM Functions

In this option, 3GPP management system (SBMA) still manages VNF non-application specific parameters with NFV-MANO by utilizing already defined 3GPP MnSs. In addition the 3GPP management system interacts with PaaS Services defined in ETSI ISG NFV-IFA 049 [2] (i.e., VNF Generic OAM functions) to manage VNF non-application specific aspects (typically those which are not managed by NFV-MANO or resulting from NFV-related orchestration procedures) but also VNF application specific aspects (e.g., VNF configuration backup etc.).

In the case of VNF application specific aspects, VNF generic OAM functions, which do not process the semantics of application specific parameters, serving simply as utility functions in the cloud-native platform with specific OAM capabilities.

Option#3:

Figure 3: Use of 3GPP MnS as unified solution

In this option, 3GPP management system (SBMA) configures/manages NF realized by VNF(s) with VNF application specific parameters and VNF non-application specific parameters (typically those which are not managed by NFV-MANO or resulting from NFV-related orchestration procedures) by utilizing already defined 3GPP MnSs (e.g. provisioning MnS). The VNF non-application specific parameters can be carried within VsDataContainer defined in TS 28.622 [45]. The VNF application specific parameters and VNF non-application specific parameters can be specified by MnS consumer or derived from MnS Producer.

In the case of VNF non-application specific aspects, the 3GPP MnSs, which do not process the semantics of VNF non-application specific parameters, serve simply to provide unified OAM capabilities for managing NF realized by VNF(s).

* * * Next Change * * * *

5.1.1	Use case #1: Cloud-native VNF configuration management

5.1.1.1	Description

Besides allocation and configuration of the necessary resources, to bring up a cloud native VNF into operation, i.e. full instance provisioning and configuration of the corresponding VNF instances needs to take place. Configuration actions can also be performed during the cloud native VNF instance operation.

An operator needs to be able to manage and orchestrate through the 3GPP management system the configuration of the cloud native VNF instances. The operator needs to be able to perform through the 3GPP management system, operations like set and query configuration information.

For data governance reasons, the operator needs to be able to control the location where configuration data is stored. Additional information is needed that can be used by the MnS producer to support operations like:

- backup configuration information,

- rollback configuration information (e.g., after reverting software to a previous stable version),

- remove unnecessary configuration files (e.g., very old versions),

5.1.1.2	Potential requirements

REQ-CVNF_CM-1 The 3GPP management system should have the capability to perform configuration operations for cloud-native VNF instances..

REQ-CVNF_CM-2 The reference point between the 3GPP management system and external OAM entities should have the capability enabling the 3GPP management system to interact with external (non-3GPP) configuration management entities for the purpose of performing configuration operations for cloud-native VNF instances.

Editor's Note: The use of the term cloud native network function and cloud native VNFs in the present document is FFS.

5.1.1.3	Potential solutions

5.1.1.3.1	VNF Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of the VNF Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. Some key functionalities supported by the VNF Configuration Manager function are the capability to convey configuration information to one or more cloud-native VNF/VNFC instances, the capability to perform pre-configuration actions (e.g. create configuration backup) and post-configuration actions (e.g. rollback running configuration) and the capability to query configuration information of cloud-native VNF/VNFC instances.

From 3GPP management system perspective, MnS provisioning handles configuration of managed NFs defined by 3GPP, which include the necessary configuration related to the behavior and role of a NF (e.g., configuration of a gNB CU UP function). The 3GPP management system fully understands the semantics of this kind of configuration since it is needed to ensure that the 3GPP mobile network operates as intended by the network operator.

The VNF Configuration Manager does not understand the semantics of the configuration information that is conveyed to the cloud-native VNF/VNFC instances (both VNF application configuration parameters and VNF non-application configuration parameters).

Figure 5.1.1.3.1-1 depicts the interaction and reference point between 3GPP management system and the VNF Configuration Manager.

Figure 5.1.1.3.1-1: Interaction and reference point between 3GPP management system and VNF Configuration Manager

NOTE:	As specified in ETSI GS NFV-IFA 049 [2], the VNF Configuration Manager is a VNF generic OAM function, while close mapping between VNF generic OAM functions and Platform as a Service (PaaS) Services exists. According to ETSI GS NFV-IFA 049 [2], any VNF generic OAM function can be understood as a PaaS Service of a specific type, i.e. a PaaS Service that provides and handles generic OAM functionality for VNFs. See ETSI GR NFV 003 [11] for a definition of the term PaaS Service.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

Editor’s Note: update the solution description to better reflect the impact to the 3GPP management system.

5.1.1.3.2	Network Configuration Manager function

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution considers the use of the Network Configuration Manager function defined in ETSI GS NFV-IFA 049 [2]. The Network Configuration Manager function can be used to set network configuration information related to one or more VNF/VNFC instances. For example, the Network Configuration Manager can be used to configure the VNF Connection Points (CPs), VNF networking for VNF instances in a service mesh, etc.

Figure 5.1.1.3.2-1 depicts the interaction and reference point between 3GPP management system and the Network Configuration Manager.

Figure 5.1.1.3.2-1: Interaction and reference point between 3GPP management system and Network Configuration Manager

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5.

5.1.1.3.3	Configuration Server

This solution introduces a platform entity that interacts with the 3GPP management system via a new reference point for performing the configuration management of cloud-native VNFs.

This solution proposes the use of a Configuration Server introduced in ETSI GR NFV-EVE 022 [44]. The Configuration Server can be understood as a logically centralized storage for configuration, without limiting the implementation of the storage. In addition, Configuration Server services can be used to store and fetch configuration data, convert configuration data formats, validate configuration data schema and apply version control for configuration data. The interface exposed and operations supported by the Configuration Server are defined in ETSI GS NFV-IFA 049 [2].

NOTE:	Implementation of the Configuration Server, e.g. structure of configuration data, format of storage is not in scope of the present solution.

Figure 5.1.1.3.3-1 depicts the interaction and reference point between 3GPP management system and the Configuration Server.

Figure 5.1.1.3.3-1: Interaction and reference point between 3GPP management system and Configuration Server

As depicted in Figure 5.1.1.3.3-1, a consumer of the Configuration Server interfaces could be other VNF generic OAM function such as the VNF Configuration Manager defined in ETSI GS NFV-IFA 049 [2] or the 3GPP management system.

The solution enables the 3GPP management system having the capability to handle configuration data, like distributing, fetching and transforming configuration data, for one or multiple cloud-native VNFs.

The present solution addresses the potential requirements REQ-CVNF_CM-1 and REQ-CVNF_CM-2.

This potential solution related to option 2 in clause 5.1.5

5.1.1.3.4	Using the existing 3GPP provisioning management service and ETSI NFV MANO

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for VNF application specific parameters configuration purposes.

- The 3GPP management system configures VNF with VNF application specific parameters by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of  TS 28.532 [10]).

- The 3GPP management system interacts with NFV-MANO to configure VNF non-application parameters (as defined in clause 5.1.18 of TS 28.531[7]).

This solution is not using the VNF Generic OAM functions described in ETSI ISG NFV-IFA049 [2].

This potential solution related to option 1 in clause 5.1.5.

5.1.1.3. X	Using the existing 3GPP provisioning management service

In this solution a MnS producer offering the MnS provisioning service directly interacts with the VNF for both VNF application specific parameters configuration and VNF non-application parameters purposes.

- The 3GPP management system configures the VNF with VNF application specific parameters by utilizing the already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]) and the network resource model defined in TS 28.541 [7].

- The 3GPP management system configures VNF with VNF non-application specific parameters by utilizing already defined 3GPP provisioning MnS (as defined in clause 11.1 of TS 28.532 [10]) and the VsDataContainer IOC defined in TS 28.622 [45]. The VNF non-application parameters (including both standardized and non-standardized parameters) can be carried by the VsDataContainer IOC.

This solution provides a unified solution for configuring both VNF application specific parameters configuration and VNF non-application specific parameters by utilizing the existing 3GPP provisioning management service. This mechanism also supports the association between VNF application specific parameters configuration and VNF non-application specific parameters configuration, for example, add VsDataContainer name contained by EP_RP to carry the CpConfiguration defined in ETSI NFV IFA 008 [9] to support association between 3GPP defined EP_RP with ETSI NFV defined External Connection Point. Further, this solution supports carrying any type of configuration data that is required to support specific implementations.

This potential solution is related to option#3 in clause 5.1.5.

* * * End of Changes * * * *