3GPP TSG-SA5 Meeting #161	S5-252324

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | - | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to draftCR TS 28.537 for MADCOL Add solution reference management data registry and discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MADCOL_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution reference description for serveral capabillities in TS 28.537 was added in SA5 #159 meeting. However, the soluton description for Discovery of management data in clause 6.5 in TS 28.537 is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add solution description for Discovery of management data in clause 6.5 in TS 28.537 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.5	Discovery of management data

6.5.1	Description

Discovery of management data mechanism allows MnS consumers to discover what management data can be produced by the 3GPP management system without direct involvement of those MnS services producing the data, which can be time and resource consuming process.

For this mechanism to work MnS producers as entities producing data, need to register what data they can produce by adding a corresponding record in the 3GPP management system. The MnS consumer needs to discover or retrieve what management data (represented by management data name) or what type of management data (represented by management data category) can be produced by MnS producer. In addition, thee MnS consumer needs to know what reporting methods are supported for the specified management data.

NOTE: The term "management data produced by 3GPP management system" relates to

-	5G performance measurements as defined by TS 28.552 [4]

-	5G end to end key performance indicators as defined by TS 28.554 [5], and

-	Trace metrics as defined by TS 32.423 [8].

6.5.2	Void

6.5.3	Requirements

REQ-MDM-DIS-1: The 3GPP management system shall enable an authorized data consumer to discover what management data can be produced by 3GPP management system.

REQ-DMSDIS-CON-2: The 3GPP management system shall enable an authorized data consumer to discover what category of management data can be produced by 3GPP management system.

REQ-DMSDIS-CON-3: The 3GPP management system shall enable an authorized data consumer to obtain the reporting methods that are supported for the management data which can be produced by 3GPP management system.

6.5.X	Solutions

6.5.X.1	Stage 2 definition

Following are the stage 2 definition for discovery of management data:

- The operations and notifications of generic provisioning MnS defined in clause 11.1 in TS 28.532 [2].

- The MnSRegistry NRM fragment (including MnSRegistry IOC and MgmtDataInfo IOC) defined in clause 4.3.41 and clause 4.3.Z in TS 28.622 [9].

6.5.X.2	Stage 3 definition

Following are the stage 3 definition for discovery of management data:

- RESTful HTTP-based solution set

- RESTful HTTP-based solution set for generic provisioning management service is defined in clause 12.1.1 in TS 28.532 [2]. OpenAPI document "TS28532_ProvMnS.yaml" in clause A.1 in TS 28.532 [2].

- OpenAPI document " TS28623_MnSRegistryNrm.yaml" in clause 4.3 in TS 28.623 [10].

End of Changes