3GPP TSG-SA5 Meeting #161	S5-252380

# Fukuoka, Japan, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0065 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 CAPIF Triggers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Enable CCF once is reached to interact with the CHF, so the Charging Data can be available, quota is managed and ultimately CDRs are generated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Triggers clause for the CCF in the Northbound API converged online and offline charging scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging Data Message exhange between CCF and CHF is not done |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.1, 5.4.1.x, 5.4.1.x.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 5.4	Northbound API converged online and offline charging scenarios

### 5.4.1	Basic principles

#### 5.4.1.1	General

Converged charging may be performed by the NEF and CCF interacting with CHF using Nchf specified in TS 32.290 [57] and TS 32.291 [58]. In order to provide the data required for the management activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.), the NEF and CCF shall be able to perform converged charging for the northbound API access.

The NEF and CCF shall be able to perform convergent charging by interacting with CHF, for charging data related to Northbound API access. The Charging Data Request and Charging Data Response are exchanged between the NEF and the CHF, and also between CCF and CHF, based on PEC, either IEC or ECUR scenarios specified in TS 32.290 [57]. The Charging Data Request can either be issued by the NEF or the CCF towards the CHF when certain conditions (chargeable events) are met.

Converged charging uses centralized or decentralized unit determination and centralized rating scenarios for event based convergent charging specified in TS 32.290 [57].

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [58].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

The selection of the CHF can be configured in the NEF but may also rely on NRF.

Second change

#### 5.4.1.x	Applicable Triggers in the CCF

##### 5.4.1.x.y	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. identifier of the AF).

Each trigger condition (i.e. chargeable event) defined for the CCF converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.4.1.x.y-1 summarizes the set of default trigger conditions and their category which shall be supported by the CCF when charging is active for the corresponding CAPIF functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from CCF towards the CHF.

Table 5.4.1.x.y-1: CAPIF default trigger conditions in NEF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Service API Publish | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Service API Publish Notification | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Service API Publish Acknowledgement | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Service API Unpublish | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Service API Unpublish Acknowledgement | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |
| Service API update | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Service API update Acknowledgement | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |

End of changes